COMPARISON AND DESIGN OF SIMPLIFIED GENERAL PERTURBATION MODELS (SGP4) AND CODE FOR NASA JOHNSON SPACE CENTER,

ORBITAL DEBRIS PROGRAM OFFICE

A Graduate Project

presented to

the Faculty of California Polytechnic State University,

San Luis Obispo

In Partial Fulfillment

of the Requirements for the Degree

Master of Science  in Aerospace Engineering

by

Nicholas Zwiep Miura

May 2009

© 2009

Nicholas Zwiep Miura

ALL RIGHTS RESERVED

COMMITTEE MEMBERSHIP

TITLE:	Comparison and Design of Simplified General Perturbation Models (SGP4) and Code for NASA Johnson Space Center, Orbital Debris Program Office.

AUTHOR:	Nicholas Zwiep Miura

DATE SUBMITTED:	May 29, 2009

COMMITTEE CHAIR: 	 Kira Abercromby, Ph.D

COMMITTEE MEMBER:	Eric Mehiel, Ph.D

COMMITTEE MEMBER:	Jordi Puig-Suari, Ph.D

COMMITTEE MEMBER:	Edwin Barker, Ph.D

# ABSTRACT

Comparison and Design of Simplified General Perturbation Models (SGP4) and Code for NASA Johnson Space Center, Orbital Debris Program Office

Nicholas Zwiep Miura

This graduate project compares legacy simplified general perturbation model (SGP4) code developed by NASA Johnson Space Center, Orbital Debris Program Office, to a recent public release of SGP4 code by David Vallado.  The legacy code is a subroutine in a larger program named PREDICT, which is used to predict the location of orbital debris in GEO.  Direct comparison of the codes showed that the new code yields better results for GEO objects, which are more accurate by orders of magnitude (error in meters rather than kilometers).  The public release of SGP4 also provides effective results for LEO and MEO objects on a short time scale.  The public release code was debugged and modified to provide instant functionality to the Orbital Debris Program Office.  Code is provided in an appendix to this paper along with an accompanying CD.  A User’s Guide is presented in Chapter 7.

# ACKNOWLEDGMENTS

I would like to thank my wife, Jesselle, for all her support throughout the year.  Also, a special thanks to Dr. Abercromby, my thesis advisor, who provided excellent guidance and resources during the accelerated process.

TABLE OF CONTENTS

LIST OF FIGURES	x

LIST OF TABLES	xiv

LIST OF ACRONYMS	xv

1.  Introduction	1

2.  Space Propagation Models	2

3.  SGP4 for Johnson Space Center	4

4.  Procedure	6

4.1 Understanding the Software	6

4.1.1 MATLAB Code	8

4.1.2 FORTRAN Code	8

4.1.3 PREDICT Code	9

4.2 Analyzing Differences	9

4.2.1 Objects 8832, 25000, and 30000	11

4.2.2 Other Objects	13

4.3 Programming Deliverables	13

5. Results	15

5.1 Data Points	15

5.2 MATLAB vs. SATRAK	16

5.2.1 Object 8832	16

5.2.2 Object 25000	18

5.2.3 Object 30000	20

5.2.4 Analysis - MATLAB	22

5.3 FORTRAN vs. SATRAK	23

5.3.1 Object 8832	23

5.3.2 Object 25000	25

5.3.3 Object 30000	27

5.3.4 Analysis - FORTRAN	29

5.4 Modified Code vs. SATRAK	31

5.4.1 Object 8832	31

5.4.2 Object 25000	33

5.4.3 Object 30000	35

5.4.4 Analysis – Modified Code	37

5.5 JSC Code vs. SATRAK	38

5.5.1 Object 8832	39

5.5.2 Object 25000	41

5.5.3 Object 30000	44

5.5.4 Analysis – JSC Code	45

5.6 Polysat	46

5.6.1 CP3	47

5.6.2 CP4	50

5.7 ISS Tool bag	50

5.8 Iridium Debris	52

5.9 Operational Issues	53

5.9.1 Program Effectiveness	53

5.9.2 Operational Speed	57

6. Conclusion	58

7. User’s Guide	60

7.1 MATLAB	60

7.1.1 Installation	60

7.1.2 Verification Run	61

7.1.3 TLE inputs	62

7.1.4 Catalog Mode	62

7.1.5 Manual Mode	63

7.1.6 Understanding the Output File	64

7.2 FORTRAN (Vallado Code)	65

7.2.1 Installation	66

7.2.2 Verification Run	66

7.2.3 TLE Input	67

7.2.4 Catalog Mode	67

7.2.5 Manual Mode	67

7.2.6 Understanding the Output File	69

7.3 Modified FORTRAN Code – For JSC	69

7.3.1 Installation	69

7.3.2 Code Options	71

BIBLIOGRAPHY	74

APPENDIX A.  SGP4 Code	76

A.1 Modified FORTRAN Code (Vallado)	76

A.2 Other Codes	124

APPENDIX B. Verification TLE File	125

APPENDIX C – Verification Results Data	127

APPENDIX D. REFERENCES	137

# LIST OF FIGURES

Figure 1 - 8832 Graphical Orbit Data	10

Figure 2 - 25000 Graphical Orbit Data	10

Figure 3 - 30000 Graphical Orbit Data	11

Figure 4 - 8832 Position Error MATLAB (60 min intervals)	16

Figure 5 - 8832 Velocity Error MATLAB (60 min intervals)	17

Figure 6 - 8832 Position Error MATLAB (360 min intervals)	17

Figure 7 - 8832 Velocity Error MATLAB (360 min intervals)	18

Figure 8 - 25000 Position Error MATLAB (60 min intervals)	18

Figure 9 - 25000 Velocity Error MATLAB (60 min interval)	19

Figure 10 - 25000 Position Error MATLAB (360 min interval)	19

Figure 11 - 25000 Velocity Error MATLAB (360 min interval)	20

Figure 12 - 30000 Position Error MATLAB (60 min interval)	20

Figure 13 - 30000 Velocity Error MATLAB (60 min interval)	21

Figure 14 - 30000 Position Error MATLAB (360 min intervals)	21

Figure 15 - 30000 Velocity Error MATLAB (360 min intervals)	22

Figure 16 - Comparison of 60 and 360 minute intervals	23

Figure 17 - 8832 Position Error FORTRAN (60 min intervals)	24

Figure 18 - 8832 Velocity Error FORTRAN (60 min intervals)	24

Figure 19 – 8832 Position Error FORTRAN (360 min intervals)	25

Figure 20 – 8832 Velocity Error FORTRAN (360 min intervals)	25

Figure 21 - 25000 Position Error FORTRAN (60 min intervals)	26

Figure 22 - 25000 Velocity Error FORTRAN (60 min intervals)	26

Figure 23 - 25000 Position Error FORTRAN (360 min intervals)	27

Figure 24 - 25000 Velocity Error FORTRAN (60 min intervals)	27

Figure 25 - 30000 Position Error FORTRAN (60 min intervals)	28

Figure 26 - 30000 Velocity Error FORTRAN (60 min intervals)	28

Figure 27 - 30000 Position Error FORTRAN (360 min intervals)	29

Figure 28 - 30000 Velocity Error FORTRAN (360 min intervals)	29

Figure 29 - Comparison of MATLAB and FORTRAN results (8832 position)	30

Figure 30 - Comparison of MATLAB and FORTRAN (8832 Velocity)	31

Figure 31 - 8832 Position Error Vallado (60 min intervals)	32

Figure 32 - 8832 Velocity Error Vallado (60 min intervals)	32

Figure 33 – 8832 Position Error Vallado (360 min intervals)	33

Figure 34 - 8832 Velocity Error Vallado (360 min intervals)	33

Figure 35 - 25000 Position Error Vallado (60 min intervals)	34

Figure 36 - 25000 Velocity Error Vallado (60 min intervals)	34

Figure 37 - 25000 Position Error Vallado (360 min intervals)	35

Figure 38 - 25000 Velocity Error Vallado (360 min intervals)	35

Figure 39 - 30000 Position Error Vallado (60 min intervals)	36

Figure 40 - 30000 Velocity Error Vallado (60 min intervals)	36

Figure 41 - 30000 Position Error Vallado (360 min intervals)	37

Figure 42 - 30000 Velocity Error Vallado (360 min intervals)	37

Figure 43 - Comparison of Vallado to MATLAB (8832 Velocity Error)	38

Figure 44 - 8832 Position Error Matney (60 min intervals)	39

Figure 45 - 8832 Velocity Error Matney (60 min intervals)	40

Figure 46 - 8832 Position Error Matney (360 min intervals)	40

Figure 47 - 8832 Velocity Error Matney (360 min intervals)	41

Figure 48 - 25000 Position Error Matney (60 min intervals)	41

Figure 49 - 25000 Velocity Error Matney (60 min intervals)	42

Figure 50 - 25000 Position Error Matney (360 min intervals)	42

Figure 51 - 25000 Velocity Error Matney (360 min intervals)	43

Figure 52 - 30000 Position Error Matney (60 min intervals)	44

Figure 53 - 30000 Velocity Error Matney (60 min intervals)	44

Figure 54 - 30000 Position Error Matney (360 min intervals)	45

Figure 55 - 30000 Velocity Error Matney (360 min intervals)	45

Figure 56 - CP3 Graphical Orbit Data	47

Figure 57 - CP4 Graphical Orbit Data	47

Figure 58 - CP3 Position Error Vallado (10 sec intervals)	48

Figure 59 - CP3 Velocity Error Vallado (10 sec intervals)	48

Figure 60 - ISS Tool bag Graphical Orbit Data	51

Figure 61 - Iridium Debris Graphical Orbit Data	52

Figure 62 - 8832 Operational Effectiveness	54

Figure 63 - 25000 Operational Effectiveness	55

Figure 64 - 30000 Operational Effectiveness	55

Figure 65 - 30000 Operational Effectiveness (new code)	56

Figure 66 - CP3 Operational Effectiveness	57

Figure 67 - Code Addition to SURVEYCHASENEW	70

Figure 68 - Changing WGS values (1)	71

Figure 69 - Changing WGS values (2)	72

Figure 70 - Changing the B* term	73

# LIST OF TABLES

Table 1 - TLE readings for CP3	49

Table 2 - CP3 TLE Results	49

Table 3 - CP4 TLE Data	50

Table 4 - CP4 Results	50

Table 5 - ISS Tool bag TLE Data	51

Table 6 - ISS Tool bag Results	51

Table 7 - Iridium Debris TLE Data	52

Table 8 - Iridium Results	53

# LIST OF ACRONYMS

NASA	National Aeronautics and Space Administration

SGP(4)	Simplified General Perturbation

PREDICT	NASA code that predicts position of orbital debris

SPACETRACK	Government program initiated to track satellites

AIAA	American Institute of Aeronautics and Astronautics

JSC	Johnson Space Center

SATRAK	Satellite Tracking (Government owned propagator)

GSFC	Goddard Space Flight Center

JPL	Jet Propulsion Laboratory

FORTRAN	Formula Translating (IBM developed language)

COES	Classical Orbital Element Set

TLE	Two Line Element

WGS	World Geodetic System

LES	Lincoln Experimental Satellite

GEO	Geosynchronous Earth Orbit

LEO	Low Earth Orbit

MEO	Medium Earth Orbit

ISS	International Space Station

min	minutes

m	meters

cm	centimeters

mm	millimeters

sec	seconds

km	kilometers

Polysat	Cube satellites developed by Cal Poly

CP3	Polysat 3

CP4	Polysat 4

FOV	Field of view

# 1.  Introduction

This project has been completed for NASA Johnson Space Center, Orbital Debris Program Office, with the purpose of comparing the effectiveness of legacy SGP4 propagation code and a recent public release of SGP4 propagation code.  Furthermore, new SGP4 propagation code based on the public release was developed and delivered.

The Orbital Debris Program Offices uses a program called PREDICT to predict the motion of deep space debris.  The program was developed in-house and uses a simplified version of Simplified General Perturbation Theory to propagate the position and velocity of a spacecraft forward in time based on classic orbital elements.  In 2004, there was a public release of Simplified General Perturbation 4 (SGP4) code, based on the same theory.  The Orbital Debris Program Office wanted to know how their legacy code compared with the public release.

This project compares the two codes to truth data provided by the Orbital Debris Program Office, and shows the accuracy of the delivered results.  Additionally, the public release code was debugged and provided for use and the public release code was modified to run within the PREDICT program.

This paper describes the problems involved in this undertaking, as well as the procedure taken to solve these problems.  Next, a detailed display of the results plus analysis and conclusions is given.  Finally, a User’s Guide is provided to help navigate using the public release code and to help update the legacy code.

2.  Space Propagation Models

Space propagation models use current state information of a satellite to predict a future state of the satellite.  As a simplistic example, imagine a car driving down a highway.  If we know the location and the speed of the car now, we can make an accurate prediction of where the car will be in an hour.  Similarly for satellites, if we know the position and the velocity now, we can make a reasonable guess where the satellite will be in the future.  The satellite however, encounters disturbances, or perturbations, along its path that complicates its motion.  These perturbations are caused by the Earth’s shape (spherical harmonics), drag, radiation, and effects from other bodies (the sun and moon generally).  In our car example, imagine driving up and down hills, over different terrain, with changing speed limits.  It makes our simplified example much harder.

Space propagation models are used primarily by agencies that track orbiting objects.  The ultimate goal is to know where everything in space is at all times.  Propagation models are needed because there are simply too few telescopes to watch everything in the sky at all times.  Thus, you find the position and velocity of an object once, and then use the propagator to tell you where it is in the future if you ever need to locate it again.  The first attempt to use these models came in 1959, by the National Space Surveillance Control Center.  This was a highly simplified model that failed when trying to propagate position more than a week.  In 1960, the initial model was replaced by the ‘Simplified General Perturbations (SGP)’ model.  Though this model provided accurate results, by 1969 there were too many satellite in orbit to maintain a catalog based on SGP.  SGP4, published in SPACETRACK Report #3, was created based on a simplified version of SGP.  By 1979, it was the sole model for catalog maintenance.   SGP4 failed when objects were in ‘deep space’ orbits, so in 1977, an extension to the program was developed to track object with an altitude over 255 km – the height where solar/lunar perturbations have a larger effect than atmospheric drag.

After SPACETRAK Report #3 was released, the ‘official’ code maintained by the U.S. government became an export controlled black-box.  Changes to the code to reflect new programming techniques, better computing power, and more accurate constants were never made public; instead, were made available to select agencies through an executable program called SATRAK.

Agencies not privy to SATRAK, or who needed customized code for their project (including NASA) were forced to manipulate the original code found in SPACETRAK Report #3 themselves, leading to a wide variety of propagation codes based on the same model.  David Vallado, working through the Center for Space, released an AIAA paper in 2004, which attempted to reconcile the many codes into one standardized code.  This new code was made available to the public through celestrak.com.

# 3.  SGP4 for Johnson Space Center

Johnson Space Center (JSC) – Orbital Debris Program Office was one project that required the use of a Space Propagation Model.  Though they had access to an executable version of SATRAK, customized code was needed to run within a larger program called PREDICT.  A telescope observes a debris object and takes right ascension and declination data over a short period of time.  The PREDICT code then takes this data and formulates a classical orbital element set (COES) that is fed into the space propagation model.  The model can then estimate where the object will be in the near future so it can be found again and analyzed further.

Dr. Mark Matney of the Orbital Debris Program Office was the original architect of the PREDICT space propagation model in 1998.  Dr. Matney based his work on the original SPACETRACK Report #3 as well as a publicly released SGP4 model from Goddard Space Flight Center (1997).  Though Dr. Matney’s program outputs workable results, the Orbital Debris Program Office was intrigued by Vallado’s standardized code and wanted to explore the differences between codes.  Furthermore, Dr. Matney’s code did not accurately predict the location of the objects in some instances, though specific cases were not supplied for analysis.

This project, made possible through a research contract between JSC and Cal Poly San Luis Obispo, explores the differences in the two propagation codes.  The scope of the project is as follows:

- Quantify the errors in the two codes based on ‘truth’ data.

- Deliver new PREDICT code integrated with updated SGP4 module.

- Deliver SGP4 propagator in MATLAB.

All analysis and deliverables are contained within this final report, as well as user guides for attached software code.

# 4.  Procedure

This project was divided into three distinct stages.

- Understanding the software.

- Analyzing differences.

- Programming deliverables.

## 4.1 Understanding the Software

The starting point of this phase was David Vallado’s (et al) AIAA paper, Revisiting Spacetrack Report #3.  In this paper, updates to the code are described in detail, and a link to the code in both FORTRAN and MATLAB is available at:

http://CelesTrak.com/software/vallado-sw.asp

In Vallado’s analysis, results from the new code are compared against selected results from existing public code (i.e. GSFC, JPL, T.S. Kelso’s FORTRAN).  Though there are obvious differences, there is no proof that the new code is closer or further from the truth.  What the paper does do is analyze each difference and explain why the results differ.  These differences are commented as ‘fixes’ within the code and are cataloged in detail in Vallado’s paper.

The code that Vallado created first prompts users for an operations mode.  This selects how the code calculates epoch of the moon and sun for lunar/solar perturbations.  Next, the code prompts users to input the type of run.  Of the three choices, the first is catalog mode, where the code displays position, velocity, and COES for 12 hours forwards and backwards in time with a step of 20 minutes.  Second is verification mode, designed to test the output of a given two line element (TLE) set and verify the results against a given file.  Third is manual mode, where users define the propagation parameters.  If manual mode is chosen, the user gets to determine if they want to use minutes from epoch, epoch, or day of year approach to define the window of time to analyze.  The code itself converts epoch and day of year inputs to minutes from epoch when calling the propagator.  Next, the user chooses which Earth constants to use, WGS-72 or WGS-84 referring to the World Geodetic System constants and when they became standard.  Though the WGS-84 numbers are the current standard (through 2010), WGS-72 values are used in most SGP4 propagators because the 1984 values were not available at the time of creation.  Though it is unknown what values SATRAK uses, analysis shows that WGS-72 numbers give more accurate results than WGS-84.  Finally, the code calls for TLE inputs.  The TLE file must be in the same directory as the main program, and the format of the TLE must be congruent with standards.  Multiple TLEs may be in the same file with non TLE lines marked with a ‘#’ symbol.  The program then opens or creates an OUT file and prints the data.  The OUT file has a header line containing the satellite number.  Following lines are in the following form:  a column for minutes from epoch, three columns of position data, three columns of velocity data, and four columns of date and time, delineated by a space.

### 4.1.1 MATLAB Code

The MATLAB code provided by Vallado contained some bugs.  There were two major issues encountered.  The first involved calling an undefined subroutine.  The code tried to call a subroutine called ‘days2mdhms’, but the subroutine needed was named ‘days2mdh’.  This subroutine had inputs of the year and day of year, and output the month, day, hour, minute, and second.  After this bugged was corrected, another problem was encountered with time.  The propagator uses minutes since epoch as an input, but instead was receiving days since epoch.  This bug was corrected by dividing critical numbers by 1440 (minutes in a day), and reformatting the output to reflect changes.  This code was tested in this debugged form and will be delivered to NASA in its current form to be used as an alternative to SATRAK.

### 4.1.2 FORTRAN Code

Before working on the FORTRAN code a compiler was needed.  The chosen compiler was a free compiler available online called Silverfrost.

(http://download.cnet.com/Silverfrost-FTN95/3000-2069_4-10491439.html)

The code is written in fixed-format FORTRAN 90 language.  Additionally, the complete code was spread amongst 25 small .FOR files and 4 common files upon initial download.  Based on my understanding of the MATLAB code, I was able to collect all the relevant subroutines and create a new .FOR file that provided the same functionality as the MATLAB code.  The code itself did not have any major bugs like the MATLAB code; however, significant time was spent formatting the combined code to fit the fixed-format FORTRAN standards.

### 4.1.3 PREDICT Code

JSC Orbital Debris Program Office provided their PREDICT code written in FORTRAN.  The code is written as a series of subroutines.  Though the focus was directed towards the SGP4 subroutine, the interactions with other parts of the program also needed to be taken into account.  The SGP4 subroutine is called with a COES input and a time from epoch (when the COES was taken).  The output is a single position and velocity vector based on the time from epoch.  The code itself is relatively simplistic.  It only takes into account J2 perturbations (the first mode of Earth’s spherical harmonics), and then uses Kepler’s solution with Newton’s method to solve for position and velocity.

## 4.2 Analyzing Differences

JSC had three distinct test cases to analyze that were all debris in geosynchronous orbits.  For these cases (8832, 25000, and 30000), JSC sent a TLE and SATRAK truth data.  The truth data consists of a data set with 60 minute intervals between predictions and a data set with 360 minute intervals between predictions – but with a larger time range.  These objects are of particular importance to the Orbital Debris. All three objects are pieces of debris from a Titan 3C transtage launch vehicle.  The pieces themselves are not exactly in GEO, but are relatively close in a subGEO orbit.  Below describes the break-up of objects 25000 and 30000.

The breakup of the Titan 3C-4 transtage occurred on  21February 1992 at an altitude of ~35,600 km, inclination (INC) of 11.9 degrees and a right ascension (RA) of 21.8 hours.  The operator of the GEODSS sensor on Maui, Hawaii witnessed approximately 20 pieces in the breakup, but none were tracked at the time.  Subsequent to the breakup, the U. S. Space Surveillance Network (SSN) identified three pieces of the debris and assigned them to the catalogue as SSN25000, SSN25001 and SSN30000.

Figure 1 - 8832 Graphical Orbit Data

Figure 2 - 25000 Graphical Orbit Datavi

Figure 3 - 30000 Graphical Orbit Datavi

Next, to further the analysis, TLEs were gathered from Celestrak for non-geosynchronous orbits.  Then, the same TLEs were again gathered on a later date.  Analysis then compared where the object is based on the latter TLE, with where the object was predicted to be based on the former TLE.  Analysis was completed on both the MATLAB and FORTRAN codes, as well as JSC’s code.

### 4.2.1 Objects 8832, 25000, and 30000

First, each TLE was processed through the MATLAB and FORTRAN versions of Vallado’s code and compared side by side with the SATRAK data.  The MATLAB and FORTRAN codes can be programmed to have the same output as the SATRAK data.  Results were first compared by hand, and then further analyzed using Excel.  Error between SATRAK and Vallado’s code is quantified as the absolute magnitudes of difference in the position and velocity vectors.

#### 4.2.1.1 Code Modification

Since the ultimate goal of the project is to replace the current SGP4 propagator in JSC’s code, the next step was to modify the FORTRAN code to act as a subroutine, instead of a stand-alone executable, with the same input and output variables as the original PREDICT model.  Because the PREDICT model called for less functionality than the new code provided, much of the work done came in deleting unnecessary lines of code.  New lines of code were added to remove any prompts given to the user; however, because the new code can accept LEO objects, a B* prompt was added to the process.  There are comments within the new code with instructions on how to remove this if the user does not find it necessary.

Next, a simple program was created in which the user defines the COES from the TLE and gives a ‘minutes from epoch’ value.  The program then calls the propagator and prints the expected position and velocity on the screen.

At this point, the inputs and outputs to the FORTRAN propagator and the PREDICT propagator were the same, and it was trivial to copy and paste the SGP4 subroutine from PREDICT into a new program called by the same simple program from above.

#### 4.2.1.2 SATRAK Comparison

The objects given by JSC were also run through the modified program.  However, since the new programs were restrictive in their input and output, only select data points were analyzed until trends were discovered.

### 4.2.2 Other Objects

To complete the analysis, truth data for LEO and MEO objects were found on SATRAK.  These objects were:

- Polysat CP3 (31129)

- Polysat CP4 (31132)

- Iridium 33 Debris (33771)

- LES 9 (08747)

- ISS Tool bag (33442)

The analysis was completed using the modified code because of the single input and output.  Error from truth data is again expressed as the absolute magnitude differences in position and velocity vectors.

## 4.3 Programming Deliverables

The final step of the procedure was installing the new code into the original PREDICT code.  The majority of the leg work had already been completed in the analysis phase, though there were still some issues to be resolved.  First, some of the variable and subroutine names needed to be altered to work with the PREDICT code.  Specifically, both the PREDICT code and the new code have a subroutine labeled ‘JDAY’.

Next, through analysis, it was found that the replacement code needs an extra bit of data to work properly.  The PREDICT code only looks at J2 effects, which use a statistical average of spherical harmonic forces to determine perturbations.  This makes epoch data irrelevant because it doesn’t matter when you start your calculation, it only matters how long the J2 force has been acting on the object.  Thus, when the propagator is called in the PREDICT model, it only inputs time from epoch and not the epoch itself.  In contrast, the new code not only takes into account J2 effects, but also looks at forces on the object from lunar and solar influences.  These effects are heavily dependent on epoch data because the location of the moon and sun need to be known to correctly calculate the direction and magnitude of the forces.

Epoch data for objects analyzed by the PREDICT code is determined in another subroutine, which is a predecessor for the SGP4 subroutine.  To solve the problem, a global common file was created to pass the necessary data to the new SGP4 routine.  Details of this can be found in comments in the code and in the User’s Guide portion of this report.

Complete testing of the PREDICT code with the modified SGP4 code will be completed by the Orbital Debris Program Office.

# 5. Results

Sections 5.2 through 5.5 graphically display results that represent the magnitude of the difference in position and velocity vectors between the output of various SGP propagators and SATRAK output over the time span defined by given SATRAK truth data.  The given time span ranges from ~15 days to ~27 days. Each section highlights results from the four main codes that were generated from this project.  Furthermore, analysis of the results are presented after each section.

Sections 5.6 through 5.8 show the results of the four objects not in GEO orbit, with truth data taken from the TLE data provided by Celestrak.

Section 5.9 links the results to real-world application.

## 5.1 Data Points

All results were generated from TLEs for the following objects:

- 8832 (Titan debris)

- 25000 (Titan debris)

- 30000 (Titan debris)

- 31129 (Poly Sat 3)

- 31132 (Poly Sat 4)

- 33771 (Iridium 33 debris)

- 33442 (ISS tool bag debris)

## 5.2 MATLAB vs. SATRAK

Results are displayed for both scenarios created in SATRAK (60 and 360 minute intervals).

### 5.2.1 Object 8832

The maximum position error calculated between SATRAK and the MATLAB results is 32 cm and the maximum velocity error calculated is 0.023 mm/sec.

Figure 4 - 8832 Position Error MATLAB (60 min intervals)

Figure 5 - 8832 Velocity Error MATLAB (60 min intervals)

Figure 6 - 8832 Position Error MATLAB (360 min intervals)

Figure 7 - 8832 Velocity Error MATLAB (360 min intervals)

### 5.2.2 Object 25000

The maximum position error calculated between the SATRAK and MATLAB results is 1.5m and the maximum velocity error calculated is .11 mm/sec.

Figure 8 - 25000 Position Error MATLAB (60 min intervals)

Figure 9 - 25000 Velocity Error MATLAB (60 min interval)

Figure 10 - 25000 Position Error MATLAB (360 min interval)

Figure 11 - 25000 Velocity Error MATLAB (360 min interval)

### 5.2.3 Object 30000

The maximum position error calculated is 51 cm and the maximum velocity error calculated is 0.038 mm/sec.

Figure 12 - 30000 Position Error MATLAB (60 min interval)

Figure 13 - 30000 Velocity Error MATLAB (60 min interval)

Figure 14 - 30000 Position Error MATLAB (360 min intervals)

Figure 15 - 30000 Velocity Error MATLAB (360 min intervals)

### 5.2.4 Analysis - MATLAB

In a broad sense, the error between SATRAK and the MATLAB outputs are insignificant.  For practical uses of this data, JSC is looking at units of kilometers, which is orders of magnitudes larger than the error given by MATLAB (1.5 m maximum).  However, in each case, there is an obvious periodic variation which either corresponds to the orbital period of the satellite or rotation of the Earth.  This can be explained by additional types of perturbations that are modeled in SATRAK that are ignored in Vallado’s code.  The hypothesis is that SATRAK encompasses some form of triaxilary effects; though there is no way to test this hypothesis.

Next, the difference between the 60 minute intervals and 360 minute intervals graphs was examined to check for contradictory data (see figure 16) by overlaying data for object 8832).  As expected, the two graphs show the same long periodic trend; however, the 60 minute interval captures some short-term perturbations as well.  The order of magnitude difference between the two intervals is less than the initial error and can safely be ignored.  Thus, there is no contradictory data.  Similar analysis was performed on all subsequent objects and codes with similar results and therefore, are not shown.

Figure 16 - Comparison of 60 and 360 minute intervals

## 5.3 FORTRAN vs. SATRAK

Results are displayed for both scenarios created in SATRAK (60 and 360 minute intervals).

### 5.3.1 Object 8832

The maximum position error calculated is 32 cm and the maximum velocity error calculated is 0.028 mm/sec.

Figure 17 - 8832 Position Error FORTRAN (60 min intervals)

Figure 18 - 8832 Velocity Error FORTRAN (60 min intervals)

Figure 19 – 8832 Position Error FORTRAN (360 min intervals)

Figure 20 – 8832 Velocity Error FORTRAN (360 min intervals)

### 5.3.2 Object 25000

The maximum position error calculated is 1.5 m and the maximum velocity error calculated is 0.11 mm/sec

Figure 21 - 25000 Position Error FORTRAN (60 min intervals)

Figure 22 - 25000 Velocity Error FORTRAN (60 min intervals)

Figure 23 - 25000 Position Error FORTRAN (360 min intervals)

Figure 24 - 25000 Velocity Error FORTRAN (60 min intervals)

### 5.3.3 Object 30000

The maximum position error between the FORTRAN and SATRAK results is 53 cm and the maximum velocity error is 4.4 mm/sec.

Figure 25 - 30000 Position Error FORTRAN (60 min intervals)

Figure 26 - 30000 Velocity Error FORTRAN (60 min intervals)

Figure 27 - 30000 Position Error FORTRAN (360 min intervals)

Figure 28 - 30000 Velocity Error FORTRAN (360 min intervals)

### 5.3.4 Analysis - FORTRAN

The FORTRAN code outputs very similar data to the MATLAB code in terms of position (see below).  From visual inspection the two sets of data are indistinguishable.

Figure 29 - Comparison of MATLAB and FORTRAN results (8832 position)

There is slightly more difference in the velocity output, see figure 26 (object 8832).  The cause of the variation in the output of the FORTRAN code is unknown, but consistent with results found with object 25000.  An initial hypothesis was a difference in the precision of the numbers carried through the FORTRAN calculations versus the MATLAB calculations; however, further results from the next section show otherwise.  Though the fluctuations look significant, please remember that the graph is displaying differences of .004 mm/sec between the output of the data which is describing the motion of an object traveling roughly 3 km/sec.

Figure 30 - Comparison of MATLAB and FORTRAN (8832 Velocity)

## 5.4 Modified Code vs. SATRAK

Results are displayed for both scenarios created in SATRAK (60 and 360 minute intervals).  The modified code (labeled Vallado in the graphs) is designed to simulate the inputs and outputs of NASA JSC’s existing code.  Due to increased complexity, epoch data is also included as an input to the system.  This leads to further complications when installing the upgrade into the PREDICT code, which is discussed in a later section.

### 5.4.1 Object 8832

The maximum position error calculated is 32 cm and the maximum velocity error calculated is 0.024 mm/sec.

Figure 31 - 8832 Position Error Vallado (60 min intervals)

Figure 32 - 8832 Velocity Error Vallado (60 min intervals)

Figure 33 – 8832 Position Error Vallado (360 min intervals)

Figure 34 - 8832 Velocity Error Vallado (360 min intervals)

### 5.4.2 Object 25000

The maximum position error calculated is 1.5 m and the maximum velocity error calculated is .11 mm/sec.

Figure 35 - 25000 Position Error Vallado (60 min intervals)

Figure 36 - 25000 Velocity Error Vallado (60 min intervals)

Figure 37 - 25000 Position Error Vallado (360 min intervals)

Figure 38 - 25000 Velocity Error Vallado (360 min intervals)

### 5.4.3 Object 30000

The maximum position error calculated is 6.3 m and the maximum velocity error calculated is 0.47 mm/sec.

Figure 39 - 30000 Position Error Vallado (60 min intervals)

Figure 40 - 30000 Velocity Error Vallado (60 min intervals)

Figure 41 - 30000 Position Error Vallado (360 min intervals)

Figure 42 - 30000 Velocity Error Vallado (360 min intervals)

### 5.4.4 Analysis – Modified Code

Since the Vallado code is a stripped down version of the FORTRAN code, it is expected that the results come out equal or close to equal.  Looking at the resultant graphs for object 8832 and object 25000 show some noteworthy trends.  First, that the position error for the full FORTRAN code provides results slightly closer to truth.  In the two cases, FORTRAN code is consistently fractions of a millimeter better.  Why this happens is unknown.  Second, the velocity error is more similar to the MATLAB error than the FORTRAN error.  The reason for this is also unknown, but can be seen in figure 39.

Figure 43 - Comparison of Vallado to MATLAB (8832 Velocity Error)

Finally, the results of object 30000 do not match the pattern initiated by the first two objects.  The error involved is much greater than other runs and though has periodic tendencies, adheres more towards linear growth.  Even though the error is larger than other runs, it is still orders of magnitude less than the operating threshold.

## 5.5 JSC Code vs. SATRAK

The JSC SGP4 subroutine code was taken directly from October 2008 version of the PREDICT code.  In these tests, it is called by the same program that calls the modified code.  In the resultant graphs, the JSC code is labeled as Matney, after Dr. Matney who wrote the original code.

### 5.5.1 Object 8832

The maximum position error calculated is 356.1 km and the maximum velocity error calculated is 25.4 m/sec.

Figure 44 - 8832 Position Error Matney (60 min intervals)

Figure 45 - 8832 Velocity Error Matney (60 min intervals)

Figure 46 - 8832 Position Error Matney (360 min intervals)

Figure 47 - 8832 Velocity Error Matney (360 min intervals)

### 5.5.2 Object 25000

The maximum position error calculated is 137.8 km and the maximum velocity error calculated is 10 m/sec.

Figure 48 - 25000 Position Error Matney (60 min intervals)

Figure 49 - 25000 Velocity Error Matney (60 min intervals)

Figure 50 - 25000 Position Error Matney (360 min intervals)

Figure 51 - 25000 Velocity Error Matney (360 min intervals)

### 5.5.3 Object 30000

The maximum position error calculated is 61.2 km and the maximum velocity error calculated is 4.4 m/sec.

Figure 52 - 30000 Position Error Matney (60 min intervals)

Figure 53 - 30000 Velocity Error Matney (60 min intervals)

Figure 54 - 30000 Position Error Matney (360 min intervals)

Figure 55 - 30000 Velocity Error Matney (360 min intervals)

### 5.5.4 Analysis – JSC Code

From visual inspection and direct comparison with the three previous codes, it is easy to see that the JSC code does not provide the same level of accuracy in the output.  It is  suspected that there are two main reasons for this.  First, it is believed that there is a fundamental flaw in the programming of Kepler’s solution.  This is skewing output results even before any perturbations are taken into account.  Second, the JSC code does not account for any third body interactions.  This causes the periodic variations seen in the graphs, as sometimes the third body effects help the simulation while at other times it hurts the simulation.

Furthermore, the scope of the error is of similar magnitude to the threshold, and thus cannot be ignored like the previous three cases.  This will be discussed further in Section 5.9 Operational Issues.

## 5.6 Polysat

Polysats are small satellites designed by Cal Poly under the Cube Sat project..  CP3 and CP4 were launched into 640 km sun-synchronous orbits on April 17, 2007.  For the Polysats, there were two sets of truth data.  First, there was SATRAK data of the CP3 available.  Second, there was TLE data provided by Celestrak.  Using archived and current TLE data, it is possible to see how far away the prediction is from official data.

For this analysis, only the modified code was compared to truth data.  This is to check the operation potential of the new software beyond GEO objects.  The original code was not tested because it was not designed for LEO or MEO objects.

Figure 56 - CP3 Graphical Orbit Datavi

Figure 57 - CP4 Graphical Orbit Datavi

### 5.6.1 CP3

Below are graphs showing the magnitude of the vector error in both position and velocity of the modified code compared to SATRAK truth data.

Figure 58 - CP3 Position Error Vallado (10 sec intervals)

Figure 59 - CP3 Velocity Error Vallado (10 sec intervals)

These graphs show two trends.  First, the variation in error is a function of the objects period and not of the Earth’s rotation.  The periodic motion of both the position and velocity corresponds to the mean motion of the spacecraft.  Second, the slope of the error trends is also proportional to the mean motion of the object.  In other words, the faster the object is moving, the faster error will be a significant factor.

Beyond SATRAK, there is also TLE data that can be used as truth data.  TLE’s were pulled from Celestrak at four different times and are shown in the table below.

Table 1 - TLE readings for CP3

| Date of Reading | TLE |
| --- | --- |
| 4/22/09 | 1 31129U 07012N 09112.08372896 .00000092 00000-0 31739-4 0 8719 2 31129 97.9954 167.8345 0103695 98.8601 262.4373 14.52123890106741 |
| 5/1/09 | 1 31129U 07012N 09121.11032672 .00000119 00000-0 37832-4 0 8897 2 31129 97.9938 176.4337 0103587 70.9779 290.2602 14.52126376108059 |
| 5/6/09 | 1 31129U 07012N 09126.14038987 .00000017 00000-0 14486-4 0 9001 2 31129 97.9934 181.2249 0103743 55.7944 305.3039 14.52126523108786 |
| 5/23/09 | 1 31129U 07012N 09143.15980035 .00000024 00000-0 16378-4 0 9438 2 31129 97.9925 197.4331 0102509 3.5446 356.6451 14.52130919111251 |

Table 2 - CP3 TLE Results

| Days since 4/22 TLE | Error in Position (km) | Error in Velocity (km/sec) |
| --- | --- | --- |
| 9.03 | 0.638 | 0.008662 |
| 14.06 | 2.802 | 0.002583 |
| 31.08 | 9.197 | 0.000703 |

These results are significantly better than expected given the SATRAK analysis.  This shows that SATRAK data and TLE data do not necessarily match.  Based on this, the modified code may give more accurate results than SATRAK.  If the new code contains more modeled perturbations than SATRAK, this would also explain the periodic variations seen in most of the preceding figures.

### 5.6.2 CP4

Table 3 - CP4 TLE Data

| Date of Reading | TLE |
| --- | --- |
| 4/22/09 | 1 31132U 07012Q 09112.17610138 .00000080 00000-0 27710-4 0 5883 2 31132 97.9989 171.5508 0086862 87.9115 273.2019 14.55199299106882 |
| 5/1/09 | 1 31132U 07012Q 09120.15222967 .00000158 00000-0 45009-4 0 5943 2 31132 97.9986 179.1896 0086768 63.3890 297.6162 14.55201137108040 |
| 5/23/09 | 1 31132U 07012Q 09143.18655556 -.00000231 00000-0 -40476-4 0 6101 2 31132 097.9973 201.2422 0085319 352.7051 007.2899 14.55204047111390 |

Table 4 - CP4 Results

| Days since 4/22 TLE | Error in Position (km) | Error in Velocity (km/sec) |
| --- | --- | --- |
| 7.98 | 0.5924 | 0.000514 |
| 31.01 | 12.253 | 0.0115 |

The CP4 results are similar to the CP3 results.  The error in both position and velocity are of similar magnitude after similar amounts of time.  From this, a general rate of error propagation can be assessed.

## 5.7 ISS Tool bag

The ISS Tool bag was a piece of debris dropped by astronauts working on the outside of the International Space Station.  The ISS is in orbit at approximately 350 km altitude.  The ISS Tool bag is currently in orbit at approximately 300 km altitude due to drag perturbations and lack of station keeping.

Figure 60 - ISS Tool bag Graphical Orbit Datavi

The following TLE’s were downloaded from Celestrak.

Table 5 - ISS Tool bag TLE Data

| Date of Reading | TLE |
| --- | --- |
| 5/1/09 | 1 33442U 98067BL 09120.27106401 .00062601 00000-0 24248-3 0 2271 2 33442 51.6371 184.2223 0003583 339.3988 20.6682 15.86587612 25511 |
| 5/6/09 | 1 33442U 98067BL 09125.49770747 .00067504 00000-0 25238-3 0 2356 2 33442 51.6390 156.8544 0001434 341.7735 18.3083 15.87287894 26344 |
| 5/23/09 | 1 33442U 98067BL 09143.17121749 .00091180 14199-4 29936-3 0 2571 2 33442 51.6360 64.0961 0002463 89.7331 270.1497 15.90545869 29150 |

Table 6 - ISS Tool bag Results

| Days since 4/22 TLE | Error in Position (km) | Error in Velocity (km/sec) |
| --- | --- | --- |
| 5.23 | 33.645 | 0.0388 |
| 22.90 | 3041.2 | 3.519 |

The SGP4 propagator breaks down quickly when dealing with extremely low orbits.  After 5 days, the error in position has already become significant and after 23 days, the projection is useless.  The error in the projection is caused by an ever changing B* term in the TLE, or the term used to approximate drag.  The force of drag is changing constantly due to the object falling further and further into the atmosphere.  Propagation for LEO objects should be limited to short periods of time using the new code.

## 5.8 Iridium Debris

Iridium Debris is a piece of the Iridium 33 satellite that broke up in LEO.  The altitude of the debris is approximately 775 km, putting it in a similar orbit to the Polysats.

Figure 61 - Iridium Debris Graphical Orbit Data vi

Table 7 - Iridium Debris TLE Data

| Date of Reading | TLE |
| --- | --- |
| 4/22/09 | 1 33771U 97051J 09111.44856353 .00002894 00000-0 10166-2 0 511 2 33771 86.4092 92.3272 0016638 106.1355 254.1660 14.34588702 9984 |
| 5/1/09 | 1 33771U 97051J 09120.30665597 .00004031 00000-0 14163-2 0 587 2 33771 86.4081 88.6364 0017979 83.2643 277.0615 14.34660268 11251 |
| 5/23/09 | 1 33771U 97051J 09142.83354878 .00003334 00000-0 11665-2 0 776 2 33771 086.4045 079.2409 0018814 026.0516 334.1596 14.34826651 14481 |

Table 8 - Iridium Results

| Days since 4/22 TLE | Error in Position (km) | Error in Velocity (km/sec) |
| --- | --- | --- |
| 8.59 | 41.315 | 0.0444 |
| 31.39 | 470.04 | 0.4969 |

## 5.9 Operational Issues

There are two main issues to consider when investigating whether or not to switch to the new system.  First, how will the improvements in the code affect the program effectiveness?  Second, how will the additions to the code affect the speed of operation?

### 5.9.1 Program Effectiveness

Working with the assumption that the original JSC code provides ‘good enough’ answers, the following estimates how long the JSC code could successfully track the satellite.  If this range of time falls within the scope of program operations, then JSC would not be able to differentiate between the old and new SGP4 versions.

To calculate this estimation, there were some simplifying assumption made.  First, NASA uses a telescope with a 2 degree square FOV to initially find the object.  It then uses the PREDICT software to try to find the same object with a 0.2 degree square FOV telescope.  For this estimation, a 0.2 degree round FOV was assumed.  This means that the threshold for the angle away from the position truth vector is half of this, or 0.1 degrees.  Second, the angle from the center of the Earth was calculated rather than from some point on the surface of the Earth.  This assumption has both the ability to help or harm the prediction based on the vector error and where the telescope actually is, but should give a reasonable answer.  Third, since SATRAK data was not available for all the points needed, the MATLAB data was used as truth.  Earlier tests showed that MATLAB provided accurate outputs consistently.

Below are graphs of the three objects defined by NASA JSC showing the angle at which the object is from where the old SGP4 program tells the telescope to point.  A threshold line of 0.1 degrees is also drawn for a point of comparison.

Figure 62 - 8832 Operational Effectiveness

Figure 63 - 25000 Operational Effectiveness

Figure 64 - 30000 Operational Effectiveness

All three cases show that the existing SGP4 propagator breaks down and is no longer effective after a matter of days.  From these limited examples, it takes 12 days for the first object to leave the FOV.  Furthermore, these examples show that the propagator is most effective between three and nine days.  What is troubling about these graphs, mentioned briefly earlier, is the fact that at time zero, the telescope will not be centered on the target.  Again, it is believed this to be caused by a flaw in programming Kepler’s solution.  The above graphs can be compared to the following:

Figure 65 - 30000 Operational Effectiveness (new code)

The error with the new code is essentially zero throughout the entirety of the simulation and is difficult to read in the above chart.  Object 30000 offered the poorest results and thus was chosen as a point of comparison.  The new code offers significant improvement over the old code and will stay within threshold limits indefinitely.  Additionally, even in short term situations, the new code offers superior accuracy.

Below is the operational effectiveness of the new code in relation to Polysat 3 (CP3):

Figure 66 - CP3 Operational Effectiveness

Though the values only extend to roughly 5 days, extrapolating the data using a quadratic suggests that the modified code will be able to keep the object in the FOV for roughly 15 days.  Using the TLE truth data points to a different model.  Using data between 4/22/09 and 5/23/2009 which is roughly 31 days, shows that the object will be about .071 degrees away from the center of the FOV – still within threshold limits.  This shows that the growth in error is linear instead of polynomial, suggesting that the modified code will be able to keep the object in the FOV for roughly 40 days.

### 5.9.2 Operational Speed

Comparing lines of code, the new program contains 4475 lines with comments and blank lines, while the existing code only contains 2510 lines.  No noticeable difference was encountered when running the two programs.

# 6. Conclusion

The preceding results lead to the following conclusions:

In reference to the original question of whether or not NASA JSC should change their system to incorporate the new Vallado code, the answer is yes.  This should be done for two distinct reasons.  First, the modified code is more accurate by orders of magnitude over a much longer period of time.  Second, the old code provides flawed answers off by kilometers even at time zero.  Though the old code provides ‘good enough’ answers for periods up to about one week, this assumes that the input data is accurate.  In reality, the nature of the input data may contain significant errors as well.  This is because the code uses right ascension and declination data separated by brief periods of time (less than 5 minutes), thus the object being followed does not move far and the right ascension and declination change only slightly.  Calculating COES from this data is limited to the accuracy and the significant figures provided by the telescope data.  In such, the ‘good enough’ answers provided by the old code may not be ‘good enough’ when compounded with errors in telescope readings.

From the analysis, it is apparent that Vallado’s code does not exactly match the output of SATRAK, though it is close.  The periodic nature of the error between the two codes, which mimics the period of the orbiting object around the Earth, implies that there are perturbations or constants that differ between the two programs.  For this analysis, SATRAK data is taken as truth; however, the error in the SATRAK program is unknown.  It is possible that Vallado better predicts the position and velocity of orbiting objects.  For the purposes of this project, the two outputs are statistically identical.

If using the Vallado code to replace SATRAK, the MATLAB version of the code is more reliable than the FORTRAN version.  Even though the two programs yield indistinguishable results, the MATLAB version of the code is more intuitive and easier to follow.  Furthermore, FORTRAN proved more difficult to use as the program balked at times for unknown reasons.  Both programs have been provided for use on the attached CD with a User’s Guide in section 7.

Finally, the modified code has limitations based on the altitude of the satellite.  For objects in deep space orbits such as GEO, the code accurately predicts (to the same level as SATRAK) the position and the velocity of the object within threshold limits indefinitely.  In MEO, the effectiveness drops to roughly two weeks of predictions within the threshold limits.  In LEO, the effectiveness drops to about one day of predictions within the threshold limits.  Thus, the new code will work for all orbiting objects; however, it is important to keep in mind the operational timeline of activities.  Furthermore, for LEO and MEO objects, the B* term needs to be added to the system, or else the accuracy of the results may decrease.  Improvements to the PREDICT code must be made to accommodate this before use.

# 7. User’s Guide

This section provides instructions on how to install and run the three different codes discussed in this paper.  Sections 7.1 and 7.2 describe the SATRAK replacement codes written in MATLAB and FORTRAN.  Section 7.3 describes how to incorporated the modified code into the existing NASA JSC PREDICT  code as well as the various options the user has within the code itself.

## 7.1 MATLAB

**Note that the output file will be over-written each time the program is run unless the filename is changed.**

The MATLAB code mimics the SATRAK code and outputs very similar results.  Work was done using MATLAB 7.0.

### 7.1.1 Installation

On the attached CD, there is a folder named SGP4 MATLAB.  Copy and paste this folder to MATLAB’s current directory.  The folder contains the following files:

- testmat.m	Driver script for testing and example usage

- sgp4.m 		Main sgp4 routine

- sgp4init.m 		Initialization routine for sgp4

- initl.m 			Initialization for sgp4

- dsinit.m 		Deep space initialization

- dspace.m 		Deep space perturbations

- dpper.m 		Deep Space periodics

- dscom.m 		Deep Space common variables

- twoline2rv.m 		TLE conversion routine

- angl.m 			Find the angle between two vectors

- constmath.m 		set mathematical constants

- days2mdh.m 		convert days to month day hour minute second

- getgravc.m 		Get the gravity constants

- gstime.m 		Find Greenwich sidereal time

- invjday.m 		Inverse Julian Date

- jday.m 			Find the Julian Date

- mag.m 			Magnitude of a vector

- newtonnnu.m 		Kepler’s iteration given eccentricity and true anomaly

- rv2coe.m 	Convert position and velocity vectors to classical orbital elements

- SGP4-VER.TLE	Verification file

- Sgp4_CodeReadme.pdf

### 7.1.2 Verification Run

- Run testmat.m

- Enter opsmode ‘i’ for improved method

- Enter typerun ‘v’ for verification mode

- Enter constant ‘72’ corresponding to WGS-72 standards

- Enter file name ‘SGP4-VER.TLE’ – see appendix 1 for file information

- The program should output a file called ‘tmatver.out’ can be compared to the results found in appendix 2 for verification that the program is running properly.

### 7.1.3 TLE inputs

The program does not discriminate against file types when opening TLEs, though it has only been tested with .TXT and .TLE file extensions.  Make sure that the input TLE does have proper the proper format.  A simple way to load a TLE is to open a new script in MATLAB and copy and paste the desired TLE, then save the script with a .TLE file extension.

The program is capable of handling multiple TLEs in a single run and will output the file with a header representing the satellite number before each set of data.  When formatting a multiple TLE input, use a ‘#’ symbol to begin each line that does not contain TLE data.  This tells the computer to skip the line and move to the next line.

### 7.1.4 Catalog Mode

Catalog mode takes a TLE input and outputs data for -24 hours to +24 hours with 20 minute intervals.  This is useful for short term propagations of many objects (i.e. multiple TLEs).

- Run ‘testmat.m’

- Enter ‘a’ for afspc or ‘i’ for improved method

- For type of run, enter ‘c’ for catalog mode

- Enter TLE data – file must be completely written with the file extension name

- A new file should be created with the name ‘tmatall.out’ with the catalog data.

### 7.1.5 Manual Mode

Manual mode gives the user the opportunity to define when to start and stop calculating and at what time intervals.  To do this, the user is given three options to define time:

- Minutes from epoch

- Epoch

- Day of Year

The minutes from epoch approach assumes that the epoch is defined by the time stated within the TLE.  Simply input (in minutes) how much time you want to elapse before the first data point, and how much time you want to elapse before the last data point.  The epoch approach asks for a start date and time and a stop date and time.  Finally, the day of year approach asks for a year and a day of the year.  The day of the year can be in decimal form to indicate a fraction of a day.  For all three approaches, the reference time frame is UTC.  Furthermore, whatever mode chosen will prompt for the time step that will be used (in minutes).

**Note, when working with multiple TLEs in manual mode, the user will be prompted for start and stop points and time step for each individual TLE.**

- run ‘testmat.m’

- Enter ‘a’ for afspc or ‘i’ for improved mehod

- For type of run, enter ‘m’ for manual operation

- Enter how the start and stop points are determined

  - m = minutes from epoch

  - e = epoch (year, month, day, hour, min, sec)

  - d = day of year (year, day)

- Enter constant choice (72 gives answers closest to truth)

- Enter TLE filename

- Follow prompts on screen to input start and stop points and time step

- Program sends output to file named ‘tmat.out’

### 7.1.6 Understanding the Output File

The output comes without column headers, so it is important to know what the values represent.  The first line is a header line with information about the satellite number.  When working with multiple TLEs, data from each TLE is separated by a similar header with the satellite number.  For each satellite, there are 7-13 columns of data (columns 8-13 are date and time data which is not calculated for catalog mode):

Column 1	Minutes from Epoch

Column 2	X-Position

Column 3	Y-Position

Column 4	Z-Position

Column 5	X-Velocity

Column 6	Y-Velocity

Column 7 	Z-Velocity

Column 8 	Year

Column 9	Month

Column 10	Day

Column 11	Hour

Column 12	Minute

Column 13 	Second

**Note, the position vector is based in ECI coordinate frame.**

## 7.2 FORTRAN (Vallado Code)

The main FORTRAN program has many similarities with the MATLAB program and may reference certain sections.

**Note that the output file will be over-written each time the program is run unless the filename is changed.**

### 7.2.1 Installation

On the attached CD, there is a folder labeled SGP4 FORTRAN.  Copy and paste the folder to a working directory.  The files contained in the folder are:

ASTMATH.CMN		Common file with mathematical constants

SGP4.CMN			Common file with constants applicable to SGP4 model

TESTFOR.FOR		Fixed Format FORTRAN file

TESTFOR.EXE		Executable file of TESTFOR.FOR

Validation.OUT		Data to validate correct installation

SGP4-VER.TLE		Validation TLE Data

Either use the executable (compiled and built on Silverfrost), or compile and build the FORTRAN file using another compiler.  Correct installation should result in the program running in a command prompt.

### 7.2.2 Verification Run

** FORTRAN is case sensitive, please keep this in mind when entering text.**

- Run TESTFOR.EXE

- Select opsmode ‘i’

- Select run type ‘v’

- Input filename ‘SGP4-VER.TLE’

- Program will output a file named ‘tfor.out’ to the working directory

- Compare this output with data from the file labeled ‘Verification.OUT’

### 7.2.3 TLE Input

This function is identical to MATLAB.  See section 7.1.3.

### 7.2.4 Catalog Mode

Catalog mode takes a TLE input and outputs data for -24 hours to +24 hours with 10 minute intervals.  This is useful for short term propagations of many objects (i.e. multiple TLEs).

- Run ‘TESTFOR.EXE – This should bring up a command prompt

- Enter ‘a’ for afspc or ‘i’ for improved method

- For type of run, enter ‘c’ for catalog mode

- Enter TLE filename– file must be completely written with the file extension name

- A new file should be created with the name ‘tfor.out’ with the catalog data.

### 7.2.5 Manual Mode

Manual mode gives the user the opportunity to define when to start and stop calculating and at what time intervals.  To do this, the user is given three options to define time:

- Minutes from epoch

- Epoch

- Day of Year

The minutes from epoch approach assumes that the epoch is defined by the time stated within the TLE.  Simply input (in minutes) how much time you want to elapse before the first data point, and how much time you want to elapse before the last data point.  The epoch approach asks for a start date and time and a stop date and time.  Finally, the day of year approach asks for a year and a day of the year.  The day of the year can be in decimal form to indicate a fraction of a day.  For all three approaches, the reference time frame is UTC.  Furthermore, whatever mode chosen will prompt for the time step that will be used (in minutes).

**Note, when working with multiple TLEs in manual mode, the user will be prompted for start and stop points and time step for each individual TLE.**

- run ‘TESTFOR.EXE’ – a new command prompt should appear

- Enter ‘a’ for afspc or ‘i’ for improved method

- For type of run, enter ‘m’ for manual operation

- Enter how the start and stop points are determined (Case sensitive)

  - M = minutes from epoch

  - E = epoch (year, month, day, hour, min, sec)

  - D = day of year (year, day)

- Enter constant choice (72 gives answers closest to truth)

- Enter TLE filename

- Follow prompts on screen to input start and stop points and time step

- Program sends output to file named ‘tfor.out’

### 7.2.6 Understanding the Output File

The output file created is in the same format at the MATLAB output.  Please see section 7.1.6 for more details.

## 7.3 Modified FORTRAN Code – For JSC

The modified FORTRAN code is designed to be compatible with the current PREDICT program; however, epoch data needs to be passed to the new program.  This section does not describe how to run the PREDICT code.  Rather, it explains how to integrate the modified code with the existing code.

### 7.3.1 Installation

On the attached CD in the folder ‘MODIFIED CODE’, there is a file called:

SGP4_Classic Subroutine.txt

Open this file and copy all of the contents.  Next in the PREDICT source code, locate the SGP4_Classic subroutine (line 853 – line 1116 in Feb. version).  Delete the entire subroutine and replace it with the text copied above.

To solve the epoch data problem, a global file called /global2/ is created in the surveychasenew subroutine.  In the line following the definition of /global1/ (or line 4314 after addition of the new code) add the following line ( represents a space):

COMMON /global2/ DATE, UT

Figure 67 - Code Addition to SURVEYCHASENEW

Next, in the same directory on the CD, there are two common files entitled:

- SGP4.CMN

- ASTMATH.CMN

Copy and paste these files into the directory with the PREDICT source code, as the new SGP4 propagator references these files.

Now, the updated PREDICT code will be ready to be compiled.

### 7.3.2 Code Options

The Vallado code contains various options that have been pre-programmed for the Orbital Debris Program Office use; however, these can changed if necessary.

#### 7.3.2.1 Constant Values

The program is set to use World Geodetic System (WGS) 72 parameters for Earth defined constants.  These parameters are legacy values from the first code release of SGP4, and best match SATRAK truth data.  The Vallado code is programmed to allow an update to the WGS 84 parameters, which are considered up-to-date until 2010.  To make this switch, find where ‘whichconst’ is defined in the SGP4_Classic subroutine (~line 925) and change the value from 72 to 84.

Figure 68 - Changing WGS values (1)

If manual changes are necessary to the constants, find the subroutine ‘getgravconst’ (~line 1012).  The constants for specific called cases are listed in various IF statements.  The user can define radius of Earth, J2, J3, and J4 harmonics for either the WGS 72 or WGS 84 system.

Figure 69 - Changing WGS values (2)

#### 7.3.2.2 B*

The B* term is used to estimate the drag force felt by the object in orbit.  For the Orbital Debris Program Office, this value has been set to zero since B* is not calculated by the PREDICT program and the objects in GEO do not experience significant drag forces.  There are two ways to change the default value of B*.  First, the user can define it as something other than zero inside the program.  To do this, find where B* is defined in the TwoLine2RVSGP4 subroutine (~line 1157).  Change the 0.0d0 value to desired B* value.  Second, the user can comment the above line out completely and uncomment the two lines above.  Doing this will prompt the user to input a B* term.  This may cause some problems in running the PREDICT code since it will prompt the user during every iteration of the SGP4 program.  To solve this, the user can comment out all three lines and define the B* term in the main program.  The term can then be accessed by the subroutine either by a global variable or by having the B* variable be called by the subroutine itself.

Figure 70 - Changing the B* term

# BIBLIOGRAPHY

- AIAA-2006-6753.zip 700 KB, Feb 16, 2009. ,

< http://www.centerforspace.com/downloads/>

- Chobotov, Vladimir A. Orbital Mechanics, Second Edition. Reston, VA, AIAA, Inc., 1996.

- Hoots, Felix R., et al. “History of Analytical Orbit Modeling in the U.S. Space Surveillance System.” Journal of Guidance Control, and Dynamics March-April 2004: 174-185.

- Hoots, Felix R. and Roehrich, Ronal L. “Models for Propagation of NORAD Element Sets.” SPACETRACK REPORT NO.3 December 1980: Packaged compiled by Kelso, T.S.: December 31, 1988. < ftp://ftp.amsat.org/amsat/docs/spacetrk.pdf>

- Kelso, T.S. “TLE Data”, 2008-2009. < http://celestrak.com/>

- Meissner, Loren P. Fortran 90.  Boston, MA, PWS Publishing Co., 1995

- “Selected Orbital Data” 2009. <http://www.heavens-above.com>

- Vallado, David A. Fundamentals of Astrodynamics and Applications, Third Edition.  Hawthorne CA, Microcosm Press, 2007.

- Vallado, David A., et al. “Revisiting Spacetrack Report #3.” 2006.

< http://celestrak.com/publications/AIAA/2006-6753/AIAA-2006-6753.pdf>

- “World Geodetic System.” Wikipedia May 11, 2009.

< http://en.wikipedia.org/wiki/WGS84>

# APPENDIX A.  SGP4 Code

All working codes are included in the accompanying CD.  The FORTRAN code designed to replace the legacy SGP4 propagator within PREDICT is displayed below for reference.

## A.1 Modified FORTRAN Code (Vallado)

This code compiles and runs on LAHEY express 7.2.  Output files are generated; however, when testing the code with PREDICT_Feb09, there were issues with rewriting a file at the end of the program.  There is no evidence that this is linked with the new SGP4 propagator.

*************************************************************************

*                Subroutine SGP_Classic

*

* code modified by Nicholas Miura

* Cal Poly - San Luis Obispo

* nicholas.miura@gmail.com

*

* subroutine should interface with existing NASA predictor code

*

* Inputs

*  DTDAYS = Time since final observation in Days

*  MMO = Mean motion at final observation

*  ECCO = Eccentricity at final observation

*  INCDEGO = Inclination at final observation

*  APDEGO = Argument of Perigee (Degrees)at observation

*  RAANDEGO = Right Ascension of Ascending Node (Degrees)at observation

*  MADEGO = Mean Anomaly (Degrees) at observation

*  MMDOTO2 = First time derivative

*  MMDDOTO6 = Second time derivative

*

* Outputs

*  POS = Position of Spacecraft

*  VEL = Velocity of Spacecraft

*  UP = Is spacecraft still in orbit?

*

* Called Subroutines

*  getgravconst

Subroutine SGP_Classic(DTDAYS,MM0,ECC0,

&  INCDEG0,APDEG0,RAANDEG0,MADEG0,

&  MMDOTO2,MMDDOTO6,POS,VEL,UP)

IMPLICIT NONE

c these declarations are copied from NASA Predictor code

c and define the input and output of the subroutine

REAL*8 DTDAYS,MM0,ECC0

REAL*8 INCDEG0,APDEG0,RAANDEG0,MADEG0

REAL*8 MMDOTO2,MMDDOTO6,EHour, ESec

Real*8 JDSatEpoch2

REAL*8 POS(1:3),VEL(1:3)

LOGICAL UP

DOUBLE PRECISION UT

c these declarations are used in the code

Character typerun, typeinput

Character*58 Longstring

Character*10 Date

Integer Code, NumSats, error, whichconst

Real*8 ro(3),vo(3)

* ----------------------------  Locals  -------------------------------

REAL*8 J2,TwoPi,Rad,mu, RadiusEarthKm, xke,

&         de2ra, xpdotp, T, sec, JD, pi, j3, j4, j3oj2, tumin

INTEGER EYr, EDay, EMon, EHr, EMin

INCLUDE 'SGP4.CMN'

COMMON /global2/ Date, UT

* ------------------------  Implementation   --------------------------

Opsmode = 'i'

typerun = 'm'

typeinput = 'M'

whichconst = 72

pi            =    4.0D0 * datan(1.0D0)  ! 3.14159265358979D0

TwoPi         =    2.0D0 * pi    ! 6.28318530717959D0

Rad           =   180.0D0 / pi   ! 57.29577951308230D0

DE2RA         =    pi / 180.0D0  ! 0.01745329251994330D0

xpdotp        =  1440.0 / (2.0 *pi)  ! 229.1831180523293D0

c sgp4fix identify constants and allow alternate values

CALL getgravconst( whichconst, tumin, mu, radiusearthkm, xke,

&       j2, j3, j4, j3oj2 )

c Read Date and Time

READ (Date(1:4),FMT='(I4)') EYr

READ (Date(6:7),FMT='(I2)') EMon

READ (Date(9:10),FMT='(I2)') EDay

CALL HMS (UT, EHr, EMin, ESec)

CALL JDAY1 (EYr, EMon, EDay, EHr ,EMin, ESec, JDSatEpoch2)

c Open a temporary file containing initialized orbital data

c Called by other subroutines

c        OPEN(115,FILE = 'Sgp4Rec.bak', ACCESS = 'DIRECT',

c     &          FORM = 'UNFORMATTED', RECL = 1100, STATUS = 'REPLACE' )

NumSats = 1

c  bring orbital elements to the new two line element converter - trick the system

CALL TwoLine2RVSGP4 ( NumSats,typerun,typeinput,whichconst,

&                        Code, DTDAYS,MM0,ECC0,

&                        INCDEG0,APDEG0,RAANDEG0,MADEG0,

&                        MMDOTO2,MMDDOTO6, JDSatEpoch2, Error )

c  Call the SGP4 propagator based on Time in minutes

T = DTDAYS*1440

CALL SGP4 ( whichconst, T, Ro, Vo, Error )

If (Error .eq. 1)Then

Up = .FALSE.

ENDIF

c the next WRITE line displays the result on the screen

c comment this out if you don't want to test the results

c       WRITE( *,800 ) T, ro(1),ro(2),ro(3),vo(1),vo(2),vo(3)

c  800  FORMAT(F17.8,3F17.8,3(1X,F14.9))

Pos = Ro

Vel = Vo

c      CLOSE(115)

c      STOP

END

* -----------------------------------------------------------------------------

*

*                           function getgravconst

*

*  this function gets constants for the propagator. note that mu is identified to

*    facilitiate comparisons with newer models.

*

*  author        : david vallado                  719-573-2600   21 jul 2006

*

*  inputs        :

*    whichconst  - which set of constants to use  721, 72, 84

*

*  outputs       :

*    tumin       - minutes in one time unit

*    mu          - earth gravitational parameter

*    radiusearthkm - radius of the earth in km

*    xke         - reciprocal of tumin

*    j2, j3, j4  - un-normalized zonal harmonic values

*    j3oj2       - j3 divided by j2

*

*    norad spacetrack report #3

*    vallado, crawford, hujsak, kelso  2006

*  ----------------------------------------------------------------------------

SUBROUTINE getgravconst ( whichconst, tumin, mu,

&            radiusearthkm, xke, j2, j3, j4, j3oj2 )

IMPLICIT NONE

REAL*8 radiusearthkm, xke, j2, j3, j4, j3oj2, mu, tumin

INTEGER whichconst

if (whichconst.eq.721) THEN

! -- wgs-72 low precision str#3 constants --

radiusearthkm = 6378.135D0     ! km

xke    = 0.0743669161D0

mu     = 398600.79964D0            ! in km3 / s2

tumin  = 1.0D0 / xke

j2     =   0.001082616D0

j3     =  -0.00000253881D0

j4     =  -0.00000165597D0

j3oj2  =  j3 / j2

ENDIF

if (whichconst.eq.72) THEN

! ------------ wgs-72 constants ------------

mu     = 398600.8D0            ! in km3 / s2

radiusearthkm = 6378.135D0     ! km

xke    = 60.0D0 / dsqrt(radiusearthkm**3/mu)

tumin  = 1.0D0 / xke

j2     =   0.001082616D0

j3     =  -0.00000253881D0

j4     =  -0.00000165597D0

j3oj2  =  j3 / j2

ENDIF

if (whichconst.eq.84) THEN

! ------------ wgs-84 constants ------------

mu     = 398600.5D0            ! in km3 / s2

radiusearthkm = 6378.137D0     ! km

xke    = 60.0D0 / dsqrt(radiusearthkm**3/mu)

tumin  = 1.0D0 / xke

j2     =   0.00108262998905D0

j3     =  -0.00000253215306D0

j4     =  -0.00000161098761D0

j3oj2  =  j3 / j2

ENDIF

RETURN

END  !  SUBROUTINE getgravconst

* -----------------------------------------------------------------------------

*

*                           SUBROUTINE TWOLINE2RVSGP4

*

*  this function converts the two line element set character string data to

*    variables and initializes the sgp4 variables. several intermediate variables

*    and quantities are determined. note that the result is a "structure" so multiple

*    satellites can be processed simultaneously without having to reinitialize. the

*    verification mode is an important option that permits quick checks of any

*    changes to the underlying technical theory. this option works using a

*    modified TLE file in which the start, stop, and delta time values are

*    included at the end of the second line of data. this only works with the

*    verification mode. the catalog mode simply propagates from -1440 to 1440 min

*    from epoch and is useful when performing entire catalog runs.

*

*  author        : david vallado                  719-573-2600    1 mar 2001

*

*  inputs        :

*    Numsats     - Number of satellites processed. It also becomes the record

*                  number for each satellite

*    typerun     - type of run                    verification 'V', catalog 'C',

*                                                 manual 'M'

*    typeinput   - type of manual input           mfe 'M', epoch 'E', dayofyr 'D'

*    whichconst  - which set of constants to use  72, 84

*    opsmode   - type of manual input           afspc 'a', imporved 'i'

*

*  outputs       :

*    Code        - EOF indicator. Code = 999 when EOF reached

*    startmfe    - starttime of simulation,       min from epoch

*    stopmfe     - stoptime of simulation,        min from epoch

*    deltamin    - time step                      min

*

*  coupling      :

*    days2mdhms  - conversion of days to month, day, hour, minute, second

*    jday        - convert day month year hour minute second into Julian date

*    sgp4init    - initialize the sgp4 variables

*

*  Files         :

*    Unit 10     - test.elm        input 2-line element set file

*    Unit 11     - test.bak        output file

*    Unit 15     - sgp4rec.bak     temporary file of record for 2 line element sets

*

*  references    :

*    norad spacetrack report #3

*    vallado, crawford, hujsak, kelso  2006

*------------------------------------------------------------------------------

SUBROUTINE TwoLine2RVSGP4 ( NumSats, Typerun, typeinput,

&                            whichconst, Code, DTDAYS,MM0,ECC0,

&                            INCDEG0,APDEG0,RAANDEG0,MADEG0,

&                            MMDOTO2,MMDDOTO6, JDSatEpoch1, Error )

IMPLICIT NONE

Character Typerun, typeinput

Integer Code, NumSats, whichconst

REAL*8 startmfe, stopmfe, deltamin

* ----------------------------  Locals  -------------------------------

REAL*8 J2, mu, RadiusEarthKm,VKmPerSec, xke, tumin

REAL*8 BC,EPDay, sec, xpdotp, j3, j4, j3oj2

REAL*8 startsec, stopsec, startdayofyr, stopdayofyr, jdstart,

&         jdstop, JDSatEpoch1

INTEGER startyear, stopyear, startmon, stopmon, startday,

&          stopday, starthr, stophr, startmin, stopmin

INTEGER Yr,Mon,Day,Hr,Minute,  ICrdno,nexp,bexp, error

CHARACTER Show

Character*130 LongStr1,LongStr2

REAL*8 DTDAYS,MM0,ECC0

REAL*8 INCDEG0,APDEG0,RAANDEG0,MADEG0

REAL*8 MMDOTO2,MMDDOTO6

COMMON /DebugHelp/ Help

CHARACTER Help

INCLUDE 'SGP4.CMN'

INCLUDE 'ASTMATH.CMN'

! --------------------  Implementation   ----------------------

Show = 'N'

xpdotp        =  1440.0D0 / (2.0D0 * pi) ! 229.1831180523293

CALL getgravconst( whichconst, tumin, mu, radiusearthkm, xke,

&       j2, j3, j4, j3oj2 );

VKmPerSec     =  RadiusEarthKm * xke / 60.0D0

* ----------------- READ THE FIRST LINE OF ELEMENT SET ----------------

Code = 0

* ---------------------- CONVERT TO INTERNAL UNITS --------------------

* ---- RADIANS, DISTANCE IN EARTH RADII, AND VELOCITY IN ER/KEMIN) ----

NDot   = MMDOTO2 / (XPDOTP*1440)

NDDot  = MMDDOTO6 / (XPDOTP*1440*1440)

c added the possibility to put in a Bstar term.  If this is unnecessary

c comment out the next two lines and uncomment the third line.

!Write(*,*) 'Input Bstar term: '

!read(*,*) Bstar

Bstar = 0.0d0

c Changed initial element set to match NASA input

No     = MM0 / XPDOTP

a      = (No*TUMin)**(-2.0D0/3.0D0)

Inclo  = INCDEG0  * Deg2Rad

nodeo  = RAANDEG0 * Deg2Rad

Argpo  = APDEG0 * Deg2Rad

Mo     = MADEG0* Deg2Rad

IF (DABS(ECC0-1.0D0) .gt. 0.000001D0) THEN

Altp= (a*(1.0D0-ECC0))-1.0D0

Alta= (a*(1.0D0+ECC0))-1.0D0

ELSE

Alta= 999999.9D0

Altp= 2.0D0* (4.0D0/(No*No)**(1.0D0/3.0D0))

ENDIF

! ---- Ballistic Coefficient ----

IF (DABS(BStar) .gt. 0.00000001D0) THEN

BC= 1.0D0/(12.741621D0*BStar)

ELSE

BC= 1.111111111111111D0

ENDIF

! ----------------------------------------------------------------

! find sgp4epoch time of element set

! remember that sgp4 uses units of days from 0 jan 1950 (sgp4epoch)

! and minutes from the epoch (time)

! ----------------------------------------------------------------

* ------------------- MAKE INITIAL PREDICTION AT EPOCH ----------------

! 2433281.5 - 2400000.5 = 33281.0, thus time from 1950

CALL SGP4Init( whichconst,

&                 SatNum,BStar, ECC0, JDSatEpoch1-2433281.5D0,

&                 Argpo,Inclo,Mo,No, nodeo, Error )

IF(Error .GT. 0) THEN

WRITE( *,*) '# *** SGP4 Model Error ***',Error

ENDIF

c      write tle output details

c      INCLUDE 'debug8.for'

! ---- Fix to indicate end-of-file

GOTO 1000

999   Code = 999

1000   CONTINUE

RETURN

END  !       SUBROUTINE TwoLine2RVSGP4

* -----------------------------------------------------------------------------

*

*                             SUBROUTINE SGP4INIT

*

*  This subroutine initializes variables for SGP4.

*

*  author        : david vallado                  719-573-2600   28 jun 2005

*

*  inputs        :

*    satn        - satellite number

*    bstar       - sgp4 type drag coefficient              kg/m2er

*    ecco        - eccentricity

*    epoch       - epoch time in days from jan 0, 1950. 0 hr

*    argpo       - argument of perigee (output if ds)

*    inclo       - inclination

*    mo          - mean anomaly (output if ds)

*    no          - mean motion

*    nodeo      - right ascension of ascending node

*

*  outputs       :

*    satrec      - common block values for subsequent calls

*    return code - non-zero on error.

*                   1 - mean elements, ecc >= 1.0 or ecc < -0.001 or a < 0.95 er

*                   2 - mean motion less than 0.0

*                   3 - pert elements, ecc < 0.0  or  ecc > 1.0

*                   4 - semi-latus rectum < 0.0

*                   5 - epoch elements are sub-orbital

*                   6 - satellite has decayed

*

*  coupling      :

*    getgravconst-

*    initl       -

*    dscom       -

*    dpper       -

*    dsinit      -

*

*  references    :

*    hoots, roehrich, norad spacetrack report #3 1980

*    hoots, norad spacetrack report #6 1986

*    hoots, schumacher and glover 2004

*    vallado, crawford, hujsak, kelso  2006

* ---------------------------------------------------------------------------- }

SUBROUTINE SGP4Init ( whichconst,

&                      Satn,   xBStar, xEcco,  Epoch, xArgpo,

&                      xInclo, xMo, xNo, xnodeo, Error )

IMPLICIT NONE

INTEGER Satn, error, whichconst

REAL*8  xBStar, xEcco, Epoch, xArgpo, xInclo, xMo, xNo, xnodeo

REAL*8 T, r(3), v(3)

INCLUDE 'SGP4.CMN'

!COMMON /DebugHelp/ Help ! removed by NM 4/16

CHARACTER Help

* -------------------------- Local Variables --------------------------

REAL*8  Ao,ainv,con42,cosio,sinio,cosio2,Eccsq,omeosq,

&          posq,rp,rteosq, CNODM , SNODM , COSIM , SINIM , COSOMM,

&          SINOMM, Cc1sq ,

&          Cc2   , Cc3   , Coef  , Coef1 , Cosio4, DAY   , Dndt  ,

&          Eccm  , EMSQ  , Eeta  , Etasq , GAM   , Argpm , nodem,

&          Inclm , Mm  , Xn    , Perige, Pinvsq, Psisq , Qzms24,

&          RTEMSQ, S1    , S2    , S3    , S4    , S5    , S6    ,

&          S7    , SFour , SS1   , SS2   , SS3   , SS4   , SS5   ,

&          SS6   , SS7   , SZ1   , SZ2   , SZ3   , SZ11  , SZ12  ,

&          SZ13  , SZ21  , SZ22  , SZ23  , SZ31  , SZ32  , SZ33  ,

&          Tc    , Temp  , Temp1 , Temp2 , Temp3 , Tsi   , XPIDOT,

&          Xhdot1, Z1    , Z2    , Z3    , Z11   , Z12   , Z13   ,

&          Z21   , Z22   , Z23   , Z31   , Z32   , Z33

REAL*8  qzms2t, SS, mu, RadiusEarthKm, J2, j3oJ2,J4,X2o3,

&          temp4, j3, xke, tumin

INCLUDE 'ASTMATH.CMN'

* ---------------------------- INITIALIZATION -------------------------

method = 'n'

c       clear sgp4 flag

Error = 0

c      sgp4fix - note the following variables are also passed directly via sgp4 common.

c      it is possible to streamline the sgp4init call by deleting the "x"

c      variables, but the user would need to set the common values first. we

c      include the additional assignment in case twoline2rv is not used.

bstar  = xbstar

ecco   = xecco

argpo  = xargpo

inclo  = xinclo

mo     = xmo

no     = xno

nodeo  = xnodeo

! sgp4fix identify constants and allow alternate values

CALL getgravconst( whichconst, tumin, mu, radiusearthkm, xke,

&       j2, j3, j4, j3oj2 )

SS     = 78.0D0/RadiusEarthKm + 1.0D0

QZMS2T = ((120.0D0-78.0D0)/RadiusEarthKm) ** 4

X2o3   =  2.0D0 / 3.0D0

c     sgp4fix divisor for divide by zero check on inclination

c     the old check used 1.0D0 + cos(pi-1.0D-9), but then compared it to

c     1.5D-12, so the threshold was changed to 1.5D-12 for consistency

temp4    =   1.5D-12

Init = 'y'

T = 0.0D0

CALL INITL( Satn , whichconst, Ecco  , EPOCH , Inclo , No,

&     Method, AINV  , AO    , CON41 , CON42 , COSIO , COSIO2,

&     Eccsq , OMEOSQ, POSQ  , rp    , RTEOSQ, SINIO ,

&     GSTo, Opsmode )

IF(rp .lt. 1.0D0) THEN

Error = 5

ENDIF

IF(OMEOSQ .ge. 0.0D0 .OR. No .ge. 0.0D0) THEN

ISIMP = 0

IF (rp .lt. (220.0D0/RadiusEarthKm+1.0D0)) THEN

ISIMP = 1

ENDIF

SFour  = SS

QZMS24 = QZMS2T

PERIGE = (rp-1.0D0)*RadiusEarthKm

* ----------- For perigees below 156 km, S and Qoms2t are altered -----

IF(PERIGE .lt. 156.0D0) THEN

SFour = PERIGE-78.0D0

IF(PERIGE .le. 98.0D0) THEN

SFour = 20.0D0

ENDIF

QZMS24 = ( (120.0D0-SFour)/RadiusEarthKm )**4

SFour  = SFour/RadiusEarthKm + 1.0D0

ENDIF

PINVSQ = 1.0D0/POSQ

TSI    = 1.0D0/(AO-SFour)

ETA    = AO*Ecco*TSI

ETASQ  = ETA*ETA

EETA   = Ecco*ETA

PSISQ  = DABS(1.0D0-ETASQ)

COEF   = QZMS24*TSI**4

COEF1  = COEF/PSISQ**3.5D0

CC2    = COEF1*No* (AO* (1.0D0+1.5D0*ETASQ+EETA*

&               (4.0D0+ETASQ) )+0.375D0*

&         J2*TSI/PSISQ*CON41*(8.0D0+3.0D0*ETASQ*(8.0D0+ETASQ)))

CC1    = BSTAR*CC2

CC3    = 0.0D0

IF(Ecco .GT. 1.0D-4) THEN

CC3 = -2.0D0*COEF*TSI*J3OJ2*No*SINIO/Ecco

ENDIF

X1MTH2 = 1.0D0-COSIO2

CC4    = 2.0D0*No*COEF1*AO*OMEOSQ*(ETA*(2.0D0+0.5D0*ETASQ)

&              +Ecco*(0.5D0 + 2.0D0*ETASQ) - J2*TSI / (AO*PSISQ)*

&              (-3.0D0*CON41*(1.0D0-2.0D0*

&       EETA+ETASQ*(1.5D0-0.5D0*EETA))+0.75D0*X1MTH2*(2.0D0*ETASQ

&       -EETA*(1.0D0+ETASQ))*DCOS(2.0D0*Argpo)))

CC5    = 2.0D0*COEF1*AO*OMEOSQ* (1.0D0 + 2.75D0*

&               (ETASQ + EETA) + EETA*ETASQ )

COSIO4 = COSIO2*COSIO2

TEMP1  = 1.5D0*J2*PINVSQ*No

TEMP2  = 0.5D0*TEMP1*J2*PINVSQ

TEMP3  = -0.46875D0*J4*PINVSQ*PINVSQ*No

MDot   = No + 0.5D0*TEMP1*RTEOSQ*CON41 + 0.0625D0*TEMP2*

&               RTEOSQ*(13.0D0 - 78.0D0*COSIO2 + 137.0D0*COSIO4)

ArgpDot= -0.5D0*TEMP1*CON42 + 0.0625D0*TEMP2*

&               (7.0D0 - 114.0D0*COSIO2 +

&        395.0D0*COSIO4)+TEMP3*(3.0D0-36.0D0*COSIO2+49.0D0*COSIO4)

XHDOT1 = -TEMP1*COSIO

nodeDot = XHDOT1+(0.5D0*TEMP2*(4.0D0-19.0D0*COSIO2)+

&                 2.0D0*TEMP3*(3.0D0 - 7.0D0*COSIO2))*COSIO

XPIDOT = ArgpDot+nodeDot

OMGCOF = BSTAR*CC3*DCOS(Argpo)

XMCOF  = 0.0D0

IF(Ecco .GT. 1.0D-4) THEN

XMCOF = -X2O3*COEF*BSTAR/EETA

ENDIF

XNODCF = 3.5D0*OMEOSQ*XHDOT1*CC1

T2COF  = 1.5D0*CC1

c           sgp4fix for divide by zero with xinco = 180 deg

if (dabs(cosio+1.0).gt. 1.5d-12) THEN

XLCOF  = -0.25D0*J3OJ2*SINIO*

&                   (3.0D0+5.0D0*COSIO)/(1.0D0+COSIO)

else

XLCOF  = -0.25D0*J3OJ2*SINIO*

&                   (3.0D0+5.0D0*COSIO)/temp4

ENDIF

AYCOF  = -0.5D0*J3OJ2*SINIO

DELMO  = (1.0D0+ETA*DCOS(Mo))**3

SINMAO = DSIN(Mo)

X7THM1 = 7.0D0*COSIO2-1.0D0

* ------------------------ Deep Space Initialization ------------------

IF ((TWOPI/No) .ge. 225.0D0) THEN

METHOD = 'd'

ISIMP  = 1

TC     = 0.0D0

Inclm  = Inclo

CALL DSCOM( EPOCH     , Ecco  , Argpo , Tc    , Inclo ,

&                  nodeo, No    ,

&                  SNODM , CNODM , SINIM , COSIM , SINOMM, COSOMM,

&                  DAY   , E3    , Ee2   , Eccm  , EMSQ  , GAM   ,

&                  Peo   , Pgho  , Pho   , PInco , Plo   ,

&                  RTemSq, Se2   , Se3   , Sgh2  , Sgh3  , Sgh4  ,

&                  Sh2   , Sh3   , Si2   , Si3   , Sl2   , Sl3   ,

&                  Sl4   , S1    , S2    , S3    , S4    , S5    ,

&                  S6    , S7    , SS1   , SS2   , SS3   , SS4   ,

&                  SS5   , SS6   , SS7   , SZ1   , SZ2   , SZ3   ,

&                  SZ11  , SZ12  , SZ13  , SZ21  , SZ22  , SZ23  ,

&                  SZ31  , SZ32  , SZ33  , Xgh2  , Xgh3  , Xgh4  ,

&                  Xh2   , Xh3   , Xi2   , Xi3   , Xl2   , Xl3   ,

&                  Xl4   , Xn    , Z1    , Z2    , Z3    , Z11   ,

&                  Z12   , Z13   , Z21   , Z22   , Z23   , Z31   ,

&                  Z32   , Z33   , Zmol  , Zmos )

CALL DPPER( e3, ee2   , peo   , pgho  , pho   , pinco ,

&                  plo   , se2   , se3   , sgh2  , sgh3  , sgh4  ,

&                  sh2   , sh3   , si2   , si3   , sl2   , sl3   ,

&                  sl4   , T     , xgh2  , xgh3  , xgh4  , xh2   ,

&                  xh3   , xi2   , xi3   , xl2   , xl3   , xl4   ,

&                  zmol  , zmos  , Inclm , init  ,

&                  Ecco  , Inclo , nodeo, Argpo , Mo, Opsmode )

Argpm  = 0.0D0 ! add for DS to work initial

nodem  = 0.0D0

Mm     = 0.0D0

CALL DSINIT( whichconst,

&                   Cosim ,Emsq, Argpo, S1    , S2    , S3    ,

&                   S4    , S5    , Sinim , Ss1   , Ss2   , Ss3   ,

&                   Ss4   , Ss5   , Sz1   , Sz3   , Sz11  , Sz13  ,

&                   Sz21  , Sz23  , Sz31  , Sz33  , T     , Tc    ,

&                   GSTo  , Mo    , MDot  , No    ,nodeo,nodeDot,

&                   XPIDOT, Z1    , Z3    , Z11   , Z13   , Z21   ,

&                   Z23   , Z31   , Z33   , ecco  , eccsq,

&                   Eccm  , Argpm , Inclm , Mm    , Xn    , nodem,

&                   IREZ  , Atime , D2201 , D2211 , D3210 , D3222 ,

&                   D4410 , D4422 , D5220 , D5232 , D5421 , D5433 ,

&                   Dedt  , Didt  , DMDT  , DNDT  , DNODT , DOMDT ,

&                   Del1  , Del2  , Del3  , Xfact , Xlamo , Xli   ,

&                   Xni )

ENDIF

* ------------ Set variables if not deep space or rp < 220 -------------

IF (ISIMP .ne. 1) THEN

CC1SQ = CC1*CC1

D2    = 4.0D0*AO*TSI*CC1SQ

TEMP  = D2*TSI*CC1 / 3.0D0

D3    = (17.0D0*AO + SFour) * TEMP

D4    = 0.5D0*TEMP*AO*TSI*

&                  (221.0D0*AO + 31.0D0*SFour)*CC1

T3COF = D2 + 2.0D0*CC1SQ

T4COF = 0.25D0* (3.0D0*D3+CC1*(12.0D0*D2+10.0D0*CC1SQ) )

T5COF = 0.2D0* (3.0D0*D4 + 12.0D0*CC1*D3 + 6.0D0*D2*D2 +

&                  15.0D0*CC1SQ* (2.0D0*D2 + CC1SQ) )

ENDIF

ENDIF ! ------ if nodeo and No are gtr 0

init = 'n'

CALL SGP4(whichconst, 0.0D0, r, v, error)

RETURN

END  ! end sgp4init

* -----------------------------------------------------------------------------

*

*                             SUBROUTINE SGP4

*

*  this procedure is the sgp4 prediction model from space command. this is an

*    updated and combined version of sgp4 and sdp4, which were originally

*    published separately in spacetrack report #3. this version follows the

*    methodology from the aiaa paper (2006) describing the history and

*    development of the code.

*

*  author        : david vallado                  719-573-2600   28 jun 2005

*

*  inputs        :

*    satrec	 - initialised structure from sgp4init() call.

*    tsince	 - time eince epoch (minutes)

*

*  outputs       :

*    r           - position vector                     km

*    v           - velocity                            km/sec

*

*  return code - non-zero on error.

*                   1 - mean elements, ecc >= 1.0 or ecc < -0.001 or a < 0.95 er

*                   2 - mean motion less than 0.0

*                   3 - pert elements, ecc < 0.0  or  ecc > 1.0

*                   4 - semi-latus rectum < 0.0

*                   5 - epoch elements are sub-orbital

*                   6 - satellite has decayed

*

*

*  coupling      :

*    getgravconst-

*    dpper

*    dpspace

*

*  references    :

*    hoots, roehrich, norad spacetrack report #3 1980

*    hoots, norad spacetrack report #6 1986

*    hoots, schumacher and glover 2004

*    vallado, crawford, hujsak, kelso  2006

*------------------------------------------------------------------------------

SUBROUTINE SGP4 ( whichconst, T, r, v, Error )

IMPLICIT NONE

INTEGER  Error, whichconst

REAL*8   T, r(3), v(3)

INCLUDE 'SGP4.CMN'

* -------------------------- Local Variables --------------------------

REAL*8 AM    , Axnl  , Aynl  , Betal , COSIM , Cnod  ,

&         Cos2u , Coseo1, Cosi  , Cosip , Cosisq, Cossu , Cosu  ,

&         Delm  , Delomg, Eccm  , EMSQ  , Ecose , El2   , Eo1   ,

&         Eccp  , Esine , Argpm , Argpp , Omgadf, Pl    ,

&         Rdotl , Rl    , Rvdot , Rvdotl, SINIM ,

&         Sin2u , Sineo1, Sini  , Sinip , Sinsu , Sinu  ,

&         Snod  , Su    , T2    , T3    , T4    , Tem5  , Temp  ,

&         Temp1 , Temp2 , Tempa , Tempe , Templ , U     , Ux    ,

&         Uy    , Uz    , Vx    , Vy    , Vz    , Inclm , Mm  ,

&         XN    , nodem , Xinc  , Xincp , Xl    , Xlm   , Mp  ,

&         Xmdf  , Xmx   , Xmy   , Xnoddf, Xnode , nodep,

&         Tc    , Dndt

REAL*8 X2O3, J2,J3,XKE,J3OJ2, mr,mv,

&         mu, RadiusEarthkm, VKmPerSec, temp4, tumin, j4

INTEGER iter

CHARACTER Help

INCLUDE 'ASTMATH.CMN'

* ------------------------ WGS-72 EARTH CONSTANTS ---------------------

* ---------------------- SET MATHEMATICAL CONSTANTS -------------------

X2O3   = 2.0D0/3.0D0

c     Keep compiler ok for warnings on uninitialized variables

mr = 0.0D0

Coseo1 = 1.0D0

Sineo1 = 0.0D0

! sgp4fix identify constants and allow alternate values

CALL getgravconst( whichconst, tumin, mu, radiusearthkm, xke,

&       j2, j3, j4, j3oj2 )

c     sgp4fix divisor for divide by zero check on inclination

c     the old check used 1.0D0 + cos(pi-1.0D-9), but then compared it to

c     1.5D-12, so the threshold was changed to 1.5D-12 for consistency

temp4    =   1.5D-12

VKmPerSec     =  RadiusEarthKm * xke/60.0D0

* ------------------------- CLEAR SGP4 ERROR FLAG ---------------------

Error = 0

* ----------- UPDATE FOR SECULAR GRAVITY AND ATMOSPHERIC DRAG ---------

XMDF   = Mo + MDot*T

OMGADF = Argpo + ArgpDot*T

XNODDF = nodeo + nodeDot*T

Argpm  = OMGADF

Mm     = XMDF

T2     = T*T

nodem  = XNODDF + XNODCF*T2

TEMPA  = 1.0D0 - CC1*T

TEMPE  = BSTAR*CC4*T

TEMPL  = T2COF*T2

IF (ISIMP .ne. 1) THEN

DELOMG = OMGCOF*T

DELM   = XMCOF*(( 1.0D0+ETA*DCOS(XMDF) )**3-DELMO)

TEMP   = DELOMG + DELM

Mm     = XMDF + TEMP

Argpm  = OMGADF - TEMP

T3     = T2*T

T4     = T3*T

TEMPA  = TEMPA - D2*T2 - D3*T3 - D4*T4

TEMPE  = TEMPE + BSTAR*CC5*(DSIN(Mm) - SINMAO)

TEMPL  = TEMPL + T3COF*T3 + T4*(T4COF + T*T5COF)

ENDIF

XN    = No

Eccm  = Ecco

Inclm = Inclo

IF(METHOD .EQ. 'd') THEN

TC     = T

CALL DSPACE( IRez  , D2201 , D2211 , D3210 , D3222 , D4410 ,

&                 D4422 , D5220 , D5232 , D5421 , D5433 , Dedt  ,

&                 Del1  , Del2  , Del3  , Didt  , Dmdt  , Dnodt ,

&                 Domdt , Argpo , ArgpDot, T    , TC    , GSTo ,

&                 Xfact , Xlamo , No   ,

&                 Atime , Eccm  , Argpm, Inclm , Xli   , Mm  ,

&                 XNi   , nodem, Dndt  , XN  )

ENDIF

c     mean motion less than 0.0

IF(XN .LE. 0.0D0) THEN

Error = 2

ENDIF

AM = (XKE/XN)**X2O3*TEMPA**2

XN = XKE/AM**1.5D0

Eccm = Eccm-TEMPE

c   fix tolerance for error recognition

IF (Eccm .GE. 1.0D0 .or. Eccm.lt.-0.001D0 .or. AM .lt. 0.95) THEN

c	  write(6,*) '# Error 1, Eccm = ',  Eccm, ' AM = ', AM

Error = 1

ENDIF

c   sgp4fix change test condition for eccentricity

IF (Eccm .lt. 1.0D-6) Eccm = 1.0D-6

Mm     = Mm+No*TEMPL

XLM    = Mm+Argpm+nodem

EMSQ   = Eccm*Eccm

TEMP   = 1.0D0 - EMSQ

nodem  = DMOD(nodem,TwoPi)

Argpm  = DMOD(Argpm,TwoPi)

XLM    = DMOD(XLM,TwoPi)

Mm     = DMOD(XLM - Argpm - nodem,TwoPi)

* --------------------- COMPUTE EXTRA MEAN QUANTITIES -----------------

SINIM  = DSIN(Inclm)

COSIM  = DCOS(Inclm)

* ------------------------ ADD LUNAR-SOLAR PERIODICS ------------------

Eccp   = Eccm

XINCP  = Inclm

Argpp  = Argpm

nodep = nodem

Mp     = Mm

SINIP  = SINIM

COSIP  = COSIM

IF(METHOD .EQ. 'd') THEN

CALL DPPER( e3    , ee2   , peo   , pgho  , pho   , pinco ,

&                plo   , se2   , se3   , sgh2  , sgh3  , sgh4  ,

&                sh2   , sh3   , si2   , si3   , sl2   , sl3   ,

&                sl4   , T     , xgh2  , xgh3  , xgh4  , xh2   ,

&                xh3   , xi2   , xi3   , xl2   , xl3   , xl4   ,

&                zmol  , zmos  , Inclo , 'n'   ,

&                Eccp  , XIncp , nodep, Argpp, Mp, Opsmode )

IF(XINCP .lt. 0.0D0) THEN

XINCP  = -XINCP

nodep  = nodep + PI

Argpp  = Argpp - PI

ENDIF

IF(Eccp .lt. 0.0D0 .OR. Eccp .GT. 1.0D0) THEN

Error = 3

ENDIF

ENDIF

* ------------------------ LONG PERIOD PERIODICS ----------------------

IF(METHOD .EQ. 'd') THEN

SINIP =  DSIN(XINCP)

COSIP =  DCOS(XINCP)

AYCOF = -0.5D0*J3OJ2*SINIP

c         sgp4fix for divide by zero with xincp = 180 deg

if (dabs(cosip+1.0).gt. 1.5d-12) THEN

XLCOF  = -0.25D0*J3OJ2*SINIP*

&                 (3.0D0+5.0D0*COSIP)/(1.0D0+COSIP)

else

XLCOF  = -0.25D0*J3OJ2*SINIP*

&                 (3.0D0+5.0D0*COSIP)/temp4

ENDIF

ENDIF

AXNL = Eccp*DCOS(Argpp)

TEMP = 1.0D0 / (AM*(1.0D0-Eccp*Eccp))

AYNL = Eccp*DSIN(Argpp) + TEMP*AYCOF

XL   = Mp + Argpp + nodep + TEMP*XLCOF*AXNL

* ------------------------- SOLVE KEPLER'S EQUATION -------------------

U    = DMOD(XL-nodep,TwoPi)

EO1  = U

ITER=0

c   sgp4fix for kepler iteration

c   the following iteration needs better limits on corrections

Temp = 9999.9D0

DO WHILE ((Temp.ge.1.0D-12).and.(ITER.lt.10))

ITER=ITER+1

SINEO1= DSIN(EO1)

COSEO1= DCOS(EO1)

TEM5  = 1.0D0 - COSEO1*AXNL - SINEO1*AYNL

TEM5  = (U - AYNL*COSEO1 + AXNL*SINEO1 - EO1) / TEM5

Temp  = DABS(Tem5)

IF(Temp.gt.1.0D0) Tem5=Tem5/Temp ! Stop excessive correction

EO1   = EO1+TEM5

ENDDO

* ----------------- SHORT PERIOD PRELIMINARY QUANTITIES ---------------

ECOSE = AXNL*COSEO1+AYNL*SINEO1

ESINE = AXNL*SINEO1-AYNL*COSEO1

EL2   = AXNL*AXNL+AYNL*AYNL

PL    = AM*(1.0D0-EL2)

c     semi-latus rectum < 0.0

IF ( PL .lt. 0.0D0 ) THEN

Error = 4

ELSE

RL    = AM*(1.0D0-ECOSE)

RDOTL = DSQRT(AM)*ESINE/RL

RVDOTL= DSQRT(PL)/RL

BETAL = DSQRT(1.0D0-EL2)

TEMP  = ESINE/(1.0D0+BETAL)

SINU  = AM/RL*(SINEO1-AYNL-AXNL*TEMP)

COSU  = AM/RL*(COSEO1-AXNL+AYNL*TEMP)

SU    = DATAN2(SINU,COSU)

SIN2U = (COSU+COSU)*SINU

COS2U = 1.0D0-2.0D0*SINU*SINU

TEMP  = 1.0D0/PL

TEMP1 = 0.5D0*J2*TEMP

TEMP2 = TEMP1*TEMP

* ------------------ UPDATE FOR SHORT PERIOD PERIODICS ----------------

IF(METHOD .EQ. 'd') THEN

COSISQ = COSIP*COSIP

CON41  = 3.0D0*COSISQ - 1.0D0

X1MTH2 = 1.0D0 - COSISQ

X7THM1 = 7.0D0*COSISQ - 1.0D0

ENDIF

mr   = RL*(1.0D0 - 1.5D0*TEMP2*BETAL*CON41) +

&           0.5D0*TEMP1*X1MTH2*COS2U

SU   = SU - 0.25D0*TEMP2*X7THM1*SIN2U

XNODE= nodep + 1.5D0*TEMP2*COSIP*SIN2U

XINC = XINCP + 1.5D0*TEMP2*COSIP*SINIP*COS2U

mv   = RDOTL - XN*TEMP1*X1MTH2*SIN2U / XKE

RVDOT= RVDOTL + XN*TEMP1* (X1MTH2*COS2U+1.5D0*CON41) / XKE

* ------------------------- ORIENTATION VECTORS -----------------------

SINSU=  DSIN(SU)

COSSU=  DCOS(SU)

SNOD =  DSIN(XNODE)

CNOD =  DCOS(XNODE)

SINI =  DSIN(XINC)

COSI =  DCOS(XINC)

XMX  = -SNOD*COSI

XMY  =  CNOD*COSI

UX   =  XMX*SINSU + CNOD*COSSU

UY   =  XMY*SINSU + SNOD*COSSU

UZ   =  SINI*SINSU

VX   =  XMX*COSSU - CNOD*SINSU

VY   =  XMY*COSSU - SNOD*SINSU

VZ   =  SINI*COSSU

* ----------------------- POSITION AND VELOCITY -----------------------

r(1) = mr*UX * RadiusEarthkm

r(2) = mr*UY * RadiusEarthkm

r(3) = mr*UZ * RadiusEarthkm

v(1) = (mv*UX + RVDOT*VX) * VKmPerSec

v(2) = (mv*UY + RVDOT*VY) * VKmPerSec

v(3) = (mv*UZ + RVDOT*VZ) * VKmPerSec

ENDIF

* --------------------------- ERROR PROCESSING ------------------------

c     sgp4fix for decaying satellites

if (mr .lt. 1.0D0) THEN

error = 6

ENDIF

RETURN

END  ! end sgp4

* -----------------------------------------------------------------------------

*

*                           SUBROUTINE INITL

*

*  this subroutine initializes the spg4 propagator. all the initialization is

*    consolidated here instead of having multiple loops inside other routines.

*

*  author        : david vallado                  719-573-2600   28 jun 2005

*

*  inputs        :

*    ecco        - eccentricity                           0.0 - 1.0

*    epoch       - epoch time in days from jan 0, 1950. 0 hr

*    inclo       - inclination of satellite

*    no          - mean motion of satellite

*    satn        - satellite number

*

*  outputs       :

*    ainv        - 1.0 / a

*    ao          - semi major axis

*    con41       -

*    con42       - 1.0 - 5.0 cos(i)

*    cosio       - cosine of inclination

*    cosio2      - cosio squared

*    eccsq       - eccentricity squared

*    method      - flag for deep space                    'd', 'n'

*    omeosq      - 1.0 - ecco * ecco

*    posq        - semi-parameter squared

*    rp          - radius of perigee

*    rteosq      - square root of (1.0 - ecco*ecco)

*    sinio       - sine of inclination

*    gsto        - gst at time of observation               rad

*    no          - mean motion of satellite

*

*

*  coupling      :

*    getgravconst-

*

*  references    :

*    hoots, roehrich, norad spacetrack report #3 1980

*    hoots, norad spacetrack report #6 1986

*    hoots, schumacher and glover 2004

*    vallado, crawford, hujsak, kelso  2006

*------------------------------------------------------------------------------

SUBROUTINE INITL( Satn , whichconst, Ecco  , EPOCH , Inclo , No,

&         Method, AINV  , AO    , CON41 , CON42 , COSIO , COSIO2,

&         Eccsq , OMEOSQ, POSQ  , rp    , RTEOSQ, SINIO ,

&         GSTo, operationmode )

IMPLICIT NONE

CHARACTER Method, operationmode

INTEGER Satn, whichconst

REAL*8 Ecco  , EPOCH , Inclo , No   ,

&         AINV  , AO    , CON41 , CON42 , COSIO , COSIO2,

&         Eccsq , OMEOSQ, POSQ  , rp    , RTEOSQ, SINIO , GSTo

COMMON /DebugHelp/ Help

CHARACTER Help

* -------------------------- Local Variables --------------------------

c        sgp4fix use old way of finding gst

Integer ids70

REAL*8 ts70, ds70, tfrac, c1, thgr70, fk5r, c1p2p, thgr, thgro

REAL*8  RadPerDay, Temp, TUT1

REAL*8  ak, d1, del, adel, po

REAL*8  X2o3, J2, XKE, tumin, mu, radiusearthkm, j3, j4, j3oj2

INCLUDE 'ASTMATH.CMN'

* ------------------------ WGS-72 EARTH CONSTANTS ---------------------

X2o3   = 2.0D0/3.0D0

! sgp4fix identify constants and allow alternate values

CALL getgravconst( whichconst, tumin, mu, radiusearthkm, xke,

&       j2, j3, j4, j3oj2 )

* ----------------- CALCULATE AUXILLARY EPOCH QUANTITIES --------------

Eccsq  = Ecco*Ecco

OMEOSQ = 1.0D0 - Eccsq

RTEOSQ = DSQRT(OMEOSQ)

COSIO  = DCOS(Inclo)

COSIO2 = COSIO*COSIO

* ---------------------- UN-KOZAI THE MEAN MOTION ---------------------

AK   =  (XKE/No)**X2O3

D1   =  0.75D0*J2* (3.0D0*COSIO2-1.0D0) / (RTEOSQ*OMEOSQ)

DEL  =  D1/(AK*AK)

ADEL =  AK * ( 1.0D0 - DEL*DEL - DEL*

&                 (1.0D0/3.0D0 + 134.0D0*DEL*DEL / 81.0D0) )

DEL  =  D1/(ADEL*ADEL)

No   =  No/(1.0D0 + DEL)

AO   =  (XKE/No)**X2O3

SINIO=  DSIN(Inclo)

PO   =  AO*OMEOSQ

CON42=  1.0D0-5.0D0*COSIO2

CON41=  -CON42-COSIO2-COSIO2

AINV =  1.0D0/AO

POSQ =  PO*PO

rp   =  AO*(1.0D0-Ecco)

METHOD = 'n'

* ----------------- CALCULATE GREENWICH LOCATION AT EPOCH -------------

c       sgp4fix modern approach to finding sidereal time

IF (operationmode .ne. 'a') THEN

RadPerDay  = twopi * 1.002737909350795D0  !6.30038809866574D0

Temp = Epoch + 2433281.5D0

TUT1= ( DINT(Temp-0.5D0) + 0.5D0 - 2451545.0D0 ) / 36525.0D0

Gsto= 1.75336855923327D0 + 628.331970688841D0*TUT1

&             + 6.77071394490334D-06*TUT1*TUT1

&             - 4.50876723431868D-10*TUT1*TUT1*TUT1

&             + RadPerDay*( Temp-0.5D0-DINT(Temp-0.5D0) )

ELSE

! sgp4fix use old way of finding gst

! count integer number of days from 0 jan 1970

TS70  = EPOCH-7305.0D0

IDS70 = TS70 + 1.0D-8

TFRAC = TS70-IDS70

! find greenwich location at epoch

C1    = 1.72027916940703639D-2

THGR70= 1.7321343856509374D0

FK5R  = 5.07551419432269442D-15

C1P2P = C1+TWOPI

gsto  = THGR70+C1*IDS70+C1P2P*TFRAC+TS70*TS70*FK5R

ENDIF

! ------------------------ Check quadrants ---------------------

Gsto = DMOD( Gsto,TwoPi )

IF ( Gsto .lt. 0.0D0 ) THEN

Gsto= Gsto + TwoPi

ENDIF

RETURN

END  ! end initl

* -----------------------------------------------------------------------------

*

*                           SUBROUTINE DPPER

*

*  This Subroutine provides deep space long period periodic contributions

*    to the mean elements.  by design, these periodics are zero at epoch.

*    this used to be dscom which included initialization, but it's really a

*    recurring function.

*

*  author        : david vallado                  719-573-2600   28 jun 2005

*

*  inputs        :

*    e3          -

*    ee2         -

*    peo         -

*    pgho        -

*    pho         -

*    pinco       -

*    plo         -

*    se2 , se3 , Sgh2, Sgh3, Sgh4, Sh2, Sh3, Si2, Si3, Sl2, Sl3, Sl4 -

*    t           -

*    xh2, xh3, xi2, xi3, xl2, xl3, xl4 -

*    zmol        -

*    zmos        -

*    ep          - eccentricity                           0.0 - 1.0

*    inclo       - inclination - needed for lyddane modification

*    nodep       - right ascension of ascending node

*    argpp       - argument of perigee

*    mp          - mean anomaly

*

*  outputs       :

*    ep          - eccentricity                           0.0 - 1.0

*    inclp       - inclination

*    nodep       - right ascension of ascending node

*    argpp       - argument of perigee

*    mp          - mean anomaly

*

*  coupling      :

*    none.

*

*  references    :

*    hoots, roehrich, norad spacetrack report #3 1980

*    hoots, norad spacetrack report #6 1986

*    hoots, schumacher and glover 2004

*    vallado, crawford, hujsak, kelso  2006

*------------------------------------------------------------------------------

SUBROUTINE DPPER( e3    , ee2   , peo   , pgho  , pho   , pinco ,

&                  plo   , se2   , se3   , sgh2  , sgh3  , sgh4  ,

&                  sh2   , sh3   , si2   , si3   , sl2   , sl3   ,

&                  sl4   , T     , xgh2  , xgh3  , xgh4  , xh2   ,

&                  xh3   , xi2   , xi3   , xl2   , xl3   , xl4   ,

&                  zmol  , zmos  , inclo , init  ,

&                  Eccp  , Inclp , nodep, Argpp , Mp,

&                  operationmode )

IMPLICIT NONE

CHARACTER Init, operationmode

REAL*8  e3    , ee2   , peo   , pgho  , pho   , pinco , plo   ,

&          se2   , se3   , sgh2  , sgh3  , sgh4  , sh2   , sh3   ,

&          si2   , si3   , sl2   , sl3   , sl4   , T     , xgh2  ,

&          xgh3  , xgh4  , xh2   , xh3   , xi2   , xi3   , xl2   ,

&          xl3   , xl4   , zmol  , zmos  , inclo ,

&          Eccp  , Inclp , nodep, Argpp , Mp

* -------------------------- Local Variables --------------------------

REAL*8  alfdp , betdp , cosip , cosop , dalf  , dbet  , dls   ,

&          f2    , f3    , pe    , pgh   , ph    , pinc  , pl    ,

&          sel   , ses   , sghl  , sghs  , shl   , shs   , sil   ,

&          sinip , sinop , sinzf , sis   , sll   , sls   , xls   ,

&          xnoh  , zf    , zm

REAL*8  Zel   , Zes   , Znl   , Zns

!COMMON /DebugHelp/ Help

CHARACTER Help

INCLUDE 'ASTMATH.CMN'

* ----------------------------- Constants -----------------------------

ZES  = 0.01675D0

ZEL  = 0.05490D0

ZNS  = 1.19459D-5

ZNL  = 1.5835218D-4

* ------------------- CALCULATE TIME VARYING PERIODICS ----------------

ZM   = ZMOS + ZNS*T

IF (Init.eq.'y') ZM = ZMOS

ZF   = ZM + 2.0D0*ZES*DSIN(ZM)

SINZF= DSIN(ZF)

F2   =  0.5D0*SINZF*SINZF - 0.25D0

F3   = -0.5D0*SINZF*DCOS(ZF)

SES  = SE2*F2 + SE3*F3

SIS  = SI2*F2 + SI3*F3

SLS  = SL2*F2 + SL3*F3 + SL4*SINZF

SGHS = SGH2*F2 + SGH3*F3 + SGH4*SINZF

SHS  = SH2*F2 + SH3*F3

ZM   = ZMOL + ZNL*T

IF (Init.eq.'y') ZM = ZMOL

ZF   = ZM + 2.0D0*ZEL*DSIN(ZM)

SINZF= DSIN(ZF)

F2   =  0.5D0*SINZF*SINZF - 0.25D0

F3   = -0.5D0*SINZF*DCOS(ZF)

SEL  = EE2*F2 + E3*F3

SIL  = XI2*F2 + XI3*F3

SLL  = XL2*F2 + XL3*F3 + XL4*SINZF

SGHL = XGH2*F2 + XGH3*F3 + XGH4*SINZF

SHL  = XH2*F2 + XH3*F3

PE   = SES + SEL

PINC = SIS + SIL

PL   = SLS + SLL

PGH  = SGHS + SGHL

PH   = SHS + SHL

IF (Init.eq.'n') THEN

PE    = PE   - PEO

PINC  = PINC - PINCO

PL    = PL   - PLO

PGH   = PGH  - PGHO

PH    = PH   - PHO

Inclp = Inclp  + PINC

Eccp  = Eccp   + PE

SINIP = DSIN(Inclp)

COSIP = DCOS(Inclp)

* ------------------------- APPLY PERIODICS DIRECTLY ------------------

c    sgp4fix for lyddane choice

c    strn3 used original inclination - this is technically feasible

c    gsfc used perturbed inclination - also technically feasible

c    probably best to readjust the 0.2 limit value and limit discontinuity

c    0.2 rad = 11.45916 deg

c    use next line for original strn3 approach and original inclination

c            IF (inclo.ge.0.2D0) THEN

c    use next line for gsfc version and perturbed inclination

IF (Inclp.ge.0.2D0) THEN

PH     = PH/SINIP

PGH    = PGH - COSIP*PH

Argpp  = Argpp + PGH

nodep  = nodep + PH

Mp     = Mp + PL

ELSE

* ----------------- APPLY PERIODICS WITH LYDDANE MODIFICATION ---------

SINOP  = DSIN(nodep)

COSOP  = DCOS(nodep)

ALFDP  = SINIP*SINOP

BETDP  = SINIP*COSOP

DALF   =  PH*COSOP + PINC*COSIP*SINOP

DBET   = -PH*SINOP + PINC*COSIP*COSOP

ALFDP  = ALFDP + DALF

BETDP  = BETDP + DBET

nodep = DMOD(nodep,TwoPi)

! sgp4fix for afspc written intrinsic functions

! nodep used without a trigonometric function ahead

IF ((nodep .LT. 0.0D0) .and. (operationmode .eq. 'a'))

&                THEN

nodep = nodep + twopi

ENDIF

XLS    = Mp + Argpp + COSIP*nodep

DLS    = PL + PGH - PINC*nodep*SINIP

XLS    = XLS + DLS

XNOH   = nodep

nodep  = DATAN2(ALFDP,BETDP)

! sgp4fix for afspc written intrinsic functions

! nodep used without a trigonometric function ahead

IF ((nodep .LT. 0.0D0) .and. (operationmode .eq. 'a'))

&                THEN

nodep = nodep + twopi

ENDIF

IF (DABS(XNOH-nodep) .GT. PI) THEN

IF(nodep .lt. XNOH) THEN

nodep = nodep+TWOPI

ELSE

nodep = nodep-TWOPI

ENDIF

ENDIF

Mp   = Mp + PL

Argpp=  XLS - Mp - COSIP*nodep

ENDIF

ENDIF

RETURN

END  !  end dpper

* -----------------------------------------------------------------------------

*

*                           SUBROUTINE DSCOM

*

*  This Subroutine provides deep space common items used by both the secular

*    and periodics subroutines.  input is provided as shown. this routine

*    used to be called dpper, but the functions inside weren't well organized.

*

*  author        : david vallado                  719-573-2600   28 jun 2005

*

*  inputs        :

*    epoch       -

*    ep          - eccentricity

*    argpp       - argument of perigee

*    tc          -

*    inclp       - inclination

*    nodep      - right ascension of ascending node

*    np          - mean motion

*

*  outputs       :

*    sinim  , cosim  , sinomm , cosomm , snodm  , cnodm

*    day         -

*    e3          -

*    ee2         -

*    em          - eccentricity

*    emsq        - eccentricity squared

*    gam         -

*    peo         -

*    pgho        -

*    pho         -

*    pinco       -

*    plo         -

*    rtemsq      -

*    se2, se3         -

*    sgh2, sgh3, sgh4        -

*    sh2, sh3, si2, si3, sl2, sl3, sl4         -

*    s1, s2, s3, s4, s5, s6, s7          -

*    ss1, ss2, ss3, ss4, ss5, ss6, ss7, sz1, sz2, sz3         -

*    sz11, sz12, sz13, sz21, sz22, sz23, sz31, sz32, sz33        -

*    xgh2, xgh3, xgh4, xh2, xh3, xi2, xi3, xl2, xl3, xl4         -

*    nm          - mean motion

*    z1, z2, z3, z11, z12, z13, z21, z22, z23, z31, z32, z33         -

*    zmol        -

*    zmos        -

*

*

*  coupling      :

*    none.

*

*  references    :

*    hoots, roehrich, norad spacetrack report #3 1980

*    hoots, norad spacetrack report #6 1986

*    hoots, schumacher and glover 2004

*    vallado, crawford, hujsak, kelso  2006

*------------------------------------------------------------------------------

SUBROUTINE DSCOM( EPOCH , Eccp  , Argpp , Tc    , Inclp , nodep,

&                  Np    ,

&                  SNODM , CNODM , SINIM , COSIM , SINOMM, COSOMM,

&                  DAY   , E3    , Ee2   , Eccm  , EMSQ  , GAM   ,

&                  Peo   , Pgho  , Pho   , PInco , Plo   ,

&                  RTemSq, Se2   , Se3   , Sgh2  , Sgh3  , Sgh4  ,

&                  Sh2   , Sh3   , Si2   , Si3   , Sl2   , Sl3   ,

&                  Sl4   , S1    , S2    , S3    , S4    , S5    ,

&                  S6    , S7    , SS1   , SS2   , SS3   , SS4   ,

&                  SS5   , SS6   , SS7   , SZ1   , SZ2   , SZ3   ,

&                  SZ11  , SZ12  , SZ13  , SZ21  , SZ22  , SZ23  ,

&                  SZ31  , SZ32  , SZ33  , Xgh2  , Xgh3  , Xgh4  ,

&                  Xh2   , Xh3   , Xi2   , Xi3   , Xl2   , Xl3   ,

&                  Xl4   , Xn    , Z1    , Z2    , Z3    , Z11   ,

&                  Z12   , Z13   , Z21   , Z22   , Z23   , Z31   ,

&                  Z32   , Z33   , Zmol  , Zmos )

IMPLICIT NONE

REAL*8  EPOCH , Eccp  , Argpp , Tc    , Inclp , nodep, Np    ,

&          SNODM , CNODM , SINIM , COSIM , SINOMM, COSOMM, DAY   ,

&          E3    , Ee2   , Eccm  , EMSQ  , GAM   , RTemSq, Se2   ,

&          Peo   , Pgho  , Pho   , PInco , Plo   ,

&          Se3   , Sgh2  , Sgh3  , Sgh4  , Sh2   , Sh3   , Si2   ,

&          Si3   , Sl2   , Sl3   , Sl4   , S1    , S2    , S3    ,

&          S4    , S5    , S6    , S7    , SS1   , SS2   , SS3   ,

&          SS4   , SS5   , SS6   , SS7   , SZ1   , SZ2   , SZ3   ,

&          SZ11  , SZ12  , SZ13  , SZ21  , SZ22  , SZ23  , SZ31  ,

&          SZ32  , SZ33  , Xgh2  , Xgh3  , Xgh4  , Xh2   , Xh3   ,

&          Xi2   , Xi3   , Xl2   , Xl3   , Xl4   , Xn    , Z1    ,

&          Z2    , Z3    , Z11   , Z12   , Z13   , Z21   , Z22   ,

&          Z23   , Z31   , Z32   , Z33   , Zmol  , Zmos

* -------------------------- Local Variables --------------------------

REAL*8  c1ss  , c1L   , zcosis, zsinis, zsings, zcosgs,

&          Zes   , zel

INTEGER LsFlg

REAL*8  a1    , a2    , a3    , a4    , a5    , a6    , a7    ,

&          a8    , a9    , a10   , betasq, cc    , ctem  , stem  ,

&          x1    , x2    , x3    , x4    , x5    , x6    , x7    ,

&          x8    , xnodce, xnoi  , zcosg , zcosgl, zcosh , zcoshl,

&          zcosi , zcosil, zsing , zsingl, zsinh , zsinhl, zsini ,

&          zsinil, zx    , zy

CHARACTER Help

INCLUDE 'ASTMATH.CMN'

* ------------------------------ Constants ----------------------------

ZES    =  0.01675D0

ZEL    =  0.05490D0

C1SS   =  2.9864797D-6

C1L    =  4.7968065D-7

ZSINIS =  0.39785416D0

ZCOSIS =  0.91744867D0

ZCOSGS =  0.1945905D0

ZSINGS = -0.98088458D0

* ----------------- DEEP SPACE PERIODICS INITIALIZATION ---------------

XN     = Np

Eccm   = Eccp

SNODM  = DSIN(nodep)

CNODM  = DCOS(nodep)

SINOMM = DSIN(Argpp)

COSOMM = DCOS(Argpp)

SINIM  = DSIN(Inclp)

COSIM  = DCOS(Inclp)

EMSQ   = Eccm*Eccm

BETASQ = 1.0D0-EMSQ

RTEMSQ = DSQRT(BETASQ)

* --------------------- INITIALIZE LUNAR SOLAR TERMS ------------------

PEO    = 0.0D0

PINCO  = 0.0D0

PLO    = 0.0D0

PGHO   = 0.0D0

PHO    = 0.0D0

DAY    = EPOCH + 18261.5D0 + TC/1440.0D0

XNODCE = DMOD(4.5236020D0 - 9.2422029D-4*DAY,TwoPi)

STEM   = DSIN(XNODCE)

CTEM   = DCOS(XNODCE)

ZCOSIL = 0.91375164D0 - 0.03568096D0*CTEM

ZSINIL = DSQRT(1.0D0 - ZCOSIL*ZCOSIL)

ZSINHL = 0.089683511D0*STEM / ZSINIL

ZCOSHL = DSQRT(1.0D0 - ZSINHL*ZSINHL)

GAM    = 5.8351514D0 + 0.0019443680D0*DAY

ZX     = 0.39785416D0*STEM/ZSINIL

ZY     = ZCOSHL*CTEM + 0.91744867D0*ZSINHL*STEM

ZX     = DATAN2(ZX,ZY)

ZX     = GAM + ZX - XNODCE

ZCOSGL = DCOS(ZX)

ZSINGL = DSIN(ZX)

* ---------------------------- DO SOLAR TERMS -------------------------

ZCOSG = ZCOSGS

ZSING = ZSINGS

ZCOSI = ZCOSIS

ZSINI = ZSINIS

ZCOSH = CNODM

ZSINH = SNODM

CC    = C1SS

XNOI  = 1.0D0 / XN

DO LSFlg = 1,2

A1 =   ZCOSG*ZCOSH + ZSING*ZCOSI*ZSINH

A3 =  -ZSING*ZCOSH + ZCOSG*ZCOSI*ZSINH

A7 =  -ZCOSG*ZSINH + ZSING*ZCOSI*ZCOSH

A8 =   ZSING*ZSINI

A9 =   ZSING*ZSINH + ZCOSG*ZCOSI*ZCOSH

A10=   ZCOSG*ZSINI

A2 =   COSIM*A7 + SINIM*A8

A4 =   COSIM*A9 + SINIM*A10

A5 =  -SINIM*A7 + COSIM*A8

A6 =  -SINIM*A9 + COSIM*A10

X1 =  A1*COSOMM + A2*SINOMM

X2 =  A3*COSOMM + A4*SINOMM

X3 = -A1*SINOMM + A2*COSOMM

X4 = -A3*SINOMM + A4*COSOMM

X5 =  A5*SINOMM

X6 =  A6*SINOMM

X7 =  A5*COSOMM

X8 =  A6*COSOMM

Z31= 12.0D0*X1*X1 - 3.0D0*X3*X3

Z32= 24.0D0*X1*X2 - 6.0D0*X3*X4

Z33= 12.0D0*X2*X2 - 3.0D0*X4*X4

Z1 =  3.0D0* (A1*A1 + A2*A2) + Z31*EMSQ

Z2 =  6.0D0* (A1*A3 + A2*A4) + Z32*EMSQ

Z3 =  3.0D0* (A3*A3 + A4*A4) + Z33*EMSQ

Z11= -6.0D0*A1*A5 + EMSQ* (-24.0D0*X1*X7-6.0D0*X3*X5)

Z12= -6.0D0* (A1*A6 + A3*A5) + EMSQ*

&           ( -24.0D0*(X2*X7+X1*X8) - 6.0D0*(X3*X6+X4*X5) )

Z13= -6.0D0*A3*A6 + EMSQ*(-24.0D0*X2*X8 - 6.0D0*X4*X6)

Z21=  6.0D0*A2*A5 + EMSQ*(24.0D0*X1*X5-6.0D0*X3*X7)

Z22=  6.0D0* (A4*A5 + A2*A6) + EMSQ*

&           (  24.0D0*(X2*X5+X1*X6) - 6.0D0*(X4*X7+X3*X8) )

Z23=  6.0D0*A4*A6 + EMSQ*(24.0D0*X2*X6 - 6.0D0*X4*X8)

Z1 = Z1 + Z1 + BETASQ*Z31

Z2 = Z2 + Z2 + BETASQ*Z32

Z3 = Z3 + Z3 + BETASQ*Z33

S3 = CC*XNOI

S2 = -0.5D0*S3 / RTEMSQ

S4 = S3*RTEMSQ

S1 = -15.0D0*Eccm*S4

S5 = X1*X3 + X2*X4

S6 = X2*X3 + X1*X4

S7 = X2*X4 - X1*X3

* ------------------------------ DO LUNAR TERMS -----------------------

IF (LSFLG.eq.1) THEN

SS1   = S1

SS2   = S2

SS3   = S3

SS4   = S4

SS5   = S5

SS6   = S6

SS7   = S7

SZ1   = Z1

SZ2   = Z2

SZ3   = Z3

SZ11  = Z11

SZ12  = Z12

SZ13  = Z13

SZ21  = Z21

SZ22  = Z22

SZ23  = Z23

SZ31  = Z31

SZ32  = Z32

SZ33  = Z33

ZCOSG = ZCOSGL

ZSING = ZSINGL

ZCOSI = ZCOSIL

ZSINI = ZSINIL

ZCOSH = ZCOSHL*CNODM+ZSINHL*SNODM

ZSINH = SNODM*ZCOSHL-CNODM*ZSINHL

CC    = C1L

ENDIF

ENDDO

ZMOL  = DMOD( 4.7199672D0 + 0.22997150D0*DAY-GAM,TwoPi )

ZMOS  = DMOD( 6.2565837D0 + 0.017201977D0*DAY,TwoPi )

* ---------------------------- DO SOLAR TERMS -------------------------

SE2 =   2.0D0*SS1*SS6

SE3 =   2.0D0*SS1*SS7

SI2 =   2.0D0*SS2*SZ12

SI3 =   2.0D0*SS2*(SZ13-SZ11)

SL2 =  -2.0D0*SS3*SZ2

SL3 =  -2.0D0*SS3*(SZ3-SZ1)

SL4 =  -2.0D0*SS3*(-21.0D0-9.0D0*EMSQ)*ZES

SGH2=   2.0D0*SS4*SZ32

SGH3=   2.0D0*SS4*(SZ33-SZ31)

SGH4= -18.0D0*SS4*ZES

SH2 =  -2.0D0*SS2*SZ22

SH3 =  -2.0D0*SS2*(SZ23-SZ21)

* ---------------------------- DO LUNAR TERMS -------------------------

EE2 =   2.0D0*S1*S6

E3  =   2.0D0*S1*S7

XI2 =   2.0D0*S2*Z12

XI3 =   2.0D0*S2*(Z13-Z11)

XL2 =  -2.0D0*S3*Z2

XL3 =  -2.0D0*S3*(Z3-Z1)

XL4 =  -2.0D0*S3*(-21.0D0-9.0D0*EMSQ)*ZEL

XGH2=   2.0D0*S4*Z32

XGH3=   2.0D0*S4*(Z33-Z31)

XGH4= -18.0D0*S4*ZEL

XH2 =  -2.0D0*S2*Z22

XH3 =  -2.0D0*S2*(Z23-Z21)

RETURN

END  !  dscom

* -----------------------------------------------------------------------------

*

*                           SUBROUTINE DSINIT

*

*  This Subroutine provides Deep Space contributions to Mean Motion Dot due

*    to geopotential resonance with half day and one day orbits.

*

*  Inputs        :

*    Cosim, Sinim-

*    Emsq        - Eccentricity squared

*    Argpo       - Argument of Perigee

*    S1, S2, S3, S4, S5      -

*    Ss1, Ss2, Ss3, Ss4, Ss5 -

*    Sz1, Sz3, Sz11, Sz13, Sz21, Sz23, Sz31, Sz33 -

*    T           - Time

*    Tc          -

*    GSTo        - Greenwich sidereal time                   rad

*    Mo          - Mean Anomaly

*    MDot        - Mean Anomaly dot (rate)

*    No          - Mean Motion

*    nodeo       - right ascension of ascending node

*    nodeDot     - right ascension of ascending node dot (rate)

*    XPIDOT      -

*    Z1, Z3, Z11, Z13, Z21, Z23, Z31, Z33 -

*    Eccm        - Eccentricity

*    Argpm       - Argument of perigee

*    Inclm       - Inclination

*    Mm          - Mean Anomaly

*    Xn          - Mean Motion

*    nodem       - right ascension of ascending node

*

*  Outputs       :

*    Eccm        - Eccentricity

*    Argpm       - Argument of perigee

*    Inclm       - Inclination

*    Mm          - Mean Anomaly

*    Xn          - Mean motion

*    nodem       - right ascension of ascending node

*    IRez        - Resonance flags              0-none, 1-One day,  2-Half day

*    Atime       -

*    D2201, D2211, D3210, D3222, D4410, D4422, D5220, D5232, D5421, D5433       -

*    Dedt        -

*    Didt        -

*    DMDT        -

*    DNDT        -

*    DNODT       -

*    DOMDT       -

*    Del1, Del2, Del3 -

*    Ses  , Sghl , Sghs , Sgs  , Shl  , Shs  , Sis  , Sls

*    THETA       -

*    Xfact       -

*    Xlamo       -

*    Xli         -

*    Xni

*

*  Locals        :

*    ainv2       -

*    aonv        -

*    cosisq      -

*    eoc         -

*    f220, f221, f311, f321, f322, f330, f441, f442, f522, f523, f542, f543        -

*    g200, g201, g211, g300, g310, g322, g410, g422, g520, g521, g532, g533        -

*    sini2       -

*    temp, temp1 -

*    Theta       -

*    xno2        -

*

*  Coupling      :

*    getgravconst-

*

*  references    :

*    hoots, roehrich, norad spacetrack report #3 1980

*    hoots, norad spacetrack report #6 1986

*    hoots, schumacher and glover 2004

*    vallado, crawford, hujsak, kelso  2006

*------------------------------------------------------------------------------

SUBROUTINE DSINIT( whichconst,

&                   Cosim , Emsq  , Argpo , S1    , S2    , S3    ,

&                   S4    , S5    , Sinim , Ss1   , Ss2   , Ss3   ,

&                   Ss4   , Ss5   , Sz1   , Sz3   , Sz11  , Sz13  ,

&                   Sz21  , Sz23  , Sz31  , Sz33  , T     , Tc    ,

&                   GSTo  , Mo    , MDot  , No    , nodeo ,nodeDot,

&                   XPIDOT, Z1    , Z3    , Z11   , Z13   , Z21   ,

&                   Z23   , Z31   , Z33   , Ecco  , EccSq ,

&                   Eccm  , Argpm , Inclm , Mm    , Xn    , nodem,

&                   IREZ  , Atime , D2201 , D2211 , D3210 , D3222 ,

&                   D4410 , D4422 , D5220 , D5232 , D5421 , D5433 ,

&                   Dedt  , Didt  , DMDT  , DNDT  , DNODT , DOMDT ,

&                   Del1  , Del2  , Del3  , Xfact , Xlamo , Xli   ,

&                   Xni )

IMPLICIT NONE

INTEGER  IRez, whichconst

REAL*8   Cosim , Emsq  , Argpo , S1    , S2    , S3    , S4    ,

&           S5    , Sinim , Ss1   , Ss2   , Ss3   , Ss4   , Ss5   ,

&           Sz1   , Sz3   , Sz11  , Sz13  , Sz21  , Sz23  , Sz31  ,

&           Sz33  , T     , Tc    , GSTo  , Mo    , MDot  , No    ,

&           nodeo ,nodeDot,XPIDOT , Z1    , Z3    , Z11   , Z13   ,

&           Z21   , Z23   , Z31   , Z33   , Eccm  , Argpm , Inclm ,

&           Mm    , Xn    , nodem , Atime , D2201 , D2211 , D3210 ,

&           D3222 , D4410 , D4422 , D5220 , D5232 , D5421 , D5433 ,

&           Dedt  , Didt  , DMDT  , DNDT  , DNODT , DOMDT , Del1  ,

&           Del2  , Del3  , Xfact , Xlamo , Xli   , Xni   , Ecco  ,

&           Eccsq

* -------------------------- Local Variables --------------------------

REAL*8  ainv2 , aonv  , cosisq, eoc   , f220  , f221  , f311  ,

&          f321  , f322  , f330  , f441  , f442  , f522  , f523  ,

&          f542  , f543  , g200  , g201  , g211  , g300  , g310  ,

&          g322  , g410  , g422  , g520  , g521  , g532  , g533  ,

&          ses   , sgs   , sghl  , sghs  , shs   , shl   , sis   ,

&          sini2 , sls   , temp  , temp1 , Theta , xno2

REAL*8  Q22   , Q31   , Q33   , ROOT22, ROOT44, ROOT54,

&          RPTim , Root32, Root52, X2o3  , XKe   , Znl   ,

&          Zns,  Emo, emsqo , tumin, mu, radiusearthkm, j2, j3, j4,

&          j3oj2

CHARACTER Help

INCLUDE 'ASTMATH.CMN'

Q22    = 1.7891679D-6

Q31    = 2.1460748D-6

Q33    = 2.2123015D-7

ROOT22 = 1.7891679D-6

ROOT44 = 7.3636953D-9

ROOT54 = 2.1765803D-9

RPTim  = 4.37526908801129966D-3 ! this equates to 7.29211514668855e-5 rad/sec

Root32 = 3.7393792D-7

Root52 = 1.1428639D-7

X2o3   = 2.0D0 / 3.0D0

ZNL    = 1.5835218D-4

ZNS    = 1.19459D-5

! sgp4fix identify constants and allow alternate values

CALL getgravconst( whichconst, tumin, mu, radiusearthkm, xke,

&       j2, j3, j4, j3oj2 )

* ------------------------ DEEP SPACE INITIALIZATION ------------------

IREZ = 0

IF ((XN.lt.0.0052359877D0).AND.(XN.GT.0.0034906585D0)) THEN

IREZ = 1

ENDIF

IF ((XN.ge.8.26D-3).AND.(XN.LE.9.24D-3).AND.(Eccm.GE.0.5D0))THEN

IREZ = 2

ENDIF

* ---------------------------- DO SOLAR TERMS -------------------------

SES  =  SS1*ZNS*SS5

SIS  =  SS2*ZNS*(SZ11 + SZ13)

SLS  = -ZNS*SS3*(SZ1 + SZ3 - 14.0D0 - 6.0D0*EMSQ)

SGHS =  SS4*ZNS*(SZ31 + SZ33 - 6.0D0)

SHS  = -ZNS*SS2*(SZ21 + SZ23)

c       sgp4fix for 180 deg incl

IF ((Inclm.lt.5.2359877D-2).or.(Inclm.gt.pi-5.2359877D-2)) THEN

SHS = 0.0D0

ENDIF

IF (SINIM.ne.0.0D0) THEN

SHS = SHS/SINIM

ENDIF

SGS  = SGHS - COSIM*SHS

* ----------------------------- DO LUNAR TERMS ------------------------

DEDT = SES + S1*ZNL*S5

DIDT = SIS + S2*ZNL*(Z11 + Z13)

DMDT = SLS - ZNL*S3*(Z1 + Z3 - 14.0D0 - 6.0D0*EMSQ)

SGHL = S4*ZNL*(Z31 + Z33 - 6.0D0)

SHL  = -ZNL*S2*(Z21 + Z23)

c       sgp4fix for 180 deg incl

IF ((Inclm.lt.5.2359877D-2).or.(Inclm.gt.pi-5.2359877D-2)) THEN

SHL = 0.0D0

ENDIF

DOMDT= SGS+SGHL

DNODT= SHS

IF (SINIM .ne. 0.0D0) THEN

DOMDT = DOMDT-COSIM/SINIM*SHL

DNODT = DNODT+SHL/SINIM

ENDIF

* --------------- CALCULATE DEEP SPACE RESONANCE EFFECTS --------------

DNDT  = 0.0D0

THETA = DMOD(GSTo + TC*RPTIM,TwoPi)

Eccm  = Eccm + DEDT*T

emsq  = eccm**2

Inclm = Inclm + DIDT*T

Argpm = Argpm + DOMDT*T

nodem = nodem + DNODT*T

Mm    = Mm + DMDT*T

* ------------------ Initialize the resonance terms -------------------

IF (IREZ .ne. 0) THEN

AONV = (XN/XKE)**X2O3

* -------------- GEOPOTENTIAL RESONANCE FOR 12 HOUR ORBITS ------------

IF (IREZ .eq. 2) THEN

COSISQ = COSIM*COSIM

emo    = Eccm

emsqo  = emsq

Eccm   = ecco

emsq   = eccsq

EOC    = Eccm*EMSQ

G201   = -0.306D0-(Eccm-0.64D0)*0.440D0

IF (Eccm.le.0.65D0) THEN

G211 =   3.616D0 -  13.2470D0*Eccm +  16.2900D0*EMSQ

G310 = -19.302D0 + 117.3900D0*Eccm - 228.4190D0*EMSQ +

&                 156.591D0*EOC

G322 = -18.9068D0+ 109.7927D0*Eccm - 214.6334D0*EMSQ +

&                 146.5816D0*EOC

G410 = -41.122D0 + 242.6940D0*Eccm - 471.0940D0*EMSQ +

&                 313.953D0*EOC

G422 =-146.407D0 + 841.8800D0*Eccm - 1629.014D0*EMSQ +

&                1083.435D0*EOC

G520 =-532.114D0 + 3017.977D0*Eccm - 5740.032D0*EMSQ +

&                3708.276D0*EOC

ELSE

G211 =  -72.099D0 +  331.819D0*Eccm -  508.738D0*EMSQ +

&                  266.724D0*EOC

G310 = -346.844D0 + 1582.851D0*Eccm - 2415.925D0*EMSQ +

&                 1246.113D0*EOC

G322 = -342.585D0 + 1554.908D0*Eccm - 2366.899D0*EMSQ +

&                 1215.972D0*EOC

G410 =-1052.797D0 + 4758.686D0*Eccm - 7193.992D0*EMSQ +

&                 3651.957D0*EOC

G422 =-3581.690D0 + 16178.11D0*Eccm - 24462.77D0*EMSQ +

&                12422.52D0*EOC

IF (Eccm.gt.0.715D0) THEN

G520 =-5149.66D0 + 29936.92D0*Eccm -54087.36D0*EMSQ

&                    + 31324.56D0*EOC

ELSE

G520 = 1464.74D0 -  4664.75D0*Eccm + 3763.64D0*EMSQ

ENDIF

ENDIF

IF (Eccm.lt.0.7D0) THEN

G533 = -919.22770D0 + 4988.6100D0*Eccm-9064.7700D0*EMSQ

&               + 5542.21D0*EOC

G521 = -822.71072D0 + 4568.6173D0*Eccm-8491.4146D0*EMSQ

&               + 5337.524D0*EOC

G532 = -853.66600D0 + 4690.2500D0*Eccm-8624.7700D0*EMSQ

&               + 5341.4D0*EOC

ELSE

G533 =-37995.780D0 + 161616.52D0*Eccm-229838.20D0*EMSQ+

&              109377.94D0*EOC

G521 =-51752.104D0 + 218913.95D0*Eccm-309468.16D0*EMSQ+

&              146349.42D0*EOC

G532 =-40023.880D0 + 170470.89D0*Eccm-242699.48D0*EMSQ+

&              115605.82D0*EOC

ENDIF

SINI2 =  SINIM*SINIM

F220  =  0.75D0* (1.0D0+2.0D0*COSIM+COSISQ)

F221  =  1.5D0*SINI2

F321  =  1.875D0*SINIM * (1.0D0-2.0D0*COSIM-3.0D0*COSISQ)

F322  = -1.875D0*SINIM * (1.0D0+2.0D0*COSIM-3.0D0*COSISQ)

F441  = 35.0D0*SINI2*F220

F442  = 39.3750D0*SINI2*SINI2

F522  =  9.84375D0*SINIM * (SINI2* (1.0D0-2.0D0*COSIM-

&               5.0D0*COSISQ)+0.33333333D0 * (-2.0D0+4.0D0*COSIM+

&               6.0D0*COSISQ) )

F523  =  SINIM * (4.92187512D0*SINI2 * (-2.0D0-4.0D0*COSIM+

&               10.0D0*COSISQ) + 6.56250012D0*

&               (1.0D0+2.0D0*COSIM-3.0D0*COSISQ))

F542  =  29.53125D0*SINIM * (2.0D0-8.0D0*COSIM+COSISQ*

&               (-12.0D0+8.0D0*COSIM+10.0D0*COSISQ) )

F543  = 29.53125D0*SINIM * (-2.0D0-8.0D0*COSIM+COSISQ*

&               (12.0D0+8.0D0*COSIM-10.0D0*COSISQ) )

XNO2   =  XN * XN

AINV2  =  AONV * AONV

TEMP1  =  3.0D0*XNO2*AINV2

TEMP   =  TEMP1*ROOT22

D2201  =  TEMP*F220*G201

D2211  =  TEMP*F221*G211

TEMP1  =  TEMP1*AONV

TEMP   =  TEMP1*ROOT32

D3210  =  TEMP*F321*G310

D3222  =  TEMP*F322*G322

TEMP1  =  TEMP1*AONV

TEMP   =  2.0D0*TEMP1*ROOT44

D4410  =  TEMP*F441*G410

D4422  =  TEMP*F442*G422

TEMP1  =  TEMP1*AONV

TEMP   =  TEMP1*ROOT52

D5220  =  TEMP*F522*G520

D5232  =  TEMP*F523*G532

TEMP   =  2.0D0*TEMP1*ROOT54

D5421  =  TEMP*F542*G521

D5433  =  TEMP*F543*G533

XLAMO  =  DMOD(Mo+nodeo+nodeo-THETA-THETA,TwoPi)

XFACT  = MDot + DMDT + 2.0D0 * (nodeDot+DNODT-RPTIM) - No

Eccm = emo

emsq = emsqo

ENDIF

IF (Irez .eq. 1) THEN

* -------------------- SYNCHRONOUS RESONANCE TERMS --------------------

G200  = 1.0D0 + EMSQ * (-2.5D0+0.8125D0*EMSQ)

G310  = 1.0D0 + 2.0D0*EMSQ

G300  = 1.0D0 + EMSQ * (-6.0D0+6.60937D0*EMSQ)

F220  = 0.75D0 * (1.0D0+COSIM) * (1.0D0+COSIM)

F311  = 0.9375D0*SINIM*SINIM*

&               (1.0D0+3.0D0*COSIM) - 0.75D0*(1.0D0+COSIM)

F330  = 1.0D0+COSIM

F330  = 1.875D0*F330*F330*F330

DEL1  = 3.0D0*XN*XN*AONV*AONV

DEL2  = 2.0D0*DEL1*F220*G200*Q22

DEL3  = 3.0D0*DEL1*F330*G300*Q33*AONV

DEL1  = DEL1*F311*G310*Q31*AONV

XLAMO = DMOD(Mo+nodeo+Argpo-THETA,TwoPi)

XFACT = MDot + XPIDOT - RPTIM + DMDT + DOMDT + DNODT - No

ENDIF

* ---------------- FOR SGP4, INITIALIZE THE INTEGRATOR ----------------

XLI   = XLAMO

XNI   = No

ATIME = 0.0D0

XN    = No + DNDT

ENDIF ! Ires non-zero

RETURN

END  ! end dsinit

* -----------------------------------------------------------------------------

*

*                           SUBROUTINE DSPACE

*

*  This Subroutine provides deep space contributions to mean elements for

*    perturbing third body.  these effects have been averaged over one

*    revolution of the sun and moon.  for earth resonance effects, the

*    effects have been averaged over no revolutions of the satellite.

*    (mean motion)

*

*  author        : david vallado                  719-573-2600   28 jun 2005

*

*  inputs        :

*    d2201, d2211, d3210, d3222, d4410, d4422, d5220, d5232, d5421, d5433       -

*    dedt        -

*    del1, del2, del3  -

*    didt        -

*    dmdt        -

*    dnodt       -

*    domdt       -

*    irez        - flag for resonance           0-none, 1-one day, 2-half day

*    argpo       - argument of perigee

*    argpdot     - argument of perigee dot (rate)

*    t           - time

*    tc          -

*    gsto        - gst

*    xfact       -

*    xlamo       -

*    no          - mean motion

*    atime       -

*    em          - eccentricity

*    ft          -

*    argpm       - argument of perigee

*    inclm       - inclination

*    xli         -

*    mm          - mean anomaly

*    xni         - mean motion

*    nodem       - right ascension of ascending node

*

*  outputs       :

*    atime       -

*    em          - eccentricity

*    argpm       - argument of perigee

*    inclm       - inclination

*    xli         -

*    mm          - mean anomaly

*    xni         -

*    nodem       - right ascension of ascending node

*    dndt        -

*    nm          - mean motion

*

*

*  coupling      :

*    none        -

*

*  references    :

*    hoots, roehrich, norad spacetrack report #3 1980

*    hoots, norad spacetrack report #6 1986

*    hoots, schumacher and glover 2004

*    vallado, crawford, hujsak, kelso  2006

*------------------------------------------------------------------------------

SUBROUTINE DSPACE( IRez  , D2201 , D2211 , D3210 , D3222 , D4410 ,

&                   D4422 , D5220 , D5232 , D5421 , D5433 , Dedt  ,

&                   Del1  , Del2  , Del3  , Didt  , Dmdt  , Dnodt ,

&                   Domdt , Argpo , ArgpDot, T    , TC    , GSTo  ,

&                   Xfact , Xlamo , No    ,

&                   Atime , Eccm  , Argpm , Inclm , Xli   , Mm  ,

&                   XNi   , nodem, Dndt  , XN  )

IMPLICIT NONE

INTEGER  IRez

Real*8   D2201 , D2211 , D3210 , D3222 , D4410 , D4422 , D5220 ,

&           D5232 , D5421 , D5433 , Dedt  , Del1  , Del2  , Del3  ,

&           Didt  , Dmdt  , Dnodt , Domdt , Argpo , ArgpDot,T     ,

&           TC    , GSTo  , Xfact , Xlamo , No    , Atime , Eccm  ,

&           Argpm , Inclm , Xli   , Mm    , Xni   , nodem, Dndt  ,

&           XN

* -------------------------- Local Variables --------------------------

INTEGER  iretn , iret

REAL*8   Delt  , Ft    , theta , x2li  , x2omi , xl    , xldot ,

&           xnddt , xndt  , xomi

REAL*8   G22   , G32   , G44   , G52   , G54   , Fasx2 ,

&           Fasx4 , Fasx6 , RPtim , Step2 , Stepn , Stepp

COMMON /DebugHelp/ Help

CHARACTER Help

INCLUDE 'ASTMATH.CMN'

* ----------------------------- Constants -----------------------------

FASX2 = 0.13130908D0

FASX4 = 2.8843198D0

FASX6 = 0.37448087D0

G22   = 5.7686396D0

G32   = 0.95240898D0

G44   = 1.8014998D0

G52   = 1.0508330D0

G54   = 4.4108898D0

RPTIM = 4.37526908801129966D-3

STEPP =    720.0D0

STEPN =   -720.0D0

STEP2 = 259200.0D0

* --------------- CALCULATE DEEP SPACE RESONANCE EFFECTS --------------

DNDT  = 0.0D0

THETA = DMOD(GSTo + TC*RPTIM,TwoPi)

Eccm  = Eccm + DEDT*T

Inclm = Inclm + DIDT*T

Argpm = Argpm + DOMDT*T

nodem = nodem + DNODT*T

Mm    = Mm + DMDT*T

c   sgp4fix for negative inclinations

c   the following if statement should be commented out

c        IF(Inclm .lt. 0.0D0) THEN

c            Inclm  = -Inclm

c            Argpm  = Argpm-PI

c            nodem = nodem+PI

c          ENDIF

c   sgp4fix for propagator problems

c   the following integration works for negative time steps and periods

c   the specific changes are unknown because the original code was so convoluted

c      sgp4fix take out atime = 0.0 and fix for faster operation

Ft    = 0.0D0      ! Just in case - should be set in loops if used.

IF (IREZ .ne. 0) THEN

* ----- UPDATE RESONANCES : NUMERICAL (EULER-MACLAURIN) INTEGRATION ---

* ---------------------------- EPOCH RESTART --------------------------

! sgp4fix streamline check

IF ((atime .eq. 0.0D0) .or. (t * atime .le. 0.0D0) .or.

&       (dabs(t) .lt. dabs(atime)) ) THEN

atime  = 0.0D0

xni    = no

xli    = xlamo

ENDIF

! sgp4fix move check outside loop

IF (t .gt. 0.0D0) THEN

delt = stepp

else

delt = stepn

ENDIF

iretn = 381 ! added for do loop

iret  =   0 ! added for loop

DO WHILE (IRetn.eq.381)

* --------------------------- DOT TERMS CALCULATED --------------------

* ------------------- NEAR - SYNCHRONOUS RESONANCE TERMS --------------

IF (IREZ .ne. 2) THEN

XNDT  = DEL1*DSIN(XLI-FASX2) +

&                  DEL2*DSIN(2.0D0*(XLI-FASX4)) +

&                  DEL3*DSIN(3.0D0*(XLI-FASX6))

XLDOT = XNI + XFACT

XNDDT = DEL1*DCOS(XLI-FASX2) +

&            2.0D0*DEL2*DCOS(2.0D0*(XLI-FASX4)) +

&            3.0D0*DEL3*DCOS(3.0D0*(XLI-FASX6))

XNDDT = XNDDT*XLDOT

ELSE

* --------------------- NEAR - HALF-DAY RESONANCE TERMS ---------------

XOMI = Argpo + ArgpDot*ATIME

X2OMI= XOMI + XOMI

X2LI = XLI + XLI

XNDT = D2201*DSIN(X2OMI+XLI-G22) + D2211*DSIN(XLI-G22) +

&                 D3210*DSIN( XOMI+XLI-G32) +

&                 D3222*DSIN(-XOMI+XLI-G32) +

&                 D4410*DSIN(X2OMI+X2LI-G44)+ D4422*DSIN(X2LI-G44)+

&                 D5220*DSIN( XOMI+XLI-G52) +

&                 D5232*DSIN(-XOMI+XLI-G52) +

&                 D5421*DSIN( XOMI+X2LI-G54)+

&                 D5433*DSIN(-XOMI+X2LI-G54)

XLDOT = XNI+XFACT

XNDDT = D2201*DCOS(X2OMI+XLI-G22) + D2211*DCOS(XLI-G22)+

&                  D3210*DCOS( XOMI+XLI-G32) +

&                  D3222*DCOS(-XOMI+XLI-G32) +

&                  D5220*DCOS( XOMI+XLI-G52) +

&                  D5232*DCOS(-XOMI+XLI-G52) +

&                  2.0D0*(D4410*DCOS(X2OMI+X2LI-G44) +

&                  D4422*DCOS(X2LI-G44) +

&                  D5421*DCOS( XOMI+X2LI-G54) +

&                  D5433*DCOS(-XOMI+X2LI-G54))

XNDDT = XNDDT*XLDOT

ENDIF

* ------------------------------- INTEGRATOR --------------------------

!  sgp4fix move end checks to end of routine

IF (DABS(T-ATIME).ge.STEPP) THEN

IRET  = 0

IRETN = 381

ELSE

FT    = T-ATIME

IRETN = 0

ENDIF

IF (IRETN.EQ.381) THEN

XLI   = XLI + XLDOT*DELT + XNDT*STEP2

XNI   = XNI + XNDT*DELT + XNDDT*STEP2

ATIME = ATIME + DELT

ENDIF

ENDDO

XN = XNI + XNDT*FT  + XNDDT*FT*FT*0.5D0

XL = XLI + XLDOT*FT + XNDT*FT*FT*0.5D0

IF(IREZ .ne. 1) THEN

Mm   = XL-2.0D0*nodem+2.0D0*THETA

DNDT = XN-No

ELSE

Mm   = XL-nodem-Argpm+THETA

DNDT = XN-No

ENDIF

XN = No + DNDT

ENDIF

RETURN

END  ! end dspace

* -----------------------------------------------------------------------------

*

*                           SUBROUTINE JDay1

*

*  This subroutine finds the Julian date given the Year, Month, Day, and Time.

*

*  Author        : David Vallado                  719-573-2600    1 Mar 2001

*

*  Inputs          Description                    Range / Units

*    Year        - Year                           1900 .. 2100

*    Mon         - Month                          1 .. 12

*    Day         - Day                            1 .. 28,29,30,31

*    Hr          - Universal Time Hour            0 .. 23

*    Min         - Universal Time Min             0 .. 59

*    Sec         - Universal Time Sec             0.0D0 .. 59.999D0

*    WhichType   - Julian .or. Gregorian calender   'J' .or. 'G'

*

*  Outputs       :

*    JD          - Julian Date                    days from 4713 BC

*

*  Locals        :

*    B           - Var to aid Gregorian dates

*

*  Coupling      :

*    None.

*

*  References    :

*    Vallado       2007, 189, Alg 14, Ex 3-14

* -----------------------------------------------------------------------------

SUBROUTINE JDAY1        ( Year,Mon,Day,Hr,Min, Sec, JD )

IMPLICIT NONE

INTEGER Year, Mon, Day, Hr, Min

REAL*8  Sec, JD

! --------------------  Implementation   ----------------------

JD= 367.0D0 * Year

&        - INT( (7* (Year+INT ( (Mon+9)/12.0) ) ) * 0.25D0 )

&        + INT( 275*Mon / 9.0 )

&        + Day + 1721013.5D0

&        + ( (Sec/60.0D0 + Min ) / 60.0D0 + Hr ) / 24.0D0

*     &      - 0.5D0*DSIGN(1.0D0, 100.0D0*Year + Mon - 190002.5D0) + 0.5D0

RETURN

END  ! end jday1

* -----------------------------------------------------------------------------

*

*                           SUBROUTINE DAYS2MDHMS

*

*  This subroutine converts the day of the year, days, to the equivalent month

*    day, hour, Minute and second.

*

*  Algorithm     : Set up array for the Number of days per month

*                  Find Leap Year - be sure to account for the 400 years

*                  Loop through a Temp value for WHILE the value is .lt. the days

*                  Perform INTEGER conversions to the correct day and month

*                  Convert remainder into H M S using type conversions

*

*  Author        : David Vallado                  719-573-2600    1 Mar 2001

*

*  Inputs          Description                    Range / Units

*    Year        - Year                          +1900 .. 2100+

*    Days        - Julian Day of the year         0.0D0  .. 366.0D0

*

*  OutPuts       :

*    Mon         - Month                          1 .. 12

*    Day         - Day                            1 .. 28,29,30,31

*    Hr          - Hour                           0 .. 23

*    Min         - Minute                         0 .. 59

*    Sec         - Second                         0.0D0 .. 59.999D0

*

*  Locals        :

*    DayofYr     - Day of year

*    Temp        - Temporary REAL*8 values

*    IntTemp     - Temporary INTEGER value

*    i           - Index

*    LMonth[12]  - INTEGER Array containing the Number of days per month

*

*  Coupling      :

*    None.

* -----------------------------------------------------------------------------

SUBROUTINE DAYS2MDHMS  ( Year,Days,  Mon,Day,Hr,Min,Sec )

IMPLICIT NONE

REAL*8 Days,Sec

INTEGER Year, Mon, Day, Hr, Min

* ----------------------------  Locals  -------------------------------

INTEGER IntTemp,i,DayofYr, LMonth(12)

REAL*8 Temp

! --------------------  Implementation   ----------------------

! -------------- Set up array of days in month  ---------------

DO i = 1,12

LMonth(i) = 31

ENDDO

LMonth( 2) = 28

LMonth( 4) = 30

LMonth( 6) = 30

LMonth( 9) = 30

LMonth(11) = 30

DayofYr= IDINT(Days )

! ---------------- Find month and Day of month ----------------

IF (MOD(Year,4).eq.0) THEN

LMonth(2)= 29

ENDIF

i= 1

IntTemp= 0

DO WHILE ( (DayofYr.gt.IntTemp + LMonth(i) ) .and. ( i.lt.12 ))

IntTemp= IntTemp + LMonth(i)

i= i+1

ENDDO

Mon= i

Day= DayofYr - IntTemp

! ---------------- Find hours Minutes and seconds -------------

Temp= (Days - DayofYr )*24.0D0

Hr  = IDINT( Temp )

Temp= (Temp-Hr) * 60.0D0

Min = IDINT( Temp )

Sec = (Temp-Min) * 60.0D0

RETURN

END  ! end days2mdhms

*******************************************************8

Subroutine HMS (EHour, EHr, EMin, ESec)

Implicit None

Real*8 EHour, TempMin1, TempMin2, TempSec, ESec

Integer EHr, EMin

EHr = Int(EHour)

TempMin1=EHour-EHr

TempMin2=Tempmin1*60.0d0

EMin= Int(TempMin2)

TempSec = TempMin2-EMin

ESec = TempSec*60.0D0

End

## A.2 Other Codes

Attached on the accompanying CD, there are other codes in their entirety.  These include:

- PREDICT (Feb. Version) with updated SGP4 propagator.

- MATLAB version of Vallado’s SGP4 propagator.

- FORTRAN version of Vallado’s SGP4 propagator.

Additionally, the analysis files are also provided on the CD.

# APPENDIX B. Verification TLE File

#   ------------------ Verification test cases ----------------------

#                       # TEME example

1 00005U 58002B   00179.78495062  .00000023  00000-0  28098-4 0  4753

2 00005  34.2682 348.7242 1859667 331.7664  19.3264 10.82419157413667     0.00      4320.0        360.00

#                       ## fig show lyddane fix error with gsfc ver

1 04632U 70093B   04031.91070959 -.00000084  00000-0  10000-3 0  9955

2 04632  11.4628 273.1101 1450506 207.6000 143.9350  1.20231981 44145  -5184.0     -4896.0        120.00

#   DELTA 1 DEB         # near earth normal drag equation

#                       # perigee = 377.26km, so moderate drag case

1 06251U 62025E   06176.82412014  .00008885  00000-0  12808-3 0  3985

2 06251  58.0579  54.0425 0030035 139.1568 221.1854 15.56387291  6774      0.0      2880.0        120.00

#   MOLNIYA 2-14              # 12h resonant ecc in 0.65 to 0.7 range

1 08195U 75081A   06176.33215444  .00000099  00000-0  11873-3 0   813

2 08195  64.1586 279.0717 6877146 264.7651  20.2257  2.00491383225656      0.0      2880.0        120.00

#   MOLNIYA 1-36              ## fig 12h resonant ecc in 0.7 to 0.715 range

1 09880U 77021A   06176.56157475  .00000421  00000-0  10000-3 0  9814

2 09880  64.5968 349.3786 7069051 270.0229  16.3320  2.00813614112380      0.0      2880.0        120.00

#   SMS 1 AKM           # show the integrator problem with gsfc ver

1 09998U 74033F   05148.79417928 -.00000112  00000-0  00000+0 0  4480

2 09998   9.4958 313.1750 0270971 327.5225  30.8097  1.16186785 45878  -1440.0      -720.00         60.0

#                       # Original STR#3 SDP4 test

1 11801U          80230.29629788  .01431103  00000-0  14311-1      13

2 11801  46.7916 230.4354 7318036  47.4722  10.4117  2.28537848    13      0.0      1440.0        360.00

#   EUTELSAT 1-F1 (ECS1)## fig lyddane choice in GSFC at 2080 min

1 14128U 83058A   06176.02844893 -.00000158  00000-0  10000-3 0  9627

2 14128  11.4384  35.2134 0011562  26.4582 333.5652  0.98870114 46093      0.0      2880.0        120.00

#   SL-6 R/B(2)         # Deep space, perigee = 82.48 (<98) for

#                       # s4 > 20 mod

1 16925U 86065D   06151.67415771  .02550794 -30915-6  18784-3 0  4486

2 16925  62.0906 295.0239 5596327 245.1593  47.9690  4.88511875148616      0.0      1440.0        120.00

#   SL-12 R/B           # Shows Lyddane choice at 1860 and 4700 min

1 20413U 83020D   05363.79166667  .00000000  00000-0  00000+0 0  7041

2 20413  12.3514 187.4253 7864447 196.3027 356.5478  0.24690082  7978   1440.0      4320.0        120.00

#   MOLNIYA 1-83                # 12h resonant, ecc > 0.715 (negative BSTAR)

1 21897U 92011A   06176.02341244 -.00001273  00000-0 -13525-3 0  3044

2 21897  62.1749 198.0096 7421690 253.0462  20.1561  2.01269994104880      0.0      2880.0        120.00

#   SL-6 R/B(2)         # last tle given, decayed 2006-04-04, day 94

1 22312U 93002D   06094.46235912  .99999999  81888-5  49949-3 0  3953

2 22312  62.1486  77.4698 0308723 267.9229  88.7392 15.95744531 98783  54.2028672   1440.0         20.00

#   SL-6 R/B(2)         # 12h resonant ecc in the > 0.715 range

1 22674U 93035D   06176.55909107  .00002121  00000-0  29868-3 0  6569

2 22674  63.5035 354.4452 7541712 253.3264  18.7754  1.96679808 93877      0.0      2880.0        120.00

#   ARIANE 44L+ R/B     # Lyddane bug at <= 70 min for atan2(),

#                       # no quadrant fix

1 23177U 94040C   06175.45752052  .00000386  00000-0  76590-3 0    95

2 23177   7.0496 179.8238 7258491 296.0482   8.3061  2.25906668 97438      0.0      1440.0        120.00

#   WIND                        # STR#3 Kepler failes past about 200 min

1 23333U 94071A   94305.49999999 -.00172956  26967-3  10000-3 0    15

2 23333  28.7490   2.3720 9728298  30.4360   1.3500  0.07309491    70      0.0      1600.0        120.00

#   ARIANE 42P+3 R/B    ## fig Lyddane bug at > 280.5 min for AcTan()

1 23599U 95029B   06171.76535463  .00085586  12891-6  12956-2 0  2905

2 23599   6.9327   0.2849 5782022 274.4436  25.2425  4.47796565123555      0.0       720.0         20.00

#   ITALSAT 2           # 24h resonant GEO, inclination > 3 deg

1 24208U 96044A   06177.04061740 -.00000094  00000-0  10000-3 0  1600

2 24208   3.8536  80.0121 0026640 311.0977  48.3000  1.00778054 36119      0.0      1440.0        120.00

#   AMC-4               ## fig low incl, show incl shift with

#                       ## gsfc version from 240 to 1440 min

1 25954U 99060A   04039.68057285 -.00000108  00000-0  00000-0 0  6847

2 25954   0.0004 243.8136 0001765  15.5294  22.7134  1.00271289 15615  -1440.0      1440.0        120.00

#   INTELSAT 902                # negative incl at 9313 min then

#                       # 270 deg Lyddane bug at 37606 min

1 26900U 01039A   06106.74503247  .00000045  00000-0  10000-3 0  8290

2 26900   0.0164 266.5378 0003319  86.1794 182.2590  1.00273847 16981   9300.00     9400.00        60.00

#   COSMOS 1024 DEB     # 12h resonant ecc in 0.5 to 0.65 range

1 26975U 78066F   06174.85818871  .00000620  00000-0  10000-3 0  6809

2 26975  68.4714 236.1303 5602877 123.7484 302.5767  2.05657553 67521      0.0      2880.0        120.00

#   CBERS 2                     # Near Earth, ecc = 8.84E-5 (< 1.0e-4)

#                       # drop certain normal drag terms

1 28057U 03049A   06177.78615833  .00000060  00000-0  35940-4 0  1836

2 28057  98.4283 247.6961 0000884  88.1964 271.9322 14.35478080140550      0.0      2880.0        120.00

#   NAVSTAR 53 (USA 175)# 12h non-resonant GPS (ecc < 0.5 ecc)

1 28129U 03058A   06175.57071136 -.00000104  00000-0  10000-3 0   459

2 28129  54.7298 324.8098 0048506 266.2640  93.1663  2.00562768 18443      0.0      1440.0        120.00

#   COSMOS 2405         # Near Earth, perigee = 127.20 (< 156) s4 mod

1 28350U 04020A   06167.21788666  .16154492  76267-5  18678-3 0  8894

2 28350  64.9977 345.6130 0024870 260.7578  99.9590 16.47856722116490      0.0      2880.0        120.00

#   H-2 R/B                   # Deep space, perigee = 135.75 (<156) s4 mod

1 28623U 05006B   06177.81079184  .00637644  69054-6  96390-3 0  6000

2 28623  28.5200 114.9834 6249053 170.2550 212.8965  3.79477162 12753      0.0      1440.0        120.00

#   XM-3                      # 24h resonant geo, incl < 3 deg goes

#                       # negative around 1130 min

1 28626U 05008A   06176.46683397 -.00000205  00000-0  10000-3 0  2190

2 28626   0.0019 286.9433 0000335  13.7918  55.6504  1.00270176  4891      0.0      1440.0        120.00

#   MINOTAUR R/B        # Sub-orbital case - Decayed 2005-11-29

#                       #(perigee = -51km), lost in 50 minutes

1 28872U 05037B   05333.02012661  .25992681  00000-0  24476-3 0  1534

2 28872  96.4736 157.9986 0303955 244.0492 110.6523 16.46015938 10708      0.0        50.0          5.00

#   SL-14 DEB           # Last stage of decay - lost in under 420 min

1 29141U 85108AA  06170.26783845  .99999999  00000-0  13519-0 0   718

2 29141  82.4288 273.4882 0015848 277.2124  83.9133 15.93343074  6828      0.0       440.0         20.00

#   SL-12 DEB           # Near Earth, perigee = 212.24 < 220

#                       # simplified drag eq

1 29238U 06022G   06177.28732010  .00766286  10823-4  13334-2 0   101

2 29238  51.5595 213.7903 0202579  95.2503 267.9010 15.73823839  1061      0.0      1440.0        120.00

#                       # Original STR#3 SGP4 test

1 88888U          80275.98708465  .00073094  13844-3  66816-4 0    87

2 88888  72.8435 115.9689 0086731  52.6988 110.5714 16.05824518  1058      0.0      1440.0        120.00

#

# APPENDIX C – Verification Results Data

5 xx

0.00000000    7022.46529266   -1400.08296755       0.03995155  1.893841015  6.405893759  4.534807250

360.00000000   -7154.03120202   -3783.17682504   -3536.19412294  4.741887409 -4.151817765 -2.093935425

720.00000000   -7134.59340119    6531.68641334    3260.27186483 -4.113793027 -2.911922039 -2.557327851

1080.00000000    5568.53901181    4492.06992591    3863.87641983 -4.209106476  5.159719888  2.744852980

1440.00000000    -938.55923943   -6268.18748831   -4294.02924751  7.536105209 -0.427127707  0.989878080

1800.00000000   -9680.56121728    2802.47771354     124.10688038 -0.905874102 -4.659467970 -3.227347517

2160.00000000     190.19796988    7746.96653614    5110.00675412 -6.112325142  1.527008184 -0.139152358

2520.00000000    5579.55640116   -3995.61396789   -1518.82108966  4.767927483  5.123185301  4.276837355

2880.00000000   -8650.73082219   -1914.93811525   -3007.03603443  3.067165127 -4.828384068 -2.515322836

3240.00000000   -5429.79204164    7574.36493792    3747.39305236 -4.999442110 -1.800561422 -2.229392830

3600.00000000    6759.04583722    2001.58198220    2783.55192533 -2.180993947  6.402085603  3.644723952

3960.00000000   -3791.44531559   -5712.95617894   -4533.48630714  6.668817493 -2.516382327 -0.082384354

4320.00000000   -9060.47373569    4658.70952502     813.68673153 -2.232832783 -4.110453490 -3.157345433

4632 xx

0.00000000    2334.11450085  -41920.44035349      -0.03867437  2.826321032 -0.065091664  0.570936053

-5184.00000000  -29020.02587128   13819.84419063   -5713.33679183 -1.768068390 -3.235371192 -0.395206135

-5064.00000000  -32982.56870101  -11125.54996609   -6803.28472771  0.617446996 -3.379240041  0.085954707

-4944.00000000  -22097.68730513  -31583.13829284   -4836.34329328  2.230597499 -2.166594667  0.426443070

-4896.00000000  -15129.94694545  -36907.74526221   -3487.56256701  2.581167187 -1.524204737  0.504805763

6251 xx

0.00000000    3988.31022699    5498.96657235       0.90055879 -3.290032738  2.357652820  6.496623475

120.00000000   -3935.69800083     409.10980837    5471.33577327 -3.374784183 -6.635211043 -1.942056221

240.00000000   -1675.12766915   -5683.30432352   -3286.21510937  5.282496925  1.508674259 -5.354872978

360.00000000    4993.62642836    2890.54969900   -3600.40145627  0.347333429  5.707031557  5.070699638

480.00000000   -1115.07959514    4015.11691491    5326.99727718 -5.524279443 -4.765738774  2.402255961

600.00000000   -4329.10008198   -5176.70287935     409.65313857  2.858408303 -2.933091792 -6.509690397

720.00000000    3692.60030028    -976.24265255   -5623.36447493  3.897257243  6.415554948  1.429112190

840.00000000    2301.83510037    5723.92394553    2814.61514580 -5.110924966 -0.764510559  5.662120145

960.00000000   -4990.91637950   -2303.42547880    3920.86335598 -0.993439372 -5.967458360 -4.759110856

1080.00000000     642.27769977   -4332.89821901   -5183.31523910  5.720542579  4.216573838 -2.846576139

1200.00000000    4719.78335752    4798.06938996    -943.58851062 -2.294860662  3.492499389  6.408334723

1320.00000000   -3299.16993602    1576.83168320    5678.67840638 -4.460347074 -6.202025196 -0.885874586

1440.00000000   -2777.14682335   -5663.16031708   -2462.54889123  4.915493146  0.123328992 -5.896495091

1560.00000000    4992.31573893    1716.62356770   -4287.86065581  1.640717189  6.071570434  4.338797931

1680.00000000      -8.22384755    4662.21521668    4905.66411857 -5.891011274 -3.593173872  3.365100460

1800.00000000   -4966.20137963   -4379.59155037    1349.33347502  1.763172581 -3.981456387 -6.343279443

1920.00000000    2954.49390331   -2080.65984650   -5754.75038057  4.895893306  5.858184322  0.375474825

2040.00000000    3363.28794321    5559.55841180    1956.05542266 -4.587378863  0.591943403  6.107838605

2160.00000000   -4856.66780070   -1107.03450192    4557.21258241 -2.304158557 -6.186437070 -3.956549542

2280.00000000    -497.84480071   -4863.46005312   -4700.81211217  5.960065407  2.996683369 -3.767123329

2400.00000000    5241.61936096    3910.75960683   -1857.93473952 -1.124834806  4.406213160  6.148161299

2520.00000000   -2451.38045953    2610.60463261    5729.79022069 -5.366560525 -5.500855666  0.187958716

2640.00000000   -3791.87520638   -5378.82851382   -1575.82737930  4.266273592 -1.199162551 -6.276154080

2760.00000000    4730.53958356     524.05006433   -4857.29369725  2.918056288  6.135412849  3.495115636

2880.00000000    1159.27802897    5056.60175495    4353.49418579 -5.968060341 -2.314790406  4.230722669

8195 xx

0.00000000    2349.89483350  -14785.93811562       0.02119378  2.721488096 -3.256811655  4.498416672

120.00000000   15223.91713658  -17852.95881713   25280.39558224  1.079041732  0.875187372  2.485682813

240.00000000   19752.78050009   -8600.07130962   37522.72921090  0.238105279  1.546110924  0.986410447

360.00000000   19089.29762968    3107.89495018   39958.14661370 -0.410308034  1.640332277 -0.306873818

480.00000000   13829.66070574   13977.39999817   32736.32082508 -1.065096849  1.279983299 -1.760166075

600.00000000    3333.05838525   18395.31728674   12738.25031238 -1.882432221 -0.611623333 -4.039586549

720.00000000    2622.13222207  -15125.15464924     474.51048398  2.688287199 -3.078426664  4.494979530

840.00000000   15320.56770017  -17777.32564586   25539.53198382  1.064346229  0.892184771  2.459822414

960.00000000   19769.70267785   -8458.65104454   37624.20130236  0.229304396  1.550363884  0.966993056

1080.00000000   19048.56201523    3260.43223119   39923.39143967 -0.418015536  1.639346953 -0.326094840

1200.00000000   13729.19205837   14097.70014810   32547.52799890 -1.074511043  1.270505211 -1.785099927

1320.00000000    3148.86165643   18323.19841703   12305.75195578 -1.895271701 -0.678343847 -4.086577951

1440.00000000    2890.80638268  -15446.43952300     948.77010176  2.654407490 -2.909344895  4.486437362

1560.00000000   15415.98410712  -17699.90714437   25796.19644689  1.049818334  0.908822332  2.434107329

1680.00000000   19786.00618538   -8316.74570581   37723.74539119  0.220539813  1.554518900  0.947601047

1800.00000000   19007.28688729    3412.85948715   39886.66579255 -0.425733568  1.638276809 -0.345353807

1920.00000000   13627.93015254   14216.95401307   32356.13706868 -1.083991976  1.260802347 -1.810193903

2040.00000000    2963.26486560   18243.85063641   11868.25797486 -1.908015447 -0.747870342 -4.134004492

2160.00000000    3155.85126036  -15750.70393364    1422.32496953  2.620085624 -2.748990396  4.473527039

2280.00000000   15510.15191770  -17620.71002219   26050.43525345  1.035454678  0.925111006  2.408534465

2400.00000000   19801.67198812   -8174.33337167   37821.38577439  0.211812700  1.558576937  0.928231880

2520.00000000   18965.46529379    3565.19666242   39847.97510998 -0.433459945  1.637120585 -0.364653213

2640.00000000   13525.88227400   14335.15978787   32162.13236536 -1.093537945  1.250868256 -1.835451681

2760.00000000    2776.30574260   18156.98538451   11425.73046481 -1.920632199 -0.820370733 -4.181839232

2880.00000000    3417.20931586  -16038.79510665    1894.74934058  2.585515864 -2.596818146  4.456882556

9880 xx

0.00000000   13020.06750784   -2449.07193500       1.15896030  4.247363935  1.597178501  4.956708611

120.00000000   19190.32482476    9249.01266902   26596.71345328 -0.624960193  1.324550562  2.495697637

240.00000000   11332.67806218   16517.99124008   38569.78482991 -1.400974747  0.710947006  0.923935636

360.00000000     328.74217398   19554.92047380   40558.26246145 -1.593281066  0.126772913 -0.359627307

480.00000000  -10684.90590680   18057.15728839   33158.75253886 -1.383205997 -0.582328999 -1.744412556

600.00000000  -17069.78000550    9944.86797897   13885.91649059  0.044133354 -1.853448464 -3.815303117

720.00000000   13725.09398980   -2180.70877090     863.29684523  3.878478111  1.656846496  4.944867241

840.00000000   19089.63879226    9456.29670247   27026.79562883 -0.656614299  1.309112636  2.449371941

960.00000000   11106.41248373   16627.60874079   38727.35140296 -1.409722680  0.698582526  0.891383535

1080.00000000      72.40958621   19575.08054144   40492.12544001 -1.593394604  0.113655142 -0.390556063

1200.00000000  -10905.89252576   17965.41205111   32850.07298244 -1.371396120 -0.601706604 -1.782817058

1320.00000000  -17044.61207568    9635.48491849   13212.59462953  0.129244030 -1.903551430 -3.884569098

1440.00000000   14369.90303735   -1903.85601062    1722.15319852  3.543393116  1.701687176  4.913881358

1560.00000000   18983.96210441    9661.12233804   27448.99557732 -0.687189304  1.293808870  2.403630759

1680.00000000   10878.79336704   16735.31433954   38879.23434264 -1.418239666  0.686235750  0.858951848

1800.00000000    -184.03743100   19593.09371709   40420.40606889 -1.593348925  0.100448697 -0.421571993

1920.00000000  -11125.12138631   17870.19488928   32534.21521208 -1.359116236 -0.621413776 -1.821629856

2040.00000000  -17004.43272827    9316.53926351   12526.11883812  0.220330736 -1.955594322 -3.955058575

2160.00000000   14960.06492693   -1620.68430805    2574.96359381  3.238634028  1.734723385  4.868880331

2280.00000000   18873.46347257    9863.57004586   27863.46574735 -0.716736981  1.278632817  2.358448535

2400.00000000   10649.86857581   16841.14172669   39025.48035006 -1.426527152  0.673901057  0.826632332

2520.00000000    -440.53459323   19608.95524423   40343.10675451 -1.593138597  0.087147884 -0.452680559

2640.00000000  -11342.45028909   17771.44223942   32211.12535721 -1.346344015 -0.641464291 -1.860864234

2760.00000000  -16948.06005711    8987.64254880   11826.28284367  0.318007297 -2.009693492 -4.026726648

2880.00000000   15500.53445068   -1332.90981042    3419.72315308  2.960917974  1.758331634  4.813698638

9998 xx

0.00000000   25532.98947267  -27244.26327953      -1.11572421  2.410283885  2.194175683  0.545888526

-1440.00000000  -11362.18265118  -35117.55867813   -5413.62537994  3.137861261 -1.011678260  0.267510059

-1380.00000000     309.25349929  -36960.43090143   -4198.48007670  3.292429375 -0.002166046  0.402111628

-1320.00000000   11949.04009077  -35127.37816804   -2565.89806468  3.119942784  1.012096444  0.497284100

-1260.00000000   22400.45329336  -29798.63236321    -677.91515122  2.638533344  1.922477736  0.542792913

-1200.00000000   30640.84752458  -21525.02340201    1277.34808722  1.903464941  2.634294312  0.534540934

-1140.00000000   35899.56788035  -11152.71158138    3108.72535238  0.997393045  3.079858548  0.474873291

-1080.00000000   37732.45438600     288.18821054    4643.87587495  0.016652226  3.225184410  0.371669746

-1020.00000000   36045.92961699   11706.61816230    5746.32646574 -0.942409065  3.069888941  0.236662980

-960.00000000   31076.77273609   22063.44379776    6325.93403705 -1.794027976  2.642072476  0.083556127

-900.00000000   23341.26015320   30460.88002531    6342.91707895 -2.469409743  1.990861658 -0.073612096

-840.00000000   13568.39733054   36204.45930900    5806.79548733 -2.919354203  1.178920217 -0.221646814

-780.00000000    2628.58762420   38840.10855897    4771.91979854 -3.114400514  0.276239109 -0.348926401

-720.00000000   -8535.81598158   38171.79073851    3331.00311285 -3.043839958 -0.644462527 -0.445808894

11801 xx

0.00000000    7473.37102491     428.94748312    5828.74846783  5.107155391  6.444680305 -0.186133297

360.00000000   -3305.22148694   32410.84323331  -24697.16974954 -1.301137319 -1.151315600 -0.283335823

720.00000000   14271.29083858   24110.44309009   -4725.76320143 -0.320504528  2.679841539 -2.084054355

1080.00000000   -9990.05800009   22717.34212448  -23616.88515553 -1.016674392 -2.290267981  0.728923337

1440.00000000    9787.87836256   33753.32249667  -15030.79874625 -1.094251553  0.923589906 -1.522311008

14128 xx

0.00000000   34747.57932696   24502.37114079      -1.32832986 -1.731642662  2.452772615  0.608510081

120.00000000   18263.33439094   38159.96004751    4186.18304085 -2.744396611  1.255583260  0.528558932

240.00000000   -3023.38840703   41783.13186459    7273.03412906 -3.035574793 -0.271656544  0.309645251

360.00000000  -23516.34391907   34424.42065671    8448.49867693 -2.529120477 -1.726186020  0.009582303

480.00000000  -37837.46699511   18028.39727170    7406.25540271 -1.360069525 -2.725794686 -0.292555349

600.00000000  -42243.58460661   -3093.72887774    4422.91711801  0.163110919 -3.009980598 -0.517584362

720.00000000  -35597.57919549  -23407.91145393     282.09554383  1.641405246 -2.506773678 -0.606963478

840.00000000  -19649.19834455  -37606.11623860   -3932.71525948  2.689647056 -1.349150016 -0.537710698

960.00000000    1431.30912160  -41982.04949668   -7120.45467057  3.035263353  0.160882945 -0.327993994

1080.00000000   22136.97605384  -35388.19823762   -8447.62393401  2.587624889  1.630097136 -0.032349004

1200.00000000   37050.15790219  -19537.23321425   -7564.83463543  1.461844494  2.674654256  0.272202191

1320.00000000   42253.81760945    1431.81867593   -4699.87621174 -0.049247334  3.019518960  0.505890058

1440.00000000   36366.59147396   22023.54245720    -601.47121821 -1.549681546  2.571788981  0.607057418

1560.00000000   20922.12287985   36826.33975981    3654.91125886 -2.644070068  1.447521216  0.548722983

1680.00000000     -23.77224182   41945.51688402    6950.29891751 -3.043358385 -0.057417440  0.346112094

1800.00000000  -20964.17821076   36039.06206172    8418.91984963 -2.642795221 -1.546099886  0.052725852

1920.00000000  -36401.63863057   20669.75286162    7677.19769359 -1.549488154 -2.627052310 -0.254079652

2040.00000000  -42298.30327543    -119.03351118    4922.96388841 -0.052232768 -3.018152669 -0.493827331

2160.00000000  -37125.62383511  -20879.63058368     879.86971348  1.456499841 -2.619358421 -0.604081694

2280.00000000  -22250.12320553  -36182.74736487   -3393.15365183  2.583161226 -1.536647628 -0.556404555

2400.00000000   -1563.06258654  -42035.43179159   -6780.02161760  3.034917506 -0.052702046 -0.363395654

2520.00000000   19531.64069587  -36905.65470956   -8395.46892032  2.693682199  1.446079999 -0.075256054

2640.00000000   35516.53506142  -22123.71916638   -7815.04516935  1.646882125  2.568416058  0.232985912

2760.00000000   42196.03535976   -1547.32646751   -5187.39401981  0.166491841  3.019211549  0.480665780

2880.00000000   37802.25393045   19433.57330019   -1198.66634226 -1.359930580  2.677830903  0.602507466

16925 xx

0.00000000    5559.11686836  -11941.04090781     -19.41235206  3.392116762 -1.946985124  4.250755852

120.00000000   12339.83273749   -2771.14447871   18904.57603433 -0.871247614  2.600917693  0.581560002

240.00000000   -3385.00215658    7538.13955729     200.59008616 -2.023512865 -4.261808344 -6.856385787

360.00000000   12805.22442200  -10258.94667177   13780.16486738  0.619279224  1.821510542  2.507365975

480.00000000    5682.46556318    7199.30270473   15437.67134070 -2.474365406  2.087897336 -2.583767460

600.00000000    7628.94243982  -12852.72097492    2902.87208981  2.748131081 -0.740084579  4.125307943

720.00000000   11531.64866625    -858.27542736   19086.85993771 -1.170071901  2.660311986  0.096005705

840.00000000   -3866.98069515    2603.73442786   -4577.36484577  1.157257298 -8.453281164 -4.683959407

960.00000000   13054.77732721   -8707.92757730   15537.63259903  0.229846748  2.119467054  2.063396852

1080.00000000    3496.91064652    8712.83919778   12845.81838327 -2.782184997  1.552950644 -3.554436131

1200.00000000    9593.07424729  -13023.75963608    6250.46484931  2.072666376  0.278735334  3.778111073

1320.00000000   10284.79205084    1487.89914169   18824.37381327 -1.530335053  2.663107730 -0.542205966

1440.00000000    -984.62035146   -5187.03480813   -5745.59594144  4.340271916 -7.266811354  1.777668888

20413 xx

0.00000000   25123.29290741  -13225.49966287    3249.40351869  0.488683419  4.797897593 -0.961119693

1440.00000000 -151669.05280515   -5645.20454550   -2198.51592118 -0.869182889 -0.870759872  0.156508219

1560.00000000 -157497.71657495  -11884.99595074   -1061.44439402 -0.749657961 -0.864016715  0.157766101

1680.00000000 -162498.32255577  -18062.99733167      81.00915253 -0.638980378 -0.853687105  0.158098992

1800.00000000 -166728.76010920  -24155.99648299    1222.84128677 -0.535600687 -0.840455444  0.157680857

1920.00000000 -169935.81924592  -31767.29787964    2749.01540345 -0.430050431 -0.828904183  0.157812340

2040.00000000 -172703.07831815  -37662.95639336    3883.60052579 -0.338004891 -0.810277487  0.156020035

2160.00000000 -174823.19337404  -43417.55605219    5003.26312809 -0.250258622 -0.789828672  0.153764903

2280.00000000 -176324.63925775  -49018.51958648    6104.85025002 -0.166136613 -0.767706262  0.151092242

2400.00000000 -177231.42142458  -54454.12699497    7185.48661607 -0.085067854 -0.744001567  0.148033403

2520.00000000 -177563.73583232  -59713.14859144    8242.48472591 -0.006561730 -0.718760309  0.144608676

2640.00000000 -177338.48026483  -64784.54644698    9273.27220003  0.069809946 -0.691990238  0.140829236

2760.00000000 -176569.65151461  -69657.21976255   10275.33063459  0.144426878 -0.663665876  0.136698419

2880.00000000 -175268.65299073  -74319.77625463   11246.14177160  0.217631370 -0.633731091  0.132212491

3000.00000000 -173444.53039609  -78760.31560396   12183.13775212  0.289737325 -0.602099929  0.127361017

3120.00000000 -171104.14813653  -82966.21323591   13083.65278381  0.361037779 -0.568655903  0.122126889

3240.00000000 -168252.31543803  -86923.89363433   13944.87382716  0.431811396 -0.533249797  0.116486022

3360.00000000 -164891.86832887  -90618.58225954   14763.78794247  0.502328269 -0.495695896  0.110406725

3480.00000000 -161023.71139825  -94034.02398835   15537.12375729  0.572855321 -0.455766412  0.103848688

3600.00000000 -156646.82136725  -97152.15370791   16261.28409305  0.643661538 -0.413183688  0.096761524

3720.00000000 -151758.21285737  -99952.70098346   16932.26607548  0.715023254 -0.367609561  0.089082727

3840.00000000 -146352.86521283 -102412.70506284   17545.56394158  0.787229695 -0.318630913  0.080734873

3960.00000000 -140423.60777444 -104505.90799734   18096.04807097  0.860588979 -0.265739987  0.071621768

4080.00000000 -133960.95961851 -106201.98091318   18577.81121953  0.935434758 -0.208307307  0.061623110

4200.00000000 -126952.91860010 -107465.51906186   18983.96903112  1.012133628 -0.145543878  0.050587007

4320.00000000 -119384.69396454 -108254.71115372   19306.39581892  1.091093313 -0.076447479  0.038319282

21897 xx

0.00000000  -14464.72135182   -4699.19517587       0.06681686 -3.249312013 -3.281032707  4.007046940

120.00000000  -19410.46286123  -19143.03318969   23114.05522619  0.508602237 -1.156882269  2.379923455

240.00000000  -12686.06129708  -23853.75335645   35529.81733588  1.231633829 -0.221718202  1.118440291

360.00000000   -2775.46649359  -22839.64574119   39494.64689967  1.468963405  0.489481769 -0.023972788

480.00000000    7679.87883570  -16780.50760106   34686.21815555  1.364171080  1.211183897 -1.385151371

600.00000000   14552.40023028   -4819.50121461   17154.70672449  0.109201591  2.176124494 -3.854856805

720.00000000  -15302.38845375   -5556.43440300    1095.95088753 -2.838224312 -3.134231137  3.992596326

840.00000000  -19289.20066748  -19427.04851118   23759.45685636  0.552495087 -1.112499437  2.325112654

960.00000000  -12376.21976437  -23893.38020018   35831.33691892  1.246701529 -0.194294048  1.074867282

1080.00000000   -2400.55677665  -22698.62264640   39482.75964390  1.472582922  0.513555654 -0.069306561

1200.00000000    8031.66819252  -16455.77592085   34298.94391742  1.351357426  1.239633234 -1.448195324

1320.00000000   14559.48780372   -4238.43773813   16079.23154704 -0.026409655  2.218938770 -4.012628896

1440.00000000  -16036.04980660   -6372.51406468    2183.44834232 -2.485113443 -2.994994355  3.955891272

1560.00000000  -19156.71583814  -19698.89059957   24389.29473934  0.594278133 -1.069418599  2.271152044

1680.00000000  -12062.72925552  -23925.82362911   36120.66680667  1.261238798 -0.167201856  1.031478939

1800.00000000   -2024.96136966  -22551.56626703   39458.50085787  1.475816889  0.537615764 -0.114887472

1920.00000000    8379.80916204  -16123.95878459   33894.75123231  1.337468254  1.268432783 -1.512473301

2040.00000000   14527.86748873   -3646.33817120   14960.74306518 -0.180035839  2.261273515 -4.179355590

2160.00000000  -16680.12147335   -7149.80800425    3257.64227208 -2.178897351 -2.863927095  3.904876943

2280.00000000  -19013.58793448  -19958.93766022   25003.81778666  0.634100431 -1.027559823  2.218002685

2400.00000000  -11745.76155818  -23951.19438627   36397.87676581  1.275261813 -0.140425132  0.988259441

2520.00000000   -1648.81945070  -22398.50594576   39421.83273890  1.478660174  0.561671519 -0.160733093

2640.00000000    8723.97652795  -15784.99406275   33473.35215527  1.322433593  1.297602497 -1.578055493

2760.00000000   14452.25571587   -3043.42332645   13796.84870805 -0.355190169  2.302485443 -4.355767077

2880.00000000  -17246.31075678   -7890.72601508    4315.39410307 -1.910968458 -2.740945672  3.844722726

22312 xx

0.00000000    1442.10132912    6510.23625449       8.83145885 -3.475714837  0.997262768  6.835860345

54.20286720     306.10478453   -5816.45655525   -2979.55846068  3.950663855  3.415332543 -5.879974329

74.20286720    3282.82085464    2077.46972905   -5189.17988770  0.097342701  7.375135692  2.900196702

94.20286720     530.82729176    6426.20790003    1712.37076793 -3.837120395 -1.252430637  6.561602577

114.20286720   -3191.69170212     170.27219912    5956.29807775 -1.394956872 -7.438073471 -0.557553115

134.20286720   -1818.99222465   -6322.45146616     681.95247154  3.349795173 -1.530140265 -6.831522765

154.20286720    2515.66448634   -2158.83091224   -5552.13320544  2.571979660  7.311930509 -1.639865620

174.20286720    2414.52833210    5749.10150922   -1998.59693165 -2.681032960  3.527589301  6.452951429

194.20286720   -1877.98944331    3862.27848302    5112.48435863 -3.261489804 -6.026859137  3.433254768

214.20286720   -3117.36584395   -4419.74773864    3840.85960912  1.545479182 -5.475416581 -5.207913748

234.20286720     815.32034678   -5231.67692249   -3760.04690354  3.870864200  4.455588552 -5.211082191

254.20286720    3269.54341810    3029.00081083   -4704.67969713 -0.526711345  6.812157950  3.929825087

274.20286720     -10.18099756    6026.23341453    2643.50518407 -3.953623254 -2.616070012  6.145637500

294.20286720   -3320.58819584   -1248.42679945    5563.06017927 -0.637046974 -7.417786044 -2.076120187

314.20286720   -1025.48974616   -6366.98945782    -911.23559153  3.811771909  0.438071490 -6.829260617

334.20286720    3003.75996128    -413.85708003   -5706.15591435  1.674350083  7.694169068  0.316915204

354.20286720    1731.42816980    6258.27676925    -409.32527982 -3.400497806  1.447945424  6.904010052

374.20286720   -2582.52111460    2024.19020680    5647.55650268 -2.530348121 -7.221719393  1.438141553

394.20286720   -2440.56848578   -5702.77311877    1934.81094689  2.731792947 -3.350576075 -6.527773339

414.20286720    1951.22934391   -3423.59443045   -5121.67808201  3.249039133  6.465974362 -3.069806659

434.20286720    2886.50939356    4888.68626216   -3096.29885989 -1.973162139  4.877039020  5.832414910

454.20286720   -1276.55532182    4553.26898463    4406.19787375 -3.715146421 -5.320176914  4.418210777

474.20286720   -3181.54698042   -3831.29976506    4096.80242787  1.114159970 -6.104773578 -4.829967400

22674 xx

0.00000000   14712.22023280   -1443.81061850       0.83497888  4.418965470  1.629592098  4.115531802

120.00000000   25418.88807860    9342.60307989   23611.46690798  0.051284086  1.213127306  2.429004159

240.00000000   21619.59550749   16125.24978864   36396.79365831 -0.963604380  0.685454965  1.177181937

360.00000000   12721.50543331   19258.96193362   40898.47648359 -1.457448565  0.179955469  0.071502601

480.00000000    1272.80760054   18458.41971897   37044.74742696 -1.674863386 -0.436454983 -1.201040990

600.00000000  -10058.43188619   11906.60764454   21739.62097733 -1.245829683 -1.543789125 -3.324449221

720.00000000   10924.40116466   -2571.92414170   -2956.34856294  6.071727751  1.349579102  3.898430260

840.00000000   25332.14851525    8398.91099924   21783.90654357  0.222320754  1.272214306  2.580527192

960.00000000   22317.71926039   15574.82086129   35495.77144092 -0.892750056  0.737383381  1.291738834

1080.00000000   13795.68675885   19088.83051008   40803.69584385 -1.420277669  0.235599456  0.185517056

1200.00000000    2515.17145049   18746.63776282   37864.58088636 -1.668016053 -0.360431458 -1.052854596

1320.00000000   -9084.48602106   12982.62608646   24045.63900249 -1.378032363 -1.373184736 -3.013963835

1440.00000000    5647.00909495   -3293.90518693   -5425.85235063  8.507977176  0.414560797  2.543322806

1560.00000000   25111.63372210    7412.55109488   19844.25781729  0.416496290  1.332106006  2.739301737

1680.00000000   22961.47461641   14985.74459578   34511.09257381 -0.816711048  0.789391108  1.407901804

1800.00000000   14841.15301459   18876.91439870   40626.25901619 -1.380403341  0.290228810  0.298258120

1920.00000000    3750.70174081   18978.57939698   38578.11783220 -1.656939412 -0.287930881 -0.910825599

2040.00000000   -8027.30219489   13939.54436955   26136.49045637 -1.474476061 -1.222693624 -2.737178731

2160.00000000   -1296.95657092   -2813.69369768   -5871.09587258  9.881929371 -1.978467207 -1.922261005

2280.00000000   24738.60364819    6383.41644019   17787.27631900  0.639556952  1.392554379  2.906206324

2400.00000000   23546.85388669   14358.15602832   33441.67679479 -0.734895006  0.841564851  1.526009909

2520.00000000   15855.87696303   18624.05633582   40367.13420574 -1.337753546  0.343969522  0.410018472

2640.00000000    4976.44933591   19156.75504042   39189.68603184 -1.642084365 -0.218525096 -0.774148204

2760.00000000   -6909.20746210   14790.44707042   28034.46732222 -1.545152610 -1.088119523 -2.487447214

2880.00000000   -7331.65006707    -604.17323419   -2723.51014575  6.168997265 -3.634011554 -5.963531682

23177 xx

0.00000000   -8801.60046706      -0.03357557      -0.44522743 -3.835279101 -7.662552175  0.944561323

120.00000000   -1684.34352858  -31555.95196340    3888.99944319  2.023055719 -2.151306405  0.265065778

240.00000000   12325.51410155  -38982.15046244    4802.88832275  1.763224157 -0.102514446  0.012397139

360.00000000   22773.66831936  -34348.02176606    4228.77407391  1.067616787  1.352427865 -0.166956367

480.00000000   26194.40441089  -19482.94203672    2393.84774063 -0.313732186  2.808771328 -0.346204118

600.00000000    8893.50573448    5763.38890561    -713.69884164 -7.037399220  3.022613131 -0.370272416

720.00000000   -6028.75686537  -25648.99913786    3164.37107274  1.883159288 -3.177051976  0.390793162

840.00000000    8313.57299056  -38146.45710922    4697.80777535  1.905002133 -0.625883074  0.076098187

960.00000000   20181.29108622  -36842.60674073    4529.12568218  1.326244476  0.921916487 -0.114527455

1080.00000000   26302.61794569  -25173.39539436    3084.65309986  0.245398835  2.329974347 -0.287495880

1200.00000000   19365.07045602   -2700.00490122     317.42727417 -3.009733018  3.902496058 -0.478928582

1320.00000000   -9667.81878780  -16930.19112642    2095.87469034  1.279288285 -4.736005905  0.582878255

1440.00000000    4021.31438583  -36066.09209609    4442.91587411  2.007322354 -1.227461376  0.149383897

23333 xx

0.00000000   -9301.24542292    3326.10200382    2318.36441127 -8.729303005 -0.828225037 -0.122314827

120.00000000  -44672.91239679   -6213.11996581   -1738.80131727 -3.719475070 -1.336673022 -0.621888261

240.00000000  -67053.08885387  -14994.69685946   -5897.99072793 -2.860576613 -1.183771565 -0.568473909

360.00000000  -85227.84253168  -22897.08484471   -9722.59184564 -2.426469823 -1.078592475 -0.525341431

480.00000000 -100986.00419136  -30171.19698695  -13283.77044765 -2.147108978 -1.000530827 -0.491587582

600.00000000 -115093.00686386  -36962.56316477  -16634.15682929 -1.945446188 -0.938947736 -0.464199202

720.00000000 -127965.80064891  -43363.32967165  -19809.90480432 -1.789652016 -0.888278463 -0.441254468

840.00000000 -139863.28332206  -49436.45704153  -22836.80438139 -1.663762568 -0.845315913 -0.421548627

960.00000000 -150960.22978258  -55227.45413896  -25734.01408879 -1.558730986 -0.808061065 -0.404293846

1080.00000000 -161381.71414630  -60770.64040903  -28516.26290017 -1.468977174 -0.775190459 -0.388951810

1200.00000000 -171221.18736947  -66092.76474442  -31195.19847387 -1.390837596 -0.745785633 -0.375140398

1320.00000000 -180550.82888745  -71215.23290630  -33780.24938270 -1.321788672 -0.719184752 -0.362579495

1440.00000000 -189427.87533074  -76155.54943344  -36279.19882816 -1.260024473 -0.694896053 -0.351058133

1560.00000000 -197898.69401408  -80928.29015181  -38698.57972447 -1.204211888 -0.672544709 -0.340413731

1600.00000000 -200638.82986236  -82484.14969882  -39488.34331447 -1.186748462 -0.665472422 -0.337037582

23599 xx

0.00000000    9892.63794341      35.76144969      -1.08228838  3.556643237  6.456009375  0.783610890

20.00000000   11931.95642997    7340.74973750     886.46365987  0.308329116  5.532328972  0.672887281

40.00000000   11321.71039205   13222.84749156    1602.40119049 -1.151973982  4.285810871  0.521919425

60.00000000    9438.29395675   17688.05450261    2146.59293402 -1.907904054  3.179955046  0.387692479

80.00000000    6872.08634639   20910.11016811    2539.79945034 -2.323995367  2.207398462  0.269506121

100.00000000    3933.37509798   23024.07662542    2798.25966746 -2.542860616  1.327134966  0.162450076

120.00000000     816.64091546   24118.98675475    2932.69459428 -2.626838010  0.504502763  0.062344306

140.00000000   -2334.41705804   24246.86096326    2949.36448841 -2.602259646 -0.288058266 -0.034145135

160.00000000   -5394.31798039   23429.42716149    2850.86832586 -2.474434068 -1.074055982 -0.129868366

180.00000000   -8233.35130237   21661.24480883    2636.51456118 -2.230845533 -1.875742344 -0.227528603

200.00000000  -10693.96497348   18909.88168891    2302.33707548 -1.835912433 -2.716169865 -0.329931880

220.00000000  -12553.89669904   15114.63990716    1840.93573231 -1.212478879 -3.619036996 -0.439970633

240.00000000  -13450.20591864   10190.57904289    1241.95958736 -0.189082511 -4.596701971 -0.559173899

260.00000000  -12686.60437121    4079.31106161     498.27078614  1.664498211 -5.559889865 -0.676747779

280.00000000   -8672.55867753   -2827.56823315    -342.59644716  5.515079852 -5.551222962 -0.676360044

300.00000000    1153.31498060   -6411.98692060    -779.87288941  9.689818102  1.388598425  0.167868798

320.00000000    9542.79201056    -533.71253081     -65.73165428  3.926947087  6.459583539  0.785686755

340.00000000   11868.80960100    6861.59590848     833.72780602  0.452957852  5.632811328  0.685262323

360.00000000   11376.23941678   12858.97121366    1563.40660172 -1.087665695  4.374693347  0.532207051

380.00000000    9547.70300782   17421.48570758    2118.56907515 -1.876540262  3.253891728  0.395810243

400.00000000    7008.51470263   20725.47471227    2520.56064289 -2.308703599  2.270724438  0.276138613

420.00000000    4082.28135104   22911.04184601    2786.37568309 -2.536665546  1.383670232  0.168153407

440.00000000     969.17978149   24071.23673676    2927.31326579 -2.626695115  0.557172428  0.067536854

460.00000000   -2184.71515444   24261.21671601    2950.08142825 -2.607072866 -0.236887607 -0.029125215

480.00000000   -5253.42223370   23505.37595671    2857.66120738 -2.484424544 -1.022255436 -0.124714444

500.00000000   -8108.27961017   21800.81688388    2649.72981961 -2.247597251 -1.821159176 -0.221925624

520.00000000  -10594.77795556   19117.80779221    2322.72136979 -1.863118484 -2.656426668 -0.323521502

540.00000000  -12497.32045995   15398.64085906    1869.69983897 -1.258130763 -3.551583368 -0.432338888

560.00000000  -13467.92475245   10560.90147785    1280.78399181 -0.271870523 -4.520514224 -0.550016092

580.00000000  -12848.18843590    4541.21901842     548.53826427  1.494157156 -5.489585384 -0.667472039

600.00000000   -9152.70552728   -2344.24950144    -287.98121970  5.127921095 -5.650383025 -0.685989008

620.00000000     280.38490909   -6500.10264018    -790.36092984  9.779619614  0.581815811  0.074171345

640.00000000    9166.25784315   -1093.12552651    -129.49428887  4.316668714  6.438636494  0.785116609

660.00000000   11794.48942915    6382.21138354     780.88439015  0.604412453  5.731729369  0.697574333

680.00000000   11424.30138324   12494.26088864    1524.33165488 -1.021328075  4.463448968  0.542532698

700.00000000    9652.09867350   17153.84762075    2090.48038336 -1.844516637  3.327522235  0.403915232

720.00000000    7140.41945884   20539.25485336    2501.21469368 -2.293173684  2.333507912  0.282716311

24208 xx

0.00000000    7534.10987189   41266.39266843      -0.10801028 -3.027168008  0.558848996  0.207982755

120.00000000  -14289.19940414   39469.05530051    1428.62838591 -2.893205245 -1.045447840  0.179634249

240.00000000  -32222.92014955   26916.25425799    2468.59996594 -1.973007929 -2.359335071  0.102539376

360.00000000  -41413.95109398    7055.51656639    2838.90906671 -0.521665080 -3.029172207 -0.002066843

480.00000000  -39402.72251896  -14716.42475223    2441.32678358  1.066928187 -2.878714619 -0.105865729

600.00000000  -26751.08889828  -32515.13982431    1384.38865570  2.366228869 -1.951032799 -0.181018498

720.00000000   -6874.77975542  -41530.38329422     -46.60245459  3.027415087 -0.494671177 -0.207337260

840.00000000   14859.52039042  -39302.58907247   -1465.02482524  2.869609883  1.100123969 -0.177514425

960.00000000   32553.14863770  -26398.88401807   -2485.45866002  1.930064459  2.401574539 -0.099250520

1080.00000000   41365.67576837   -6298.09965811   -2828.05254033  0.459741276  3.051680214  0.006431872

1200.00000000   38858.83295070   15523.39314924   -2396.86850752 -1.140211488  2.867567143  0.110637217

1320.00000000   25701.46068162   33089.42617648   -1308.68556638 -2.428713821  1.897381431  0.184605907

1440.00000000    5501.08137100   41590.27784405     138.32522930 -3.050691874  0.409203052  0.207958133

25954 xx

0.00000000    8827.15660472  -41223.00971237       3.63482963  3.007087319  0.643701323  0.000941663

-1440.00000000    8118.18519221  -41368.40537378       4.11046687  3.017696741  0.591994297  0.000933016

-1320.00000000   27766.34015328  -31724.97000557       9.93297846  2.314236153  2.024903193  0.000660861

-1200.00000000   39932.57237973  -13532.60040454      13.12958252  0.987382819  2.911942843  0.000213298

-1080.00000000   41341.01365441    8305.71681955      12.84988501 -0.605098224  3.014378268 -0.000291034

-960.00000000   31614.99210558   27907.29155353       9.16618797 -2.034243523  2.305014102 -0.000718418

-840.00000000   13375.75227587   39994.27017651       3.05416854 -2.915424366  0.975119874 -0.000955576

-720.00000000   -8464.89963309   41312.93549892      -3.86622919 -3.011600615 -0.617275050 -0.000939664

-600.00000000  -28026.23406158   31507.89995661      -9.76047869 -2.296840160 -2.043607595 -0.000674889

-480.00000000  -40040.01314363   13218.00579413     -13.06594832 -0.963328772 -2.919827983 -0.000231414

-360.00000000  -41268.43291976   -8632.06859693     -12.90661266  0.630042315 -3.009677376  0.000273163

-240.00000000  -31377.85317015  -28156.13970334      -9.32605530  2.054021717 -2.288554158  0.000704959

-120.00000000  -13031.41552688  -40092.33381029      -3.27636660  2.924657466 -0.950541167  0.000949381

0.00000000    8827.15660472  -41223.00971237       3.63482963  3.007087319  0.643701323  0.000941663

120.00000000   28306.85426674  -31243.80147394       9.57216891  2.279137743  2.064316875  0.000684127

240.00000000   40159.05128805  -12845.39151157      12.96086316  0.937265422  2.928448287  0.000245505

360.00000000   41192.55903455    9013.79606759      12.90495666 -0.656727442  3.003543458 -0.000257479

480.00000000   31131.69755798   28445.55681731       9.42419238 -2.073484842  2.269770851 -0.000691233

600.00000000   12687.81846530   40217.83324639       3.44726249 -2.931721827  0.924962230 -0.000940766

720.00000000   -9172.23500245   41161.63475527      -3.43575757 -3.000571486 -0.668847508 -0.000940101

840.00000000  -28562.51093192   31022.45987587      -9.39562161 -2.261449202 -2.082713897 -0.000689669

960.00000000  -40260.77504549   12529.11484344     -12.84915105 -0.913097031 -2.935933528 -0.000256181

1080.00000000  -41114.14376538   -9338.87194483     -12.87952404  0.681588815 -2.998432565  0.000245006

1200.00000000  -30890.01512240  -28690.40750792      -9.48037212  2.092989805 -2.252978152  0.000680459

1320.00000000  -12341.46194020  -40310.06316386      -3.55833201  2.940537098 -0.900219523  0.000934170

1440.00000000    9533.27750818  -41065.52390214       3.30756482  2.995596171  0.695200236  0.000938525

26900 xx

0.00000000  -42014.83795787    3702.34357772     -26.67500257 -0.269775247 -3.061854393  0.000336726

9300.00000000   40968.68133298   -9905.99156086      11.84946837  0.722756848  2.989645389 -0.000161261

9360.00000000   42135.66858481    1072.99195618      10.83481752 -0.078150602  3.074772455 -0.000380063

9400.00000000   41304.75156132    8398.27742944       9.74006214 -0.612515135  3.014117469 -0.000511575

26975 xx

0.00000000  -14506.92313768  -21613.56043281      10.05018894  2.212943308  1.159970892  3.020600202

120.00000000    7309.62197950    6076.00713664    6800.08705263  1.300543383  5.322579615 -4.788746312

240.00000000   -3882.62933791   11960.00543452  -25088.14383845 -2.146773699 -1.372461491 -2.579382089

360.00000000  -16785.45507465    -734.79159704  -34300.57085853 -1.386528125 -1.907762641 -0.220949641

480.00000000  -23524.16689356  -13629.45124622  -30246.27899200 -0.462846784 -1.586139830  1.269293624

600.00000000  -22890.23597092  -22209.35900155  -16769.91946116  0.704351342 -0.671112594  2.432433851

720.00000000  -11646.39698980  -19855.44222106    3574.00109607  2.626712727  1.815887329  2.960883901

840.00000000    7665.76124241   11159.78946577     345.93813117 -0.584818007  3.193514161 -5.750338922

960.00000000   -6369.35388112   10204.80073022  -27844.52150384 -2.050573276 -1.582940542 -2.076075232

1080.00000000  -18345.64763145   -2977.76684430  -34394.90760612 -1.243589864 -1.892050757  0.060372061

1200.00000000  -23979.74839255  -15436.44139571  -28616.50540218 -0.294973425 -1.482987916  1.478255628

1320.00000000  -21921.97167880  -22852.45147658  -13784.85308485  0.945455629 -0.428940995  2.596964378

1440.00000000   -8266.43821031  -17210.74590112    6967.95546070  3.082244069  2.665881872  2.712555075

1560.00000000    6286.85464535   13809.56328971   -6321.60663781 -1.615964016  1.383135377 -5.358719132

1680.00000000   -8730.87526788    8244.63344365  -30039.92372791 -1.935622871 -1.724162072 -1.631224738

1800.00000000  -19735.81883249   -5191.76593007  -34166.14974143 -1.097835530 -1.860148418  0.324401050

1920.00000000  -24232.73847703  -17112.08243255  -26742.88893252 -0.119786184 -1.364365317  1.680220468

2040.00000000  -20654.45640708  -23184.54386047  -10611.55144716  1.209238113 -0.144169639  2.748054938

2160.00000000   -4337.15988957  -13410.46817244    9870.45949215  3.532753095  3.772236461  2.088424247

2280.00000000    4074.62263523   14698.07548285  -12248.65327973 -2.053824693  0.203325817 -4.607867718

2400.00000000  -10950.23438984    6148.66879447  -31736.65532865 -1.809875605 -1.816179062 -1.233364913

2520.00000000  -20952.40702045   -7358.71507895  -33633.06643074 -0.948973031 -1.813594137  0.573893078

2640.00000000  -24273.48944134  -18637.15546906  -24633.27702390  0.064161440 -1.228537560  1.875728935

2760.00000000  -19057.55468077  -23148.29322082   -7269.38614178  1.500802809  0.195383037  2.879031237

2880.00000000      43.69305308   -8145.90299207   11634.57079913  3.780661682  5.105315423  0.714401345

28057 xx

0.00000000   -2715.28237486   -6619.26436889      -0.01341443 -1.008587273  0.422782003  7.385272942

120.00000000   -1816.87920942   -1835.78762132    6661.07926465  2.325140071  6.655669329  2.463394512

240.00000000    1483.17364291    5395.21248786    4448.65907172  2.560540387  4.039025766 -5.736648561

360.00000000    2801.25607157    5455.03931333   -3692.12865694 -0.595095864 -3.951923117 -6.298799125

480.00000000     411.09332812   -1728.99769152   -6935.45548810 -2.935970964 -6.684085058  1.492800886

600.00000000   -2506.52558454   -6628.98655094    -988.07784497 -1.390577189 -0.556164143  7.312736468

720.00000000   -2090.79884266   -2723.22832193    6266.13356576  1.992640665  6.337529519  3.411803080

840.00000000    1091.80560222    4809.88229503    5172.42897894  2.717483546  4.805518977 -5.030019896

960.00000000    2811.14062300    5950.65707171   -2813.23705389 -0.159662742 -3.121215491 -6.775341949

1080.00000000     805.72698304    -812.16627907   -7067.58483968 -2.798936020 -6.889265977  0.472770873

1200.00000000   -2249.59837532   -6505.84890714   -1956.72365062 -1.731234729 -1.528750230  7.096660885

1320.00000000   -2311.57375797   -3560.99112891    5748.16749600  1.626569751  5.890482233  4.293545048

1440.00000000     688.16056594    4124.87618964    5794.55994449  2.810973665  5.479585563 -4.224866316

1560.00000000    2759.94088230    6329.87271798   -1879.19518331  0.266930672 -2.222670878 -7.119390567

1680.00000000    1171.50677137     125.82053748   -7061.96626202 -2.605687852 -6.958489749 -0.556333225

1800.00000000   -1951.43708472   -6251.71945820   -2886.95472355 -2.024131483 -2.475214272  6.741537478

1920.00000000   -2475.70722288   -4331.90569958    5117.31234924  1.235823539  5.322743371  5.091281211

2040.00000000     281.46097847    3353.51057102    6302.87900650  2.840647273  6.047222485 -3.337085992

2160.00000000    2650.33118860    6584.33434851    -908.29027134  0.675457235 -1.274044972 -7.323921567

2280.00000000    1501.17226597    1066.31132756   -6918.71472952 -2.361891904 -6.889669974 -1.574718619

2400.00000000   -1619.73468334   -5871.14051991   -3760.56587071 -2.264093975 -3.376316601  6.254622256

2520.00000000   -2581.04202505   -5020.05572531    4385.92329047  0.829668458  4.645048038  5.789262667

2640.00000000    -119.22080628    2510.90620488    6687.45615459  2.807575712  6.496549689 -2.384136661

2760.00000000    2486.23806726    6708.18210028      80.43349581  1.057274905 -0.294294027 -7.384689123

2880.00000000    1788.42334580    1990.50530957   -6640.59337725 -2.074169091 -6.683381288 -2.562777776

28129 xx

0.00000000   21707.46412351  -15318.61752390       0.13551152  1.304029214  1.816904974  3.161919976

120.00000000   18616.75971861    3166.15177043   18833.41523210 -2.076122016  2.838457575  1.586210535

240.00000000   -3006.50596328   18522.20742011   18941.84078154 -3.375452789  1.032680773 -1.559324534

360.00000000  -21607.02086957   15432.59962630     206.62470309 -1.306049851 -1.817011568 -3.163725018

480.00000000  -18453.06134549   -3150.83256134  -18685.83030936  2.106017925 -2.860236337 -1.586151870

600.00000000    3425.11742384  -18514.73232706  -18588.67200557  3.394666340 -1.003072030  1.610061295

720.00000000   21858.23838149  -15101.51661554     387.34517048  1.247973967  1.856017403  3.161439948

840.00000000   18360.69935796    3506.55256762   19024.81678979 -2.122684184  2.830618605  1.537510677

960.00000000   -3412.84765409   18646.85269710   18748.00359987 -3.366815728  0.986039922 -1.607874972

1080.00000000  -21758.08331586   15215.44829478    -180.82181406 -1.250144680 -1.856490448 -3.163774870

1200.00000000  -18193.41290284   -3493.85876912  -18877.14757717  2.153326942 -2.852221264 -1.536617760

1320.00000000    3833.57386848  -18635.77026711  -18388.68722886  3.384748179 -0.955363841  1.658785020

1440.00000000   22002.20074562  -14879.72595593     774.32827099  1.191573619  1.894561165  3.159953047

28350 xx

0.00000000    6333.08123128   -1580.82852326      90.69355720  0.714634423  3.224246550  7.083128132

120.00000000   -3990.93845855    3052.98341907    4155.32700629 -5.909006188 -0.876307966 -5.039131404

240.00000000    -603.55232010   -2685.13474569   -5891.70274282  7.572519907 -1.975656726  0.121722605

360.00000000    4788.22345627     782.56169214    4335.14284621 -4.954509026  3.683346464  4.804645839

480.00000000   -6291.84601644    1547.82790772    -453.67116498 -0.308625588 -3.341538574 -7.082659115

600.00000000    4480.74573428   -3028.55200374   -3586.94343641  5.320920857  1.199736275  5.626350481

720.00000000    -446.42460916    2932.28872588    5759.19389757 -7.561000245  1.550975493 -1.374970885

840.00000000   -3713.79581831   -1382.66125130   -5122.45131136  6.090931626 -3.512629733 -3.467571746

960.00000000    6058.32017522    -827.47406722    2104.04678651 -1.798403024  3.787067272  6.641439744

1080.00000000   -5631.73659006    2623.70953644    1766.49125084 -3.216401578 -2.309140959 -6.788609120

1200.00000000    2776.84991560   -3255.36941953   -4837.19667790  6.748135564 -0.193044825  4.005718698

1320.00000000    1148.04430837    2486.07343386    5826.34075913 -7.420162295  2.589456382  0.356350006

1440.00000000   -4527.90871828    -723.29199041   -4527.44608319  5.121674217 -3.909895427 -4.500218556

28623 xx

0.00000000  -11665.70902324   24943.61433357      25.80543633 -1.596228621 -1.476127961  1.126059754

120.00000000  -11645.35454950     979.37668356    5517.89500058  3.407743502 -5.183094988 -0.492983277

240.00000000    5619.19252274   19651.44862280   -7261.38496765 -2.013634213  3.106842861  0.284235517

360.00000000   -9708.68629714   26306.14553149   -1204.29478856 -1.824164290 -0.931909596  1.113419052

480.00000000  -14394.03162892    6659.30765074    5593.38345858  1.556522911 -4.681657614  0.296912248

600.00000000    7712.09476270   15565.72627434   -7342.40465571 -1.646800364  4.070313571 -0.109483081

720.00000000   -7558.36739603   27035.11367962   -2385.12054184 -1.999583791 -0.393409283  1.078093515

840.00000000  -15495.61862220   11550.15897828    5053.83178121  0.469277336 -4.029761073  0.679054742

960.00000000    9167.02568222   10363.65204210   -6871.52576042 -0.881621027  5.223361510 -0.740696297

1080.00000000   -5275.80272094   27151.78486008   -3494.50687216 -2.129609388  0.150196480  1.021038089

1200.00000000  -15601.37656145   15641.29379850    4217.03266850 -0.249183123 -3.405238557  0.888214503

1320.00000000    9301.05872300    3883.15265574   -5477.86477017  0.871447821  6.493677331 -1.885545282

1440.00000000   -2914.31065828   26665.20392758   -4511.09814335 -2.216261909  0.710067769  0.940691824

28626 xx

0.00000000   42080.71852213   -2646.86387436       0.81851294  0.193105177  3.068688251  0.000438449

120.00000000   37740.00085593   18802.76872802       3.45512584 -1.371035206  2.752105932  0.000336883

240.00000000   23232.82515008   35187.33981802       4.98927428 -2.565776620  1.694193132  0.000163365

360.00000000    2467.44290178   42093.60909959       5.15062987 -3.069341800  0.179976276 -0.000031739

480.00000000  -18962.59052991   37661.66243819       4.04433258 -2.746151982 -1.382675777 -0.000197633

600.00000000  -35285.00095313   23085.44402778       2.08711880 -1.683277908 -2.572893625 -0.000296282

720.00000000  -42103.20138132    2291.06228893      -0.13274964 -0.166974816 -3.070104560 -0.000311007

840.00000000  -37580.31858370  -19120.40485693      -2.02755702  1.394367848 -2.740341612 -0.000248591

960.00000000  -22934.20761876  -35381.23870806      -3.16495932  2.580167539 -1.672360951 -0.000134907

1080.00000000   -2109.90332389  -42110.71508198      -3.36507889  3.070935369 -0.153808390 -0.000005855

1200.00000000   19282.77774728  -37495.59250598      -2.71861462  2.734400524  1.406220933  0.000103486

1320.00000000   35480.60990600  -22779.03375285      -1.52841859  1.661210676  2.587414593  0.000168300

1440.00000000   42119.96263499   -1925.77567263      -0.19827433  0.140521206  3.071541613  0.000179561

28872 xx

0.00000000   -6131.82730456    2446.52815528    -253.64211033 -0.144920228  0.995100963  7.658645067

5.00000000   -5799.24256134    2589.14811119    2011.54515100  2.325207364 -0.047125672  7.296234071

10.00000000   -4769.05061967    2420.46580562    4035.30855837  4.464585796 -1.060923209  6.070907874

15.00000000   -3175.45157340    1965.98738086    5582.12569607  6.049639376 -1.935777558  4.148607019

20.00000000   -1210.19024802    1281.54541294    6474.68172772  6.920746273 -2.580517337  1.748783868

25.00000000     896.73799533     447.12357305    6607.22400507  6.983396282 -2.925846168 -0.872655207

30.00000000    2896.99663534    -440.04738594    5954.92675486  6.211488246 -2.926949815 -3.433959806

35.00000000    4545.78970167   -1273.55952872    4580.16512984  4.656984233 -2.568711513 -5.638510954

40.00000000    5627.43299371   -1947.94282469    2634.16714930  2.464141047 -1.873985161 -7.195743032

45.00000000    5984.72318534   -2371.37691609     349.87996209 -0.121276950 -0.911981546 -7.859613894

50.00000000    5548.43325922   -2480.16469245   -1979.24314527 -2.763269534  0.199691915 -7.482796996

29141 xx

0.00000000     423.99295524   -6658.12256149     136.13040356  1.006373613  0.217309983  7.662587892

20.00000000     931.80883587   -1017.17852239    6529.19244527 -0.298847918  7.613891977  1.226399480

40.00000000     -83.44906141    6286.20208453    2223.49837161 -1.113515974  2.530970283 -7.219445568

60.00000000    -958.57681221    3259.26005348   -5722.63732467 -0.101225813 -6.735338321 -3.804851872

80.00000000    -255.25619985   -5132.59762974   -4221.27233118  1.077709303 -4.905938824  5.892521264

100.00000000     867.44295097   -5038.40402933    4256.73810533  0.479447535  5.032326446  5.857126248

120.00000000     559.16882013    3376.30587937    5699.22017391 -0.906749328  6.646149867 -3.852331832

140.00000000    -669.85184205    6196.00229484   -2281.95741770 -0.795804092 -2.752114827 -7.202478520

160.00000000    -784.20708019   -1278.53125553   -6449.19892596  0.636702380 -7.595425203  1.431090802

180.00000000     406.15811659   -6607.03115799     148.33021477  1.009818575  0.231843765  7.692047844

200.00000000     916.34911813    -884.08649248    6491.09810362 -0.302163049  7.669887109  1.084336909

220.00000000    -104.02490970    6304.31821405    1960.08739882 -1.108873823  2.259522809 -7.351147710

240.00000000    -944.61642849    2872.17248379   -5846.94103362 -0.051117686 -6.989747076 -3.413102600

260.00000000    -187.16569888   -5404.86163467   -3731.97057618  1.094696706 -4.412110995  6.326060952

280.00000000     884.59720467   -4465.74516163    4725.83632696  0.380656028  5.691554046  5.303910983

300.00000000     446.40767236    4086.66839620    5093.05596650 -0.982424447  6.072965199 -4.791630682

320.00000000    -752.24467495    5588.35473301   -3275.04092573 -0.661161370 -4.016290740 -6.676898026

340.00000000    -643.72872525   -2585.02528560   -5923.01306608  0.807922142 -7.171597814  3.041115058

360.00000000     584.40295819   -6202.35605817    1781.00536019  0.869250450  2.226927514  7.471676765

380.00000000     779.59211765    1100.73728301    6311.59529480 -0.599552305  7.721032522 -1.275153027

400.00000000    -403.03155588    6399.18000837    -364.12735875 -1.008861924 -0.516636615 -7.799812287

420.00000000    -852.93910071     192.65232023   -6322.47054784  0.396006194 -7.882964919 -0.289331517

29238 xx

0.00000000   -5566.59512819   -3789.75991159      67.60382245  2.873759367 -3.825340523  6.023253926

120.00000000    4474.27915495   -1447.72286142    4619.83927235  4.712595822  5.668306153 -2.701606741

240.00000000    1922.17712474    5113.01138342   -4087.08470203 -6.490769651 -0.522350158 -3.896001154

360.00000000   -6157.93546882   -2094.70798790   -1941.63730960  0.149900661 -5.175192523  5.604262034

480.00000000    2482.64052411   -3268.45944555    5146.38006190  6.501814698  4.402848754 -0.350943511

600.00000000    4036.26455287    4827.43347201   -2507.99063955 -5.184409515  1.772280695 -5.331390168

720.00000000   -5776.81371622    -118.64155319   -3641.22052418 -2.539917207 -5.622701582  4.403125405

840.00000000      67.98699487   -4456.49213473    4863.71794283  7.183809420  2.418917791  2.015642495

960.00000000    5520.62207038    3782.38203554    -596.73193161 -3.027966069  3.754152525 -6.013506363

1080.00000000   -4528.05104455    1808.46273329   -4816.99727762 -4.808419763 -5.185789345  2.642104494

1200.00000000   -2356.61468078   -4852.51202272    3856.53816184  6.688446735  0.118520958  4.021854210

1320.00000000    6149.65800134    2173.59423261    1369.29488732 -0.345832777  5.109857861 -5.842951828

1440.00000000   -2629.55011449    3400.98040158   -5344.38217129 -6.368548448 -3.998963509  0.577253064

88888 xx

0.00000000    2328.96975262   -5995.22051338    1719.97297192  2.912073281 -0.983417956 -7.090816210

120.00000000    1020.69234558    2286.56260634   -6191.55565927 -3.746543902  6.467532721  1.827985678

240.00000000   -3226.54349155    3503.70977525    4532.80979343  1.000992116 -5.788042888  5.162585826

360.00000000    2456.10706533   -6071.93855503    1222.89768554  2.679390040 -0.448290811 -7.228792155

480.00000000     787.16457349    2719.91800946   -6043.86662024 -3.759883839  6.277439314  2.397897864

600.00000000   -3110.97648029    3121.73026235    4878.15217035  1.244916056 -6.124880425  4.700576353

720.00000000    2567.56229695   -6112.50383922     713.96374435  2.440245751  0.098109002 -7.319959258

840.00000000     556.05661780    3144.52288201   -5855.34636178 -3.754660143  6.044752775  2.957941672

960.00000000   -2982.47940539    2712.61663711    5192.32330472  1.475566773 -6.427737014  4.202420227

1080.00000000    2663.08964352   -6115.48290885     196.40072866  2.196121564  0.652415093 -7.362824152

1200.00000000     328.54999674    3557.09490552   -5626.21427211 -3.731193288  5.769341172  3.504058731

1320.00000000   -2842.06876757    2278.42343492    5472.33437150  1.691852635 -6.693216335  3.671022712

1440.00000000    2742.55398832   -6079.67009123    -326.39012649  1.948497651  1.211072678 -7.356193131

# APPENDIX D. REFERENCES

Hoots, Felix R., et al. “History of Analytical Orbit Modeling in the U.S. Space Surveillance System.” Journal of Guidance Control, and Dynamics March-April 2004: 174-185.

Vallado, David, et al. ‘Revisiting Spacetrack Report #3’, AIAA 2006-6753

Vallado, David, et al. ‘Revisiting Spacetrack Report #3’, AIAA 2006-6753

Abercromby, Kira. “Object 8832” E-Mail to N. Miura, 26 May 2009.

“Selected Orbital Data” 2009. <http://www.heavens-above.com>

http://en.wikipedia.org/wiki/WGS84